United States Coast Guard

### Barge Inspection Book

| ## Name of Vessel |  |
| --- | --- |
| ## Official Number | ## Class |
| ## Date Completed | ## Location |
| ## Route Oceans Limited Coastwise Lakes/Bays/Sounds Coastwise Great Lakes Rivers |  |
| ## Inspection Type Certification of Inspection (COI) Annual Drydock Inspection Periodic Internal Structure Exam (ISE) Underwater Survey in Lieu of Drydock (UNWILD) Cargo Tank Internal Exam (CTIE) |  |
| ## Inspectors 1. ___________________ 2. _________________ |  |
| ## Activity Number ___________________ |  |

Rev. Jan 2010

Total Time Spent Per Activity

| Regular Personnel (Active Duty) |  |  |  |
| --- | --- | --- | --- |
| Activity Type | Activity | Training | (PERS) MI |

| Total ADMIN Hours | Total Travel Hours |
| --- | --- |

| Reserve Personnel |  |  |  |
| --- | --- | --- | --- |
| Activity Type | Activity | Training | (PERS) MI |

| Total ADMIN Hours | Total Travel Hours |
| --- | --- |

| ### Auxiliary Resources |  |
| --- | --- |
| Total Boat Hours | Total Aircraft Hours |

Use of Barge Inspection Book:

This inspection book is intended to be used as a job aid by Coast Guard marine inspectors during inspections of U.S. flagged barges. The lists contained within this book are not intended to limit the inspection. Each marine inspector should determine the depth of inspection necessary. A checked box should be a running record of what has been inspected. It does not imply that the entire system has been inspected or that all or any items are in full compliance. This job aid does not constitute part of the official inspection record.

This document does not establish or change Federal laws or regulations. References given are only general guides. Refer to IMO publications, CFRs, NVICs or any locally produced cite guides for specific regulatory references. Not all items in this book are applicable to all vessels.

Note:  Guidance on how to conduct inspections of U.S. flagged barges can be found in the Marine Safety Manual (MSM) Volume II, Chapter B1: Inspection of Vessels for Certification. All MSM cites listed in this book refer to MSM Volume II unless otherwise indicated.

Table of Contents:

Section 1: Pre-Administrative Items

IMO Applicability Dates					6	Involved Parties and General Information			7	Vessel Information						8	Vessel Description and Hull type				8	Certificates and Documents					9

Section 2: Pre-Inspection Screening

Pre-Inspection Items					11

Certificates and Documents					12	Pollution Prevention Plans/Certificates/Records			13	Logs and Manuals						14

Section 3: Drydock Inspection

External Structural Examination				17

Section 4: Internal Structural Examination (ISE)				Internal Structural Exam					20

Section 5: Cargo Tank Internal Inspection

Cargo Tank Internals					22

Section 6: COI/ Periodic/Annual Inspection

General Health and Safety Systems				24

Lifesaving Equipment (Manned Barges Only)			27

Navigation Equipment					29

Fire Protection Equipment					29

Ground Tackle						32

Auxiliary Machinery and Machinery Spaces			33

Electrical Equipment					35

Pollution Prevention Systems					37

Observe Drills (Manned Barges Only)				39

Tank Barge Cargo Operations					39

Thermal Fluid Heaters					43

Cargo Tank Internals					44

New Construction and Modified Vessels				45

Chemical Gas Cargo Operations				46

Bulk Liquefied Flammable Gases Cargo Operations		48

Vapor Control Systems					49

MTSA Compliance						51

Section 7:  Post Inspection

Post-Inspection Items					60

Section 8: Appendices

Recommended Deficiency Procedures				61

Deficiency Summary Worksheet				62

Notes							64

Conversions						65

Use the following table to determine which sections of the Barge Inspection Book to complete.

| ## If you are performing a: | ## Then Complete Sections: | ## Page |
| --- | --- | --- |
| Drydock Inspection | Section 1 | 6 |
|  | Section 2 | 11 |
|  | Section 3 | 17 |
|  | Section 7 | 60 |
| Internal Structural Exam | Section 1 | 6 |
|  | Section 2 | 11 |
|  | Section 4 | 20 |
|  | Section 7 | 60 |
| Cargo Tank Inspection | Section 1 | 6 |
|  | Section 2 | 11 |
|  | Section 5 | 22 |
|  | Section 7 | 70 |
| Topside-COI/ Periodic/Annual | Section 1 | 6 |
|  | Section 2 | 10 |
|  | Section 6 | 23 |
|  | Section 7 | 60 |

Section 1: Pre-Administrative Items

# Task 1: IMO Applicability Dates

| Reference | Date |
| --- | --- |
| MARPOL 73/38 Annex I | 02 OCT 83 |
| MARPOL 73/38 Annex II | 06 APR 87 |
| MARPOL 73/38 Annex III | 01 JUL 92 |
| MARPOL 73/38 Annex V | 31 DEC 88 |
| MARPOL 73/38 Annex VI | 19 MAY 05 |
| IBC Code | After 01 JUL 86 |
| BCH Code | Prior to 01 JUL 86 |
| COLREGS 1972 Various additional amendments to COLREGS | 15 JUL 77 |
| Load Line 1966 | 21 JUL 68 |

Task 2: Involved Parties and General Information

Vessel’s Representatives: ______________________________________ ______________________________________

Phone Numbers:

Owner-Listed on DOC (if applicable), or COFR:

#### No Change

Operator:

#### No Change

Task 3: Vessel Information

Classification Society:

Last Drydock Date: Next Drydock Date:

Location of Last Drydocking:

Gross Tons:

Net Tons:

Built Date: (use delivery date)

Overall length: (in feet)

#### Does vessel meet double-hull requirements? YES NO If not, vessel must meet requirements by:________ (date) in accordance with 33 CFR Part 157 Appendix G.

Required Crew: Number of Tankermen Number of Lifeboatmen Required ____________ Required ____________

Cargo Carried (inspected and approved) Type of Cargo ________ Amount of Cargo ___________

Date Cargo Tanks Entered:

| #### Vessel Description | #### Hull Type |
| --- | --- |
| #### Subchapter D cargoes only | #### Type I |
| #### Subchapter I cargoes only | #### Type II |
| #### Subchapter D & O cargoes | #### Type III |
| Subchapter I & O cargoes |  |

Task 4: Pre-Administrative Certificates and Documents

| Endors. Date |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Exp. Date |  |  |  |  |  |  |  |
| Issued Date |  |  |  |  |  |  |  |
| Port Issued |  |  |  |  |  |  |  |
| ID # |  |  |  |  |  |  |  |
| Issuing Agency |  |  |  |  |  |  |  |
| Name of Certificate | Certificate of Documentation No Change | Classification Document No Change | Certificate of Financial Responsibility (COFR) | International Load Line (ILL) No Change | International Oil Pollution Prevention (IOPP) No Change | Certificate of Fitness (COF) No Change | International Tonnage (ITC) No Change |

| Endors. Date |  |  |
| --- | --- | --- |
| Exp. Date |  |  |
| Issued Date |  |  |
| Port Issued |  |  |
| ID # |  |  |
| Issuing Agency |  |  |
| Name of Certificate | International Air Pollution Prevention Certificate (IAPP) - No Change | Engine International Air Pollution Prevention Certificate (EIAPP) - No Change |

Section 2: Complete Pre-Inspection Screening:

Task 1: Pre-Inspection Items

| Step | Action | Ref |
| --- | --- | --- |
| 1.1 |  Receive the inspection request. Subchapter “D” and “I” - Written COI application 30 days before the desired inspection date - Verbal Inspection Annual/Periodic 3 months before the desired inspection date or after COI issue date | 46 CFR 31.01-15 46 CFR 91.25-5 46 CFR 31.10-17 46 CFR 31.10-21 46 CFR 91.25-5 46 CFR 91.27-1 |
| 1.2 |  Review MISLE’s appropriate critical profile records, vessel history, and outstanding discrepancies, user fee and notes. |  |
| 1.3 |  Determine examination types needed. | 46 CFR 31.10 46 CFR 151-05 46 CFR 91.15 46 CFR 151.04 |
| 1.4 |  Review Code of Federal Regulations (CFR) for applicable subchapter and dates. | 46 CFR Table 31.10-21 46 CFR Table 151-05 |
| 1.5 |  Print applicable documents and certificates from MISLE and local districts. - Temp COI - IOPP - PTP - Cargo Authorization Attachment | 46 CFR 31.05 MSM II B Ch.1.C MSM II A Ch.6.B MSM II A. Ch. 3.H MSM II A Ch.3.I MSM II A.Ch.3.J MSM II A.Ch.3.L |
| 1.6 |  Obtain required safety equipment. |  |

Notes:

Task 2: Certificates and Documents

| Step | Action | Ref |
| --- | --- | --- |
| 2.1 |  Verify that the COI is posted. | 46 CFR 31.05-5 46 CFR 91.01-5 |
| 2.2 |  Verify that the stability letter is posted. | 46 CFR 35.08-1 46 CFR 91.40.3 |
| 2.3 |  Verify that the station bill/muster list is posted in accordance with subchapter “W”. ###### Manned barges only | 46 CFR 35.10-5 46 CFR 97.13-7 46 CFR 199.80 |
| 2.4 |  Examine the waste management plan. Oceangoing manned barges >40 feet | 33 CFR 151.57 33 CFR 151.63 |
| 2.5 |  Verify annual drug testing program. Manned barges only | 46 CFR 16.260 |
| 2.6 |  Examine annual life raft servicing certificates. ###### Manned barges only | 46 CFR 160.051 |
| 2.7 |  Verify that required international safety and pollution prevention convention certificates are posted valid and accurate. ###### Barges on international route only | 46 CFR 91. 60-35 33 CFR 151.19 33 CFR 151.27 |
| 2.8 |  Examine cargo gear certificate. | 46 CFR 31.10-16 |
| 2.9 |  Examine cargo and ballast instruction manual. | 46 CFR 31.10-32(b) 46 CFR 42.15-1(a) |
| 2.10 |  Examine general arrangement/fire control plan. ###### Manned barges only | 46 CFR 35-10-3(a) 46 CFR 91.55-5(d) |
| 2.11 |  Examine equipment test records. - Piping Hydrostatic/ tightness test - Cargo Hose hydrostatic 1-½ MWAP test - Vapor hose hydrostatic 1-½ MWAP test - Alternate cargo piping test - Relief valves - Gauges | 46 CFR 31.10-16 46 CFR 35.07-10(c) 46 CFR 35.35-70 33 CFR 156.170(a) 33 CFR 156.170(f)(3) MSM Vol. II B.6.29 46 CFR 35.35-70(b) 33 CFR 156.170(c((2) |
| Notes: |  |  |

| Step | Action | Ref |
| --- | --- | --- |
| 2.12 |  Examine fixed fire extinguisher servicing certificate. | 46 CFR 31.10-18 |
| 2.13 |  Verify packaged hazardous materials for appropriate cargo documents/records. | 46 CFR 151.45-7 46 CFR 35.01-10 |
| 2.14 |  Verify information is available to master (as required)/person-in-charge. - Loading manual - Trim and stability book | 46 CFR 31.10-22 46 CFR 97.12-1 46 CFR 170.110 46 CFR 35.20-7 |

Task 3: Pollution Prevention Plans/Certificates/Records

| Step | Action | Ref |
| --- | --- | --- |
| 3.1 |  Examine the Certificate of Financial Responsibility. | 33 CFR 138.65 33 CFR 138 |
| 3.2 |  Examine the NLS Certificate. | 46 CFR 153.15 46 CFR 153.901(d)(5) MARPOL 73/78 Annex II |
| 3.3 |  Examine the Shipboard Oil Pollution Emergency Plan. - Approved by the Coast Guard/class society - Contact numbers correct - Immediate actions list | MARPOL Ax. 1/26.1 33 CFR 151.26 |
| 3.4 |  Examine Vapor Collection System Approval Letter. | 46 CFR 39.10-13 |
| 3.5 |  Examine the Benzene Monitoring Program. | 46 CFR 151.50-60 46 CFR 30-25 46 CFR 197.501 |
| 3.6 |  Examine the Stability Letter. | 46 CFR 31.10-30 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 3.7 |  Examine International Air Pollution Prevention Certificate. ###### Barges on international routes only |  |
| 3.8 |  Examine the Oil Record Book. - Each operation signed by person-in-charge - Book maintained for three years Only barges equipped to discharge any oil or oily mixture overboard | MARPOL Ax. 1/20 33 CFR 151.25 |
| 3.9 |  Examine the Vessel Response Plan. - Manned - Unmanned ###### Vessels carrying oil as primary cargo | 33 CFR 155.1030 33 CFR 155.1035 33 CFR 155.1040 46 CFR 91.25-38 |
| 3.10 | ######  Examine Response Plan. ###### Vessels carrying oil as secondary cargo | 33 CFR 155.1045 46 CFR 91.25-38 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 3.11 |  Examine the transfer procedures. Posted Procedures must contain in order or by cross reference the following: - List of products carried by vessel - Description of transfer system - Includes a line diagram of piping - Number of persons required on duty - Duties by title of each person - Means of communication - Procedures to top off tanks - Procedures to report oil discharges - VCS information - Amendments authorized - Transfer flag and light - Load Line Include the following documents if inspecting an international barge: - IOPP - Load Line convention | 33 CFR 155.720 33 CFR 155.740 33 FR 155.750 33 CFR 151.19 46 CFR 31.25-1 33 CFR 151.27 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 3.12 |  Verify waiver letters are carried. | 46 CFR 153.10 |
| 3.13 |  Verify packaged hazardous materials for appropriate cargo documents/records. |  |
| 3.14 |  Review security plan. - Approval letter issued by MSC - Vessels using vessel-specific approved plan or, if manned, verify using “Domestic Vessel Security Plan Verification Guide for MTSA/ISPS Code” - Vessel using ASP and unmanned must have letter signed by owner or operator stating which ASP is used and vessel is in full compliance with that program | NVIC 04-03 Ch. 1 33 CFR 104.140 33 CFR 104.120 |

# Task 4: Logs and Manuals

| Step | Action | Ref |
| --- | --- | --- |
| 4.1 |  Examine lifesaving equipment maintenance records. | 46 CFR 35.40-40 46 CFR 91.20-20 |
| 4.2 |  Examine required logbooks/records. | 46 CFR 35.07 46 CFR 97-35 |
| 4.3 |  Verify vessel has access to shore-based computerized damage stability program. | 33 CFR 155.240 |
| 4.4 |  Examine emergency training and drill logs. | 46 CFR 97.35-5 46 CFR 97.15-35 |
| 4.5 |  Examine the cargo securing manual. | 46 CFR 35.35-60 46 CFR 97.12-5 |

Notes:

Section 3: Drydock Inspection

Task 1: External Structural Examination

| Step | Action | Ref |
| --- | --- | --- |
| 1.1 |  Examine Gauging Periodic Report. - Date Completed________________ - Tank Vessel over 30 years old that carry oil - Class requirement - For cause | 46 CFR 31.10-21(a) ABS Steel Rules 1/3 |
| 1.2 |  Verify availability of vessel plans. (Special requirements for vessels with load lines) | MSM B.Ch1.E MSM B.Ch.3.A MSM B.Ch.3.B |
| 1.3 |  Inspect external structure members. - Plating - Caulking - Reinforcing straps - Rakes - Welds - Pitting - Rub Bars - Overall Steel Wastage | NVIC 7-68 Inland Barge Guide 46 CFR 31.10-15 |
|  | Poor; Good. |  |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 1.4 |  Inspect hull and/or structural members. Yes (attached gauging report) - Transverse belt of deck plating - Transverse belt of bottom and sideshell - Wind-and-water strakes - Keel plates - Bulkhead plating and stiffeners - Suspect areas - Other______________________ | 46 CFR 31.10-21(a) ABS Rules 1/3 Inland Barge Guide |
| 1.5 |  Inspect fastenings and welds. | MSM Vol. IV Ch.6.H NVIC3-68 |
| 1.6 |  Inspect vessel for fractures and fracture repairs. | MSM II.CH.B.E.B.6.D NVIC 7-68 |
| 1.7 |  Inspect hatches, gaskets, and fasteners. | MSM II Ch.B.1.E.5 |
| 1.8 |  Inspect dry dock plugs. |  |
| 1.9 |  Inspect draft marks and load lines. - Proper location - Legibly inscribed - Proper spacing and size - Load line marking verified | 46 CFR 32.05 46 CFR 97.40-10 MSM II Ch.B.1.E.4 |
| 1.10 |  Inspect rudders, bars, and skegs. |  |
| 1.11 |  Inspect bow/stern thrusters. |  |
| 1.12 |  Inspect sea chests, spool pieces, and through-hull fittings. |  |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 1.13 |  Inspect sea valves and nonmetallic expansion joints. - Fitted where required - Opened for examination - Body - Guides - Threads - Seat - Stems - Discs - Valves opened every five years at dry dock | 46 CFR 56.59-95 46 CFR 61.20-5 46 CFR 91.40-1(a) 46 CFR 56.20 46 CFR 56.35.10 46 CFR 91.20-20 MSM II B.3.B |
| 1.14 |  Inspect ground tackle. - Anchor cables ranged Yes/No - Cable shackles and pin - Anchors - Hawse pipes and covers - Chain pipes and covers - Chain lockers | 46 CFR 32.15-15 46 CFR 96.07-5 ABS Rules 33 CFR 155.230 |
| 1.15 |  Inspect scuppers, soil lines, and tank overflows. | 46 CFR 35.35.10 46 CFR 98.30-23(g) |
| 1.16 |  Complete underwater survey. | MSM II.B.Ch.3.C |
| 1.17 |  Check 30-year mid-body analysis. | 46 CFR 31.10-21(a) 46 CFR 32.59 46 CFR Table 30.25 |
| 1.18 |  Process drydock extension request. | MSM II. B.Ch.3.A.4 |

Notes:

Section 4: Internal Structural Examination (ISE)

# Task 1:  Internal Structural Exam

| Step | Action | Ref |
| --- | --- | --- |
| 1.1 | Verify Marine Chemist Certificate. - Marine Chemist No.____________________ - Certificate No.________________________ - Date Issued __________________________ - Location _____________________ - Last three cargoes ____________________ - All spaces certified “Safe for Workers” that are to be entered - MCC maintained by competent person | 46 CFR 35.01-1 46 CFR 91.50-1(a) MSM II Ch.A.5.H NFPA 306 |
| 1.2 |  Inspect internal structural members. - Bulkheads - Decks - Tank tops - Longitudinals - Floors - Frames - Intercoastals - Stiffeners - Beams - Connections - Wastage - Broken welds - Cargo tank exterior (double-hulled) | NVIC 7-68 MSM II A.5.E MSM II B.3.A.4.9(b)(ii) MSM II B.3.A.1 MSM II.B.3.B.3 MSM II A.5.4 33 CFR 157.10-d |
| 1.3 |  Inspect watertight integrity. | MSM II B.1.E.5 |
| 1.4 |  Inspect hull openings and closures. | MSM II.A.5.5 MSM II B.1.E.5 ABS Part 4 Ch.7 Sect. 1 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 1.5 |  Inspect all voids, ballast tanks, and cofferdams. (List all spaces entered) __________ ___________ __________ __________ ___________ __________ __________ ___________ __________ | 46 CFR 176.400 46 CFR 32.70-10 |
| 1.6 |  Inspect guards, ladders, and rails. | 46 CFR 32.02-15 |
| 1.7 |  Inspect bulkhead penetrations. | MSM II B.1.E.6 |
| 1.8 |  Inspect striker plates. |  |
| 1.10 |  Inspect fastening, rivets, and welding. | MSM Vol. IV Ch.6.H NVIC 3-68 |
| 1.11 |  Inspect the forward peak/rake. |  |
| 1.12 |  Inspect the aft peak/rake. |  |

Notes:

Section 5: Cargo Tank Internal Inspection

Task 1: Cargo Tank Internals

| Step | Action | Ref |
| --- | --- | --- |
| 1.1 | Verify Marine Chemist Certificate. - Marine Chemist No.____________________ - Certificate No.________________________ - Date Issued __________________________ - Location _____________________ - Last three cargoes ____________________ - All spaces certified “Safe for Workers” that are to be entered - MCC maintained by competent person | 46 CFR 35.01-1 MSM II Ch.A.5.H NFPA 306 |
| 1.2 | Verify cargo gauging type is correct for authorized cargos - Open Restricted - Closed | 46 CFR 151.15-10 |
| 1.3 |  Verify overfill protection device is installed. | 33 CFR 155.480 33 CFR 151.05 |
| 1.4 |  Inspect all cargo tanks. - Welds - Bulkheads - Framing - Sumps - Pumps (list all spaces entered) _________ ___________ ___________ __________ ___________ ___________ __________ ___________ ___________ | MSM II Ch.B.3.B.4 NAVIC 7-68 |

| Notes: |  |  |  |
| --- | --- | --- | --- |
| Step | Action | Ref |  |
| 1.5 |  Inspect cargo brackets, piping, valves, and reach rods. |  |  |
| 1.6 |  Inspect all openings of vents, piping and valves. |  |  |
| 1.7 |  Inspect pressure vessel cargo tanks. - Required NDT for PVCT’s 30 yrs and older having a 10-year internal exam period. - Owner submit proposal meeting guidance in NVIC 5-89 - Testing conducted by personnel qualified and certified in nondestructive testing |  | 46 CFR 38.25-1 NVIC 5-89 46 CFR 151.04-7 46 CFR 98.25.97 |

Notes:

Section 6: COI/ Periodic/ Annual Inspection

Task 1:  General Health and Safety Systems

| Step | Action | Ref |
| --- | --- | --- |
| 1.1 |  Test general alarms. ###### Manned Barges Only |  |
| 1.2 | Inspect crew accommodations. - Sleeping - Washrooms and toilets - Messrooms - Hospital space - Other spaces Manned Barges Only | 46 CFR 113.25-30 46 CFR 113.25-30 46 CFR 96.05-1 46 CFR 92.20 |
| 1.3 |  Verify means of escape. - Two means required - No locked doors - Properly sized stairway - Dead end corridors Manned Barges Only | 46 CFR 92.10 |
| 1.4 |  Inspect cooking and heating systems including liquefied flammable gas system. - Marking and Instructions - Controls - Piping - Cylinders (test, hydro) - Appliances - Safety devices - Compartment ventilation - Evidence of crew conducting tests Manned Barges Only | 46 CFR 96.05-1 46 CFR 58.16 46 CFR 61.15-10 46 CFR 31.30 46 CFR 91.25-35 |
| 1.5 |  Inspect storerooms. - Stowage - Fire hazards - Lighting |  |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 1.6 |  Conduct general safety inspection throughout the vessel. | 46 CFR 31.10-21(b) |
| 1.7 |  Verify warning signs and signals are posted. - Red warning signals (flag by day, light at night) - Warning Signs (no smoking, no open lights, no visitors) - Radio Room signage (radio equipment shall not be used during the transfer of grade A, B, or C liquids except by permission of senior Deck Officer) | 46 CFR 35.30-1 |
| 1.8 |  Examine benzene-monitoring program. - Regulated areas - Record of personal exposure | 46 CFR 197.501 NVIC 6-92 |
| 1.9 |  Inspect the combustible gas indicators. - Check operation/calibration - UL or FM approved ###### Manned Barges Only | 46 CFR 35.30-15 |
| 1.10 |  Inspect emergency outfits/equipment. - One outfit on barges with cargo Tanks<15 feet - One SCBA - Lifeline with belt/harness - Flashlight - Ax - Non-conductive boots and gloves - Helmet - Protective clothing Manned Barges Only | 46 CFR 35.30-20(b) 46 CFR 96.35-1 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 1.11 |  Inspect protection of spaces for vehicles, as applicable. - Ventilation - Alarms for loss of ventilation - Markings for failure alarm - Carbon monoxide tests - Plans showing vehicle space - Arrangements - Preparation of vehicles prior to carriage | 46 CFR 90.10-38 46 CFR 92.15-10(d) 46 CFR 97.37-50 46 CFR 97.80 |
| 1.12 |  Inspect cranes, as applicable. | 46 CFR 31.10.16(d)(3) |
| 1.13 |  Inspect ventilation systems. - Means of closure - Remote shut-offs - Cleanliness of vents and intakes | 46 CFR 92.15 |
| 1.14 |  Inspect emergency lighting. - Weekly test by crew logged - Storage batteries tested every 6 months - Marked by minimum ½ letter “E” | 46 CFR 31.35 46 CFR 96.05 46 CFR 97.15-30 46 CFR 97.37-25 |
| 1.15 |  Verify that muster and emergency instruction lists are posted. - Ensure content is IAW 46 CFR 199.80 - See exemptions in 46 CFR 199.610(c) | 46 CFR 35.10-5 46 CFR 97.13.1 46 CFR 199.80 |

Notes:

Task 2: Lifesaving Equipment: (Manned Barges Only)

| Step | Action | Ref |
| --- | --- | --- |
| 2.1 | Inspect life rafts. - Launching instructions posted - Stowage - Annual service dates - Hydro release service dates - Weak link - Float free - Markings - Capacities |  |
| 2.2 | Inspect life jackets. - Retro-reflective tape - Lights - Whistles - Work vest - Markings - Number of lifejackets rejected by inspector | 46 CFR 32.05-5 46 CFR 199.620 MSM II Ch.2.H.4 |
| 2.3 |  Inspect life jacket stowage. - Accessibility - Required notices and markings - Stowage lockers - Wearing instructions | 46 CFR 199.70 46 CFR 199.80(c) |
| 2.4 |  Verify that the life jacket donning placards are posted. |  |
| 2.5 |  Inspect ring lifebuoys. - Lights - Lines - Smoke signals - Stowage - Markings | 46 CFR 199.70 |

Notes:

| Step | Action |  | Ref |
| --- | --- | --- | --- |
| 2.6 |  Inspect life floats and buoyant apparatus. - Equipment - Stowage - Markings | 46 CFR 199.640 |  |
| 2.7 |  Inspect immersion suits. | 46 CFR 199.610(b) NVIC 2.63 |  |
| 2.8 |  Inspect the sea painter. - Secured - Weak link - Cleat | 46 CFR 199.175(b)(21) |  |
| 2.9 |  Inspect the general alarms. - Controls - Batteries and fuses - Tested - Markings - Bell locations audible | 46 CFR 32.35 46 CFR 113.25 |  |
| 2.10 |  Examine distress signals. - Required amount - Type - Check expiration date - Storage and condition |  |  |

Notes:

Task 3:  Navigation Equipment

| Step | Action | Ref |
| --- | --- | --- |
| 3.1 |  Verify/Test navigational lights and signals are properly displayed. - Running lights - Sidelight screens proper color and shape - Anchor lights - Anchor/Towing day shapes | 33 CFR 81 33 CFR 82 72 COLREGS |

Task 4: Fire Protection Equipment

| Step | ###### Action | Ref |
| --- | --- | --- |
| 4.1 |  Inspect fire and smoke detection system. - Required in vehicle spaces only - Test alarms/detection system - Remote pull stations - Annual testing/service | 46 CFR 95.05-1 46 CFR 91.25-20 |
| 4.2 |  Inspect portable and semi-portable fire extinguishers. - Controls/instructions/markings - Annually serviced - Markings (weight and hydrostatic test date) - Portable bottles hydrostatically-tested (every 5 years) - Semi-portable bottles hydrostatically-tested (every 5 years) - Flexible loops tested or replaced (same as bottled) - Hose and diffuser Required; On board. Number; Class; Number; Class. | 46 CFR 147.60 46 CFR 34.50-15(a) 46 CFR 95.50-10(a) NVIC 7-70 MSM II Ch.C2.I.3 MSM II Ch.C2.I.4 NVIC 13-86 |

| Notes: |  |  |
| --- | --- | --- |
| Step | Action | Ref |
| 4.3 | Inspect fixed fire extinguisher systems. - CO2 - Cylinders weighted annually - Cylinders hydrostatically-tested (every 12 years) Controls/instructions/markings - Alarms, time delays - Piping, heads - Flex loops tested/replaced (10% per year) - Test or renew every 12 years - Closures for openings - Sprinklers - Pumps - Pressure tanks - Piping, heads - Alarms - Foam - Pumps - Tank - Piping, heads - Foam tested Required on Barges with sleeping accommodations for >12 persons | 46 CFR 95.05-10(a) 46 CFR147.65 46 CFR 34.05-5 46 CFR 95.05-10 46 CFR 34.15 46 CFR 95.15 NVIC 6-72 46 CFR 34.30 46 CFR 95.30 NFPA-13 NVIC 6-72 |
| 4.4 |  Inspect fire main system and stations. - Operation of pumps - Inspect piping for soundness - Relief valve if installed - Two effective streams simultaneously applied - Nozzles and spanner wrenches on station - Hoses connected at station - UL-approved hoses - Correct hose length - Annual hose hydro test (minimum 100 psi) Required on Barges with sleeping accommodations for >12 persons | 46 CFR 95-05-5 46 CFR 95.10.5 |

| Notes: |  |  |
| --- | --- | --- |
| Step | Action | Ref |
| 4.5 |  Inspect fire axes. | 46 CFR 34.60 |
| 4.6 |  Verify presence of up-to-date fire control plan. Required on barges with sleeping accommodations for >6 persons. | 46 CFR 97.36-1 |
| 4.7 |  Inspect structural fire protections systems. - Bulkhead penetration - Fire doors - Noncombustible trim - Fire-resistant furnishing - Fire loads - Windows in control areas - Fire dampers All vessels 4000 gross tons and over and Industrial vessels of 330 gross tons but less than 4000 gross tons with >12 industrial personnel on board. |  |
| 4.8 |  Test fire doors and controls. |  |
| 4.9 |  Inspect paint, oil, and lamp stowage. - Fixed CO2 or other approved system - Explosion-proof fixtures - Closures - Fireproof/metal lined - Lighting/electrical - Fire protection - Markings | 46 CFR 92.07 46 CFRF 32.85-1 46 CFR 34.05.5(a)(3) 46 CFR 92.05-10 46 CFR 95.05-10(c) 46 CFR 95.15.35 46 CFR 111.105-43 |

Notes:

Task 5: Ground Tackle

| Step | Action | Ref |
| --- | --- | --- |
| 5.1 |  Verify proper tackle. - Required number of equipment - Review ABS - Safety program/single-hulled - Anchor stowed for immediate use | 46 CFR 32.15-15 46 CFR 96.07 33 CFR 155.230 ABS Rules part 2, chap 2 |
| 5.2 |  Inspect mooring, towing, standing, and running gear. | 46 CFR 32.15-15 |
| 5.3 |  Inspect emergency towing arrangement. - Available on barge or tow vessel - Retrieval equipment required - Annual inspection IAW manufacturing recommendations - Barge retrieval drills by crew | 33 CFR 155.235 |
| 5.4 |  Test anchor, windlass (including foundation), and capstans. - Drop test - Brake test - Retrieval - Check equipment for wastage - Check condition of hydraulic hoses | 46 CFR 96.07 ABS Rules part2, chap 2 |

Notes:

Task 6: Auxiliary Machinery and Machinery Spaces

| Step | Action | Ref |
| --- | --- | --- |
| 6.1 |  Inspect ventilation of machinery installations. - Closure - Remote shut-offs - Cleanliness of vents/intakes | MSM II Ch.1 MSM II Ch.0 |
| 6.2 |  Inspect fuel systems. - Flame screen - Flexible hose meets SAE J-1942 - Material fuel piping - Shutoff valves installed |  |
| 6.3 |  Test internal combustion engines. - Operating within parameters - Flexible hose that meets SAE J-1942 doesn’t exceed 30” in length | 46 CFR 32.35.5 46 CFR 35.35-85 46 CFR 58.10 |
| 6.4 |  Inspect machinery guards. | 46 CFR 58.01-20 |
| 6.5 |  Inspect watertight bulkheads and doors. - Bulkhead penetrations - Material condition - Welds - Gaskets/knife edges/dogs/welds/alignment |  |
| 6.6 |  Inspect nonmetallic piping materials. - Flexible hose meets SAE J-1942 device - Plastic pipe not in a concealed space - Shell valves | 46 CFR 56.60-25 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 6.7 |  Test bilge pumps. (not required on manned barges) | 46 CFR 32.52 |
| 6.8 |  Inspect bilge piping. - Manifolds/valves/strainers/piping - Remote shut-offs | 46 CFR 56.50-50 |
| 6.9 |  Verify air compressor is installed properly. | 46 CFR 32.35-15 |
| 6.10 |  Test pressure vessels. (hydrostatically or internally tested) Service; MAWP; Date tested or Examined Internally; Relief Valve Tested. | 46 CFR 61.10 |
| 6.11 |  Verify that relief valves are set within range. - Up to 250 psi; reset within 10% - Above 250 psi; reset within 5% | 46 CFR54.15-10(g) 46 CFR 61.10-5(i) |
| 6.12 |  Inspect refrigeration and air-conditioning systems. | 46 CFR 58.20 |
| 6.13 |  Inspect ballast tanks. (noncargo tank) - Vent 2 inch nominal pipe size - Means of vent closing (example: ball check valve) (Exempt Inland Service) | 46 CFR 56.50-85 |

Notes:

Task 7: Electrical Equipment

| Step | Action | Ref |
| --- | --- | --- |
| 7.1 |  Inspect primary power and light system. |  |
| 7.2 | Inspect independent generators. - Primer mover must meet part 58 of CFR and sections 4/5 C2.15 and 4/5C2.15 of ABS rules for Building and Classing steel vessels - Circuit breaker protected - Reverse power relay - Ground detection system | 46 CFR 111.12-1 46 CFR 111.12-11 46 CFR 111.05-21 |
| 7.3 | Inspect batteries and alternator. (if stated) - Not in a sleeping space - Ventilation provided - Trays lined - Conductors/cables sizes - Over current protection provided - Terminals cleaned and protected | 46 CFR 111.15.2 46 CFR 111.15-5 46 CFR 111.15-10 46CFR 111.15-5 |
| 7.4 | Inspect switchboards and distribution panels. - Non-conductive deck coverings front/rear of switchboards - Nameplates for circuit designations - Circuit breaker amperage does not exceed nameplate rating - Test ground detection - Meters in good working order - Distribution panels have a circuit directory - Distribution panels have an enclosure (door) | 46CFR 111.30-11 46 CFR 111.30-15 46 CFR 111.12-11 46 CFR 111.77-1 46 CFR 111.05-23 46 CFR 111.30-25 46 CFR 111.40-11 46 CFR 111.40-5 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 7.5 | Inspect cable, wiring, receptacles, outlets, and accessories. - Spot check for proper cable size - Ensure that splices are only made in cable size 12 AWG or greater - Check for splices located in Hazardous Atmosphere locations - Visually check that wires/cables are properly insulated (no naked wiring/cables) - Twist-on type connectors (wire nuts) are not located in an enclosure and/or not used as spliced joints. - Receptacles/outlets have covers and no exposed live parts - Receptacles/outlets are mounted in outlet boxes | 46 CFR 111.60-4 46 CFR 111.60-13 46 CFR 111.60.19 46 CFR 111.60 46 CFR 111.60 46 CFR 111.79.1 46 CFR 111.81-1 46 CFR 111.81-1 |
| 7.6 | Inspect miscellaneous motors and controllers. - Enclosures located in hazardous locations meet the requirements of 46 CFR 111.105 - Wiring schematic/diagrams of the controller is located on the door interior. | 46 CFR 111.70-3 |
| 7.7 | Inspect emergency lighting. - Emergency lights are marked with “E” - Emergency lighting is provided in the required spaces - Light fixtures located in hazardous locations are explosion proof - Emergency lighting is operational - Ensure general lighting switch will not secure emergency lights - Witness the emergency lights automatically energize upon loss of power to general lighting, if equipped to do so | 46 CFR 97.37 46 CFR 112.15-1 46 CFR 111.105-31 46 CFR 111.105-32 46 CFR 112.43-1 |

Notes:

Task 8: Pollution Prevention Systems

| Step | Action | Ref |
| --- | --- | --- |
| 8.1 |  Verify that the International Convention for the Prevention of Pollution from Ships (MARPOL) placard is posted. | 33 CFR 155.450 |
| 8.2 |  Inspect machinery space or bilge/ballast control. |  |
| 8.3 |  Verify that bilges are free of oil and trash/debris. |  |
| 8.4 | Inspect the marine sanitation device and associated equipment. - Marine sanitation device - Type I - Type II - Type III - Certified for inspected vessels. - Capacity satisfactory - Installation - Operation - Ventilation - Writing and piping - Maintenance - Placard posted - Safety - Accessibility to parts requiring routine servicing - Manufacturer’s instructions available | 33 CFR 159.55 MSM II Ch.B.6.F 33 CFR 159.57 33 CFR 159.59 |
| 8.5 |  Verify person-in-charge designation. | 33 CFR 155.700 |
| 8.6 |  Inspect fuel oil/cargo oil containment systems. | 33 CFR 155.320 |
| 8.7 |  Inspect tank level or pressuring monitor device. |  |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 8.8 |  Inspect oil water/waste retention. |  |
| 8.9 |  Inspect oily bilge/ballast discharge. (oceangoing, 400 gross tons discharge shore connection or portable adapter) | 33 CFR 155.330 33 CFR 155.350 |
| 8.10 |  Inspect prohibited oil spaces. | 33 CFR 155.470 |
| 8.11 |  Inspect discharge removal equipment spill kit. | 33 CFR 155.210 |
| 8.12 |  Inspect emergency lightering equipment. - Single-hulled >54,000 gross tons carrying oil - Permanently marked with sizes - One extra spare set must be onboard | 33 CFR 157.410 |
| 8.13 |  Verify that shipboard garbage is properly disposed. (oceangoing manned only) | 33 CFR 151.63 |
| 8.14 |  Verify that the proper placards are in place. | MARPOL Ax.V/3 |
| 8.15 |  Conduct MARPOL survey. - Discharge of cargo residue - Approved monitoring and control system | 33 CFR 151.09 |
| 8.16 |  Conduct MARPOL survey. - Discharge of cargo residue - Approved monitoring and control system | 33 CFR 151.30 |
| 8.17 |  Inspect barges that ballast cargo tanks. - Pumping, piping, and discharge arrangements - Designated observation area - Slop tank - Cargo and ballast information - Instruction manual | 33 CFR Part 157 33 CFR 157.11 33 CFR 157.13 33 CFR 157.15 33 CFR 157.23 33 CFR 157.49 |

Notes:

Task 9: Observe Drills (Manned Barges Only)

| Step | Action | Ref |
| --- | --- | --- |
| 9.1 |  Observe fire drills. | 46 CFR 97.15 46 CFR 199.180 |
| 9.2 |  Observe abandon ship drills. | 46 CFR 97.15 46 CFR 199.180 |
| 9.3 |  Observe man overboard drills. | 46 CFR 97.15 46 CFR 199.180 |
| 9.3 |  Observe security drills. | 33 CFR 104.230 |

Task 10: Tank Barge Cargo Operations

| Step | Action | Ref |
| --- | --- | --- |
| 10.1 |  Inspect cargo tanks. - Trunks/hatches - Ullage openings - Liquid level gauges (open, restricted, closed) - Deck penetrations - Heating oils - Explosion-proof electrical fittings - Overfill device | 46 CFR 151.15-10 46 CFR 39.20-3 46 CFR 32.50-15 46 CFR 111.105 33 CFR 155.480 |
| 10.2 | Inspect cargo tank ventilation. - Common header system - P/V valves - Flame arrestors - Flush and drain connections - Independent PV valves - Flame screen - Valve material (dangerous cargoes) - Independent goosenecks - Flame screen - Closure device | 46 CFR 32.60-20 45 CFR 36.20-5 46 CFR 34.05-05 |
| Notes: |  |  |

| Step | Action | Ref |
| --- | --- | --- |
| 10.3 |  Inspect piping and valves. - Expansion joints - Valve controls - Supports - Flanges - Containment - Materials | 46 CFR 32.50-15 33 CFR 155-310 46 CFR 151.56 |
| 10.4 |  Inspect hoses. - External examination - Hydrostatic test - Markings - Test dates - Hose log (if available) | 46 CFR 56.04 46 CFR 32.50-15 33 CFR 155.310 46 CFR 151,56 |
| 10.5 |  Inspect pumps and engines. - Controls and shutdowns - Relief valves - Gauges - Engine fuel system - Spark arrestor - Cover | 46 CFR 32.50-30 33 CFR 155.800 33 CFR 156.170 33 CFR 154.500 |
| 10.6 |  Inspect pump rooms. - Electrical insulation - Ventilation - Bulkhead penetrations - Gas-tight boundaries - Cofferdams - Fire extinguishing system | 46 CFR 91.25-37 46 CFR 151.15-10 46 CFR 39.20-3 46 CFR 32.50-15 46 CFR 111.105 33 CFR 155.480 |

| Notes: |  |  |
| --- | --- | --- |
| Step | Action | Ref |
| 10.7 |  Inspect independent tanks, fixed, portable, or marine portable. - External exam - Date of internal exam - Date of hydrostatic test - Metal information plate - Marking and labeling - Saddles: foundations and stowage - Piping and valves - Lifting fittings - Securing devices - Pumps and controls - Cargo hoses - Electrical grounding - Firefighting requirements - Authorized cargo | 46 CFR 32.20-5 46 CFR 32.20-10 46 CFR 151.15-5 46 CFR 32.55-25 46 CFR 32.20-10 46 CFR 151.56 |
| 10.8 |  Conduct external examination of inert gas systems. |  |
| 10.9 |  Ensure proper ventilation system for cargo carried. | 46 CFR 32.20-5 |
| 10.10 |  Verify warning notices and signals are posted. |  |
| 10.11 |  Inspect explosion-proof fixtures in the destination hazardous area. |  |
| 10.12 |  Inspect weather decks. |  |
| 10.13 |  Inspect air compressor intakes. |  |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 10.14 |  Inspect tanks for liquefied flammable gas having lethal characteristics or having dangerous cargo. - Markings - Lagging and fire protection - Manholes - Piping - Fittings - Gauges - Valves - Controls - Fill and vent - Foundations and supports Type of Examination/Test; Date of Examination/Test. Internal Examination. External Examination (lagging removed). Safety Valve test. Hydrostatic test. | 46 CFR 38.01-1 |

Notes:

Task 11: Thermal Fluid Heaters

| Step | Action | Ref |
| --- | --- | --- |
| 11.1 | Conduct a visual inspection of the following: - Combustion chamber - Refractory - Exhaust - Heat exchanger - Pumps - Piping | 46 CFR 61.30-15 |
| 11.2 |  Verify that a copy of the test procedures is on board. | 46 CFR 63.10-1 |
| 11.3 |  Witness the operational test of the following: - Pre-purge - Ignition sequence - Combustion controls - Flame safeguards - Limit controls (low fluid output, low flow cutout, high temperature cutout) - Post-purge - COI endorsed to carry cargo - Pump relief valve - Flame screens - Ventilation of the pump room | 46 CFR 61.30-20 46 CFR 62.25-5 |
| 11.4 |  Verify that the COI is endorsed to carry elevated cargo. |  |
| 11.5 |  Inspect the cargo pump relief valve. |  |
| 11.6 |  Inspect the flame screens. |  |
| 11.7 |  Inspect the ventilation of the pump room. |  |

Notes:

# Task 12: Cargo Tank Internals

| Step | Action | Ref |
| --- | --- | --- |
| 12.1 | Verify Marine Chemist Certificate. - Marine Chemist No.____________________ - Certificate No.________________________ - Date Issued __________________________ - Location _____________________ - Last three cargos ____________________ - All spaces certified “Safe for Workers” that are to be entered - MCC maintained by competent person | MSM II A.5.H NFPA 306 |
| 12.2 |  Verify cargo gauging type is correct for authorized cargo. - Open - Restricted - Closed |  |
| 12.3 |  Inspect overfill device installed. |  |
| 12.4 |  Inspect all cargo tanks. - Welds - Bulkheads - Framing - Sumps - Pumps | MSM II Ch.B.3.B.4 NVIC 7-68 |
| 12.5 |  Inspect cargo brackets, piping, valves, and reach rods. |  |
| 12.6 |  Inspect openings of vents/piping/valves. |  |
| 12.7 |  Inspect Pressure Vessel Cargo Tanks. - Required NDT for PVCTs 30 yrs and older having a 10-yr internal exam period - Owner submit proposal meeting guidance in NAVIC 5-89 - Testing conducted by personnel qualified and certified in nondestructive testing. | 46 CFR 38.25-1 NVIC 5-89 46 CFR 151.04-7 46 CFR 98.25.97 |

Notes:

Task 13: New Construction and Modified Vessels

| Step | Action | Ref |
| --- | --- | --- |
| 13.1 |  Verify approved plans. - General arrangements - Hull structure - Subdivision and stability - Electrical systems - Hazardous locations 5 - Cargo gear - Fire control and systems | 46 CFR 31.01-5 46 CFR 31.10 46 CFR 31.10-5 46 CFR 110.25 46 CFR 91.20-10 46 CFR 91.40- 46 CFR 91.55 46 CFR 91.55-5 |
| 13.2 |  Inspect all items ensuring they are constructed or installed as detailed in the approved plans. |  |
| 13.3 |  Witness testing of all welds. | ABS Rules |
| 13.4 |  Witness tank testing. - Double bottom tank - Double side tank - Cargo oil tank - Fore peak void - Aft peak void (air test) - Cofferdams - Watertight bulkhead-hose test - Chemical cargo tank |  |
| 13.5 |  Inspect all repairs and modifications. | 46 CFR 31.10-35 46 CFR 91.45-1 |

Notes:

Task 14: Chemical Gas Cargo Operations

| Step | Action | Ref |
| --- | --- | --- |
| 14.1 |  Inspect containment system. | 46 CFR 151.45 |
| 14.2 |  Verify that the COI is endorsed to carry 151 cargo. | 46 CFR 151.45-2(d) |
| 14.3 |  Inspect cargo tanks. - Gravity tanks - Press vessel - External inspection - NDT | 46 CFR 151.01-10 46 CFR 151.04-5 46 CFR 151.15 46 CFR 151.25-1 |
| 14.4 |  Inspect insulation. | 46 CFR 151.15-3(g) |
| 14.5 |  Inspect valves and handling equipment. | 46 CFR 151.04-5(f) |
| 14.6 |  Verify cargo special requirements. | 46 CFR 151.50 |
| 14.7 |  Inspect pump rooms. | 46 CFR 151.56-1 |
| 14.8 |  Inspect tank venting. | 46 CFR 151.25-2 |
| 14.9 |  Inspect tank venting. | 46 CFR 151.15-5 |
| 14.10 |  Examine pressure vacuum relief valve. | 46 CFR 151.25-2 |
| 14.11 |  Examine safety relief valve. | 46 CFR 151.04-5(h) |
| 14.12 |  Conduct external examination of the inert gas system. |  |
| 14.13 |  Inspect the gauging system. | 46 CFR 151.15.10 |
| 14.14 |  Inspect the tank overflow control. | 46 CFR 151.45-6 |
| 14.15 |  Inspect temperature control systems. | 46 CFR 151.20-10 46 CFR 151.40 |
| 14.16 |  Inspect emergency equipment. |  |
| 14.17 |  Inspect toxic vapor detectors. |  |

| Notes: |  |  |
| --- | --- | --- |
| Step | Action | Ref |
| 14.18 |  Inspect the Vapor Control System (VCS). | 46 CFR 151.20-20 |
| 14.19 |  Inspect cargo transfer procedures. | 46 CFR 151.45 |
| 14.20 |  Inspect cargo hose. - Inspected every 2 years (visual exam hydro-tested one and half times) - Test pressure is marked on the hose | 46 CFR 151.20-15 46 CFR 151.04-5(j) |
| 14.21 |  Inspect the benzene or butyl acrylate cargos. | 46 CFR 151.05-2 46 CFR 151.05 |
| 14.22 |  Inspect piping and valves. | 46 CFR 151.20 |
| 14.23 |  Verify warning sign are posted. - Dangerous cargo - No visitors - No smoking - No open lights | 46 CFR 151.45-2(e) |

Notes:

Task 15: Bulk Liquefied Flammable Gases Cargo Operations

| Step | Action |  | Ref |
| --- | --- | --- | --- |
| 15.1 |  Inspect secondary barrier cargo tank. |  | 46 CFR 38.05 |
| 15.2 |  Inspect cargo piping. |  | 46 CFR 38.10 |
| 15.3 |  Inspect pump and compressor rooms. |  |  |
| 15.4 |  Inspect pressure relief valves. |  | 46 CFR 38.10 46 CFR 38.20-1 46 CFR 38.25-10 46 CFR 38.10-1(g) |
| 15.5 |  Verify maximum allowable relief valve settings for cargo tanks. |  | 46 CFR 38.05-2 |
| 15.6 |  Inspect cargo hoses. |  | 46 CFR 38.15-5 |
| 15.7 |  Inspect cargo vent systems. | 46 CFR 38.05-1(b) 46 CFR 38.01-2(a) |  |
| 15.8 |  Verify electrical safety in gas-dangerous spaces or zones. | 46 CFR 38.15-15 |  |
| 15.9 |  Inspect temperature-measuring devices. | 46 CFR 38.10-1(g) |  |
| 15.10 |  Inspect safety equipment. | 46 CFR 38.10-1(j) |  |
| 15.11 |  Verify marking is labeled vapor or liquid. | 46 CFR 38.05-05(b) |  |
| 15.12 |  Inspect cargo-gauging system. | 46 CFR 38.10-20 |  |
| 15.13 |  Inspect liquid overfill protection systems. |  |  |
| 15.14 |  Inspect insulation. | 46 CFR 38.05 |  |
| 15.15 |  Inspect high and low vapor pressure protection systems. |  |  |
| 15.16 |  Inspect operations remote shutdown. | 46 CFR 38.15.20 |  |
| 15.17 |  Inspect cargo tanks. | 46 CFR 38.05-10(f) 46 CFR 38.25 |  |
| 15.18 |  Inspect marine portable tanks. | 46 CFR 38.01-2 46 CFR 38.01-5 |  |

Notes:

Task 16: Vapor Control Systems

| Step | Action | Ref |
| --- | --- | --- |
| 16.1 |  Inspect piping. - Drain lines and plugs - Electrically bonded to hull - Last 3.3 feet painted red/yellow/red and labeled “vapor” in 2-inch black letters - Internally examine piping if Polymar cargos authorized | 46 CFR 39-20-1 |
| 16.2 |  Inspect closed gauging system. | 46 CFR 39.20-3 |
| 16.3 | Inspect liquid overfill protection. - High level and tank overfill alarms - Self-contained power supply - Install generator or shore tie - Alarm with automatic shutdown system - Independent of cargo gauging - Alarms and automatically shuts down at the facility - Operation checked at tank - Alarm junction box labeled “connector for barge overflow control system” - Spill Valve - Meets ASTM F1271 - Set pressure higher than relief - Valve and lower that rating of - Cargo tank - Approve by Commandant | 46 CFR 39.20-9 |

Notes:

| Step | Action | Ref |
| --- | --- | --- |
| 16.4 |  Inspect pressure vacuum protection. - Pressure set at not less than 1.0 psi - Vacuum set at not less than 0.5 psi - Methods to check that the device operates freely and does not remain open - Flame screen installed | 46 CFR 39.20-11 |
| 16.5 |  Verify the following: - Pressure drop calculations provided - System included in oil transfer procedures - Approval letter from Marine Safety Center. | 46 CFR 39.30-1 |

Notes:

Task 17: MTSA Compliance

Vessel Security Plans (VSP):

(If using ASP, skip to next section)

| Step | Action | Ref |  |
| --- | --- | --- | --- |
| 17.1; - Compliance documentation - Approved Vessel Security Plan; 33 CFR 104.120. 17.2; - Waiver - Approved by CG-543; 33 CFR 104.130. 17.3; - Equivalents - Approved by CG-543; 33 CFR 104.135. 17.4; - Maritime Security (MARSEC) directive - Proper safeguards - Incorporated into VSP; 33 CFR 104.145. 17.5; - Company Security Officer (CSO) - Training / experience - Valid TWIC - See list of example questions; 33 CFR 104.210. 17.6; - Vessel Security Officer (VSO) - Training / experience - Valid TWIC - See list of example questions; 33 CFR 104.215. 17.7; - Company or vessel personnel with security duties - Training / experience - Valid TWIC - See list of example questions; 33 CFR 104.220. 17.8; - Security Training for all other vessel personnel (manned barges only); 33 CFR 104.225. Step; Action; Ref. 17.9; - Vessel Record Keeping Requirements - Training - Drills and exercises - Breaches of security - Change in MARSEC levels - Maintenance, calibration, and testing of security equipment. - Security threats - Annual audit of the VSP - Declaration of Security (DoS) - Retained for Two years; CFR 104.235. 17.10; - MARSEC level coordination and implementation - Proper MARSEC level - MARSEC level at least at current port level; 33 CFR 104.240. 17.11; - Communications - Vessel security personnel - Facility - National and local authorities - Demonstrate communications operations; 33 CFR 104.245. 17.12; - Declaration of Security (DoS) - Required for cruise ships or manned CDC bulk vessels and any vessel or facilities with which it interfaces. - Valid (for MARSEC level and effective time period) Must have last 10 or continuous DoS reviewed at interval consistent with MARSEC level. - Signed; 33 CFR 104.255. 17.13; - Security systems and equipment maintenance - Testing completed IAW manufacturer’s recommendations - Working properly, effectively functions IAW VSP. - Ship Security Alert System (SSAS); 33 CFR 104.260. Step; Action; Ref. 17.14; - Security measures for access control - Access points examined – signs posted in conspicuous locations. - Control areas for authorized dangerous substances / devices - Means of identifying unauthorized personnel - TWIC for unescorted access to secure areas; 33 CFR 104.265. 17.15; - Security measures for newly hired employees (Pending receipt of TWIC) - Access permitted for up to 30 days if: - Has applied for TWIC - Accompanied by TWIC holder in secure areas - Operator enters new hire personal info in HOMEPORT - Notified via HOMEPORT that new hire has passed initial name check. - Provision does not apply to CSO, VSO or individual hired to perform security duties; 33CFR 104.267. 17.16; - Security measures for restricted areas - Secure areas protected - Properly marked - Control measures adequate - Do not conflict with safety measures; 33 CFR 104.270. 17.17; - Security measures for handling cargo - Identifying cargo tamper - Identifying approved cargo - Access point – inventory control - Checking cargo for dangerous substances; 33 CFR 104.275. 17.18; - Security measures for delivery of vessel stores and bunker - Security procedures followed - Standing agreements valid; 33 CFR 104.280. Step; Action; Ref. 17.19; - Security measures for monitoring - IAW VSP - Lighting - Test intrusion alarms - Emergency search procedures; 33 CFR 104.285. 17.20; - Security Incident Procedures - Witness during drill; 33 CFR 104.290. - Amendment and audit - CSO / VSO audit letter attached to ASP - Audits conducted as required (annually or after vessel modifications); 33 CFR 104.415. |

Notes:

# Alternative Security Program (ASP):

Vessels operating under the auspices of an approved ASP are required to address the relevant areas cited in 33 CFR parts 104. However, the ASP provision of the rule has provided a mechanism by which segments of the maritime industry, through application by the industry associations or other representative groups, are able to tailor their program to the unique functions inherent of their specific operations. The result is a set of relevant, performance-based security measures for the industry groups choosing to utilize an approved ASP. For this reason, the inspector of a vessel using an approved ASP may find that certain language or security measures contained in some parts of the rule will differ from the language or security measures listed in the ASP.

In those cases where both the vessels and the facilities serving those vessels are owned and/or operated by the same entity, an alternative plan may recognize that the same party is responsible for security in both areas and approve an approach that addresses vulnerabilities and mitigation strategies for the vessels and the facility under one ASP. Therefore, the inspector will not be using separate plans for the vessels and the facility to determine compliance and, likewise, will not see some citations addressed in the plan if they are redundant between 33 CFR 104 and 33 CFR 105.

|  | Step | Action |  |  |  |  | Ref |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 17.1 | - Name of approved ASP - ___________________________ |  |  |  |  | 33 CFR 104.140 |  |  |  |  |  |  |
|  | 17.2 | - Compliance Documentation - Copy of ASP aboard vessel - Vessel Specific Security Assessment (VSA) completed - Letter to MSC stating use of approved ASP and that it has been fully implemented |  |  |  |  | 33 CFR 104.120 |  |  |  |  |  |  |
|  | 17.3 | - Maritime Security (MARSEC) directive - Proper safeguards - Incorporated into VSP |  |  |  |  | 33 CFR 104.145 |  |  |  |  |  |  |
|  | 17.4 | - Company Security Officer (CSO) - Training / experience - Valid TWIC - See list of example questions |  |  |  |  | 33 CFR 104.210 IAW ASP |  |  |  |  |  |  |
|  | 17.5 | - Vessel Security Officer (VSO) - Training / experience - Valid TWIC - See list of example questions |  |  |  | 33 CFR 104.215 IAW ASP |  |  |  |  |  |  |  |
|  | 17.6 | - Company or vessel personnel with security duties - Training / experience - Valid TWIC - See list of example questions |  |  |  | 33 CFR 104.220 IAW ASP |  |  |  |  |  |  |  |
|  | 17.7 | - Security Training for all other vessel personnel (manned barges only) |  |  |  | 33CFR 104.225 IAW ASP |  |  |  |  |  |  |  |
|  | Step |  | Action |  |  |  |  | Ref |  |  |  |  |  |
|  | 17.8 | - Vessel Record Keeping Requirements - Training - Drills and exercises - Breaches of security - Change in MARSEC levels - Maintenance, calibration, and testing of security equipment. - Security threats - Annual audit of the VSP - Declaration of Security (DoS) - Retained for Two years |  |  |  | 33 CFR 104.235 IAW ASP |  |  |  |  |  |  |  |
|  | 17.9 | - MARSEC level coordination and implementation - Proper MARSEC level - MARSEC level at least at current port level |  |  |  | 33 CFR 104.240 IAW ASP |  |  |  |  |  |  |  |
|  | 17.10 | - Communications - Vessel security personnel - Facility - National and local authorities - Demonstrate communications operations consistent with the ASP |  |  |  | 33 CFR 104.245 IAW ASP |  |  |  |  |  |  |  |
|  | 17.11 | - Declaration of Security (DoS) - Required for cruise ships or manned CDC bulk vessels and any vessel or facilities with which it interfaces. - Valid (for MARSEC level and effective time period) Must have last 10 or continuous DoS reviewed at interval consistent with MARSEC level. - Signed |  |  |  | 33 CFR 104.255 ISPS, Part A, 5.1 |  |  |  |  |  |  |  |
|  | 17.12 | - Security systems and equipment maintenance - Testing completed IAW manufacturer’s recommendations - Working properly, effectively functions IAW ASP. |  |  |  | 33 CFR 104.260 |  |  |  |  |  |  |  |
| Step |  |  |  |  | Action |  |  |  |  | Ref |  |  |  |
|  | 17.13 | - Security measures for access control - Access points examined – signs posted in conspicuous locations. - Control areas for authorized dangerous substances / devices - Means of identifying unauthorized personnel - TWIC for unescorted access to secure areas |  |  |  | 33 CFR 104.265 |  |  |  |  |  |  |  |
|  | 17.14 | - Security measures for newly hired employees (Pending receipt of TWIC) - Access permitted for up to 30 days if: - Has applied for TWIC - Accompanied by TWIC holder in secure areas - Operator enters new hire personal info in HOMEPORT - Notified via HOMEPORT that new hire has passed initial name check. - Provision does not apply to CSO, VSO or individual hired to perform security duties |  |  |  | 33CFR 104.267 |  |  |  |  |  |  |  |
|  | 17.15 | - Security measures for restricted areas - Secure areas protected - Properly marked - Control measures adequate - Do not conflict with safety measures |  |  |  | 33 CFR 104.270 |  |  |  |  |  |  |  |
|  | 17.16 | - Security measures for handling cargo - Identifying cargo tamper - Identifying approved cargo - Access point – inventory control - Checking cargo for dangerous substances |  |  |  | 33 CFR 104.275 |  |  |  |  |  |  |  |
|  | 17.17 | - Security measures for delivery of vessel stores and bunker - Security procedures followed - Standing agreements valid |  |  |  | 33 CFR 104.280 |  |  |  |  |  |  |  |
|  | Step |  |  | Action |  |  |  |  | Ref |  |  |  |  |
|  | 17.18 | - Security measures for monitoring - IAW ASP - Lighting - Test intrusion alarms - Emergency search procedures |  |  |  | 33 CFR 104.285 |  |  |  |  |  |  |  |
|  | 17.19 | - Security Incident Procedures - Witness during drill |  |  |  | 33 CFR 104.290 |  |  |  |  |  |  |  |
|  | 17.20 | - Amendment and audit - CSO / VSO audit letter attached to ASP - Audits conducted as required (annually or after vessel modifications) |  |  |  | 33 CFR 104.415 |  |  |  |  |  |  |  |

Should an enforcement inspection reveal that an owner/operator has correctly implemented an approved ASP in its entirety but security vulnerabilities exist in the vessel operation, the COTP shall be advised.  Under 33 CFR 104.415 (a) (ii) for vessels or 33 CFR 105.415 (a) (ii) (f), the Coast Guard can determine that an amendment is necessary and advise the organization that submitted the ASP for approval accordingly. Following such notification, it will be necessary for the original submitting organization to provide their proposed amendment to the Commandant (CG-543) for review and approval. If the submitting organization does not wish to amend the ASP, the vessel owner must submit a VSP for the vessel to the MSC.

Notes:

Section 7: Post Inspection

| Step | Action | Ref |
| --- | --- | --- |
| 1.1 |  Issue or endorse letters, certificates, waivers, exemptions, and extensions. (if necessary) - Permit to proceed - Temporary COI - CG 835 | MSM II Sec (A) Ch. 3 46 CFR 31.10-35 46 CFR 91.05 46 CFR 91.01-15 |
| 1.2 |  Complete MISLE entries. |  |
| 1.3 |  Initiate Report of Violation (if necessary). | MSM II Sec (A) Ch. 3 |

Notes:

Recommended U.S. Vessel Deficiency Procedures:

Use the following table to complete deficiency issues.

| Step | Action |
| --- | --- |
| 1. | Identify deficiency. |
| 2. | Inform vessel representative. |
| 3. | Record on the Deficiency Summary Worksheet (next page). |
| 4. | If deficiency is corrected prior to end of inspection, go to step 6. |
| 5. | If deficiency is unable to be corrected prior to end of inspection, issue CG-835 in accordance with table below. If deficiency...; THEN issue CG-835.... Does not immediately affect crew/passenger safety, hull seaworthiness, or the environment, e.g., missing placards.; That provides a specific time for correcting deficiency, e.g., “x” number of days.. Allows vessel operations to be MODIFIED to meet less stringent requirements, e.g. P/V valves fail to seal properly.; That restricts operation of vessel to meet current vessel conditions, e.g., reduce cargo grade. DOES immediately affect crew/passenger safety, hull seaworthiness, or the environment, and cannot be modified to meet less stringent requirements, e.g., structural defect or damage.; That requires the deficiency to be corrected prior to operating vessel (“NO SAIL” item), e.g., prior to carrying cargo. |
| 6. | Enter information into MISLE. |
| 7. | Initiate Report of Violation (ROV) if necessary. |

Deficiency Summary Worksheet

| Name of Vessel |  | VIN |
| --- | --- | --- |

| Deficiency |  |  | Req’t Issued/ Date Completed |
| --- | --- | --- | --- |

Notes:

Notes:

Conversions:

| Distance and Energy: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kilowatts (kW) |  |  |  |  | X |  |  |  |  | 1,341 |  |  |  |  |  | = |  |  |  |  | Horsepower (hp) |  |  |  |  |  |
| Feet (ft) |  |  |  |  | X |  |  |  |  | 3.281 |  |  |  |  |  | = |  |  |  |  | Meters (m) |  |  |  |  |  |
| Long Ton (LT) |  |  |  |  | X |  |  |  |  | .98421 |  |  |  |  |  | = |  |  |  |  | Metric Ton (t) |  |  |  |  |  |
| Liquid: (NOTE: Values are approximate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Liquid |  |  |  |  | bbl/LT |  |  |  |  | m3/t |  |  |  |  |  | bbl/m3 |  |  |  |  |  | bbl/t |  |  |  |  |
| Freshwater |  |  |  |  | 6.40 |  |  |  |  | 1.00 |  |  |  |  |  | 6.29 |  |  |  |  |  | 6.29 |  |  |  |  |
| Saltwater |  |  |  |  | 6.24 |  |  |  |  | .975 |  |  |  |  |  | 6.13 |  |  |  |  |  | 5.98 |  |  |  |  |
| Heave Oil |  |  |  |  | 6.77 |  |  |  |  | 1.06 |  |  |  |  |  | 6.66 |  |  |  |  |  | 7.06 |  |  |  |  |
| DFM |  |  |  |  | 6.60 |  |  |  |  | 1.19 |  |  |  |  |  | 7.48 |  |  |  |  |  | 8.91 |  |  |  |  |
| Lube Oil |  |  |  |  | 7.66 |  |  |  |  | 1.20 |  |  |  |  |  | 7.54 |  |  |  |  |  | 9.05 |  |  |  |  |
| Weight: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Long Ton |  |  |  |  | = 2240 lb |  |  |  |  |  |  |  |  | 1 Metric Ton |  |  |  |  |  |  | = 2204 lb |  |  |  |  |  |
| 1 Short Ton |  |  |  |  | = 2000 lb |  |  |  |  |  |  |  |  | 1 Cubic Foot |  |  |  |  |  |  | = 7.48 gal |  |  |  |  |  |
| 1 Barrel (oil) |  |  |  |  | = 5.61 ft =42 gal = 6.29 |  |  |  |  |  |  |  |  | 1 psi |  |  |  |  |  |  | =.06895 Bar = 2.3106 ft of water |  |  |  |  |  |
| Temperature: Fahrenheit = Celsius ( F= 9/5 C+32 and C=5/9 ( F-32)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | = |  |  | -17.8 |  |  |  |  | 80 |  |  | = |  |  | 26.7 |  |  |  |  | 200 |  |  |  | = |  | 93.3 |
| 32 | = |  |  | 0 |  |  |  |  | 90 |  |  | = |  |  | 32.2 |  |  |  |  | 250 |  |  |  | = |  | 121.1 |
| 40 | = |  |  | 4.4 |  |  |  |  | 100 |  |  | = |  |  | 37.8 |  |  |  |  | 300 |  |  |  | = |  | 148.9 |
| 50 | = |  |  | 10.0 |  |  |  |  | 110 |  |  | = |  |  | 43.3 |  |  |  |  | 400 |  |  |  | = |  | 204.4 |
| 60 | = |  |  | 15.6 |  |  |  |  | 120 |  |  | = |  |  | 48.9 |  |  |  |  | 500 |  |  |  | = |  | 260 |
| 70 | = |  |  | 21.1 |  |  |  |  | 150 |  |  | = |  |  | 65.6 |  |  |  |  | 1000 |  |  |  | = |  | 537.8 |
| Pressure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Bar |  | = | 14.5 psi |  |  |  |  | 5 Bars |  |  | = |  | 72.5 psi |  |  |  |  |  | 9 Bars |  |  |  | = |  | 130.5 psi |  |
| 2 Bars |  | = | 29.0 psi |  |  |  |  | 6 Bars |  |  | = |  | 87.0 psi |  |  |  |  |  | 10 Bars |  |  |  | = |  | 145.0 psi |  |
| 3 Bars |  | = | 43.5 psi |  |  |  |  | 7 Bars |  |  | = |  | 101.5 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Bars |  | = | 58.0 psi |  |  |  |  | 8 Bars |  |  | = |  | 116.0 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |