United States Coast Guard

HULL
INSPECTION BOOK

| ## Name of Vessel |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## Official Number |  |  | ## Class |  |
| ## Date Completed |  |  | ## Location |  |
| - Vessel Built in Compliance with SOLAS: 60 74 74/78 N/A |  |  |  |  |
| ## Route |  |  |  |  |
| - Oceans | - Limited Coastwise |  |  | - Lakes / Bays / Sounds |
| - Coastwise | - Great Lakes |  |  | - Rivers |
| ## Inspection Type |  |  |  |  |
| - Inspection for Certification (COI) - Annual Inspection - Periodic Inspection |  | - Reinspection - First - Second Passenger vessels only - Third |  |  |
| ## Inspectors 1. 2. |  | 3. 4. |  |  |

CG-840 HI
Rev: .Oct 2009

Total Time Spent Per Activity:

| Regular Personnel (Active Duty) |  |  |  |
| --- | --- | --- | --- |
| ACTIVITY TYPE | ACTIVITY | TRAINING | (PERS) MI |

| TOTAL ADMIN HOURS | TOTAL TRAVEL HOURS |
| --- | --- |

| Reserve Personnel |  |  |  |
| --- | --- | --- | --- |
| ACTIVITY TYPE | ACTIVITY | TRAINING | (PERS) MI |

| TOTAL ADMIN HOURS | TOTAL TRAVEL HOURS |
| --- | --- |

| ##### Auxiliary Resources |  |
| --- | --- |
| TOTAL BOAT HOURS | TOTAL AIRCRAFT HOURS |

Use of Hull Inspection Book:

This inspection book is intended to be used as a job aid by Coast Guard marine inspectors during hull inspections of U.S. flagged vessels.  The lists contained within this book are not intended to limit the inspection.  Each marine inspector should determine the depth of inspection necessary.  A checked box should be a running record of what has been inspected.  It does not imply that the entire system has been inspected or that all or any items are in full compliance.  This job aid does not constitute part of the official inspection record.

This document does not establish or change Federal laws or regulations.  References given are only general guides.  Refer to IMO publications, CFRs, NVICs, or any locally produced cite guides for specific regulatory references.  Not all items in this book are applicable to all vessels.

NOTE:  Guidance on how to conduct hull inspections of U.S. flagged vessels can be found in the Marine Safety Manual (MSM) Volume II, Chapter B1:  Inspection of Vessels for Certification.  All MSM cites listed in this book refer to MSM Volume II unless otherwise indicated.

| # Pre-inspection Items: | Post-inspection Items: |
| --- | --- |
| - Review MISLE records. - COI - Vessel Critical Profile | - Issue letters/certificates to vessel. |
| - Obtain copies of forms to be issued. | - Complete MISLE activity. |
|  | - Initiate Report of Violation (ROV) if necessary |

Table of Contents

Section 1:  Administrative Items

IMO Applicability Dates	1

Involved Parties & General Information	2

Vessel Information	4

Vessel Description	4

## Section 2:  Certificates and Documents

Certificates	9

Logs and Manuals	10

Pollution Prevention Records	11

Section 3:  Inspection Items

Navigation Equipment	12

General Health and Safety	15

Structural Integrity	16

Integral Cargo Tanks	18

Ground Tackle	20

Lifesaving Equipment	20

Fire Protection	26

Pollution Prevention	28

MTSA/ISPS	30

Section 4:  Drills

Fire Drill	40

Abandon Ship Drill	41

Security Drill	42

## Section 5:  Appendices

Recommended US Vessel Deficiency Procedures	43

Deficiency Summary Worksheet	44

Notes	45

Conversions	47

# Section 1:  Administrative Items

IMO Applicability Dates:

| Reference | # Date |
| --- | --- |
| SOLAS 1960 | 26 MAY 65 |
| SOLAS 1974 | 25 MAY 80 |
| 1978 Protocol to SOLAS 1974 | 01 MAY 81 |
| 1981 Amendments (II-1 & II-2) | 01 SEP 84 |
| 1983 Amendments (III) | 01 JUL 86 |
| Various additional amendments to SOLAS |  |
| MARPOL 73/78 Annex I MARPOL 73/78 Annex II MARPOL 73/78 Annex III MARPOL 73/78 Annex V MARPOL 73/78 Annex VI | 02 OCT 83 |
|  | 06 APR 87 |
|  | 01 JUL 92 |
|  | 31 DEC 88 19 MAY 05 |
| IBC Code | After 01 JUL 86 |
| BCH Code | Prior to 01 JUL 86 |
| COLREGS 1972 | 15 JUL 77 |
| Various additional amendments to COLREGS |  |
| Load Line 1966 | 21 JUL 68 |
| STCW 1978 | 28 APR 84 |
| 1991 Amendments 1994 Amendments 1995 Amendments | 01 DEC 92 01 JAN 96 01 FEB 97 |

Involved Parties & General Information:

Vessel’s Representatives:

Phone Numbers:

Owner—Listed on DOC (if applicable), or COFR:

- No Change

Operator

- No Change

Company Security Officer

- No Change

Vessel Security Officer

- No Change

Vessel Information:

| Classification Society |  |  |  |
| --- | --- | --- | --- |
| ISM Issuer: Same as above? |  |  |  |
| - Yes | - No If not the same, which Recognized Organization? |  |  |
| NOTE: The period of validity for ISM documents should correspond to the following list. If they do NOT, ISM documents should be further investigated. |  |  |  |
| - 5 years = Full term (SMS and DOC) - 6 months = Interim (SMC) |  | - 12 months = Interim (DOC) - 5 months = Short term (SMC) |  |
| Gross Tons |  |  | - No Change |
| Net Tons |  |  | - No Change |
| Built Date (use delivery date) |  |  | - No Change |
| Overall Length (in feet) |  |  | - No Change |

Vessel Description:

| - Container Vessel | - Ro/Ro Passenger Vessel |
| --- | --- |
| - Ro/Ro Cargo | - Oceanographic Research Vessel |
| - Bulk Carrier | - Nautical School Ship |
| - Passenger Vessel | - Other |

# Section 2:  Certificates and Documents

| Endorse Date |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Exp. Date |  |  |  |  |  |  |
| Issue Date |  |  |  |  |  |  |
| Port Issued |  |  |  |  |  |  |
| ID # |  |  |  |  |  |  |
| Issuing Agency | USCG |  | # USCG |  | # USCG |  |
| Name of Certificate | Certificate of Documentation - No Change | Classification Document - No Change | Certificate of Financial Responsibility (COFR) - No Change | Safety Construction (SLC) - No Change | Safety Equipment (SLE) - No Change | Safety Radio (SLT) - No Change |

| Endorse Date |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Exp. Date |  |  |  |  |  |  |
| Issue Date |  |  |  |  |  |  |
| Port Issued |  |  |  |  |  |  |
| ID # |  |  |  |  |  |  |
| Issuing Agency |  |  |  |  |  |  |
| Name of Certificate | Passenger Ship Safety (PSS) - No Change | International Load Line (ILL) - No Change | International Oil Pollution Prevention (IOPP) - No Change | International Tonnage (ITC) - No Change | Safety Management (SMC) - No Change | Document of Compliance (DOC) - No Change |

| Endorse Date |  |  |  |
| --- | --- | --- | --- |
| Exp. Date |  |  |  |
| Issue Date |  |  |  |
| Port Issued |  |  |  |
| ID # |  |  |  |
| Issuing Agency |  |  |  |
| Name of Certificate | International Ship Security Certificate (ISSC) - No Change | International Air Pollution Prevention Certificate (IAPP) - No Change | Engine International Air Pollution Prevention Certificate (EIAPP) - No Change |

Continuous Synopsis Record:

(SOLAS Vessels only)

Review Record and Enter Most Current Data

| Flag State: |  |
| --- | --- |
| Date Registered: |  |
| Ship ID #: |  |
| Ship Name: |  |
| Port of Registry: |  |
| Registered Owners: |  |
| Company-as defined in SOLAS Chapter IX: |  |
| Issuer ISM DOC: |  |
| Issuer ISM SMC: |  |
| Issuer ISSC: |  |

Certificates:

| - ISM Code - Safety Management System - Shipboard Operations Plan | SOLAS 74/78 IX/3 |
| --- | --- |
| - Stability letter posted in pilothouse | 46 CFR 78.12-1 46 CFR 97.11-1 ICLL Reg. 10(2) |
| - Annual drug and alcohol program audit | 46 CFR Part 16 |
| - Officers’ licenses current |  |
| - Transportation Worker ID Credential (TWIC) - All MMC holders - All non-MMC holders with security duties or unrestricted access to restricted areas. |  |
| - GMDSS endorsement - FCC Station License - Safety Radio Certificate | 47 CFR Part 80 |
| - GMDSS maintenance personnel - 1st Class Radiotelegraph Operator’s Certificate - 2nd Class Radiotelegraph Operator’s Certificate - 3rd Class Radiotelegraph Operator’s Certificate | 47 CFR 80.1074 |
| - Required international safety convention certificates posted and valid |  |
| - Liferaft servicing certificates - Annual service | 46 CFR 160.151-57(p) SOLAS 74/78 III/19.8 |
| - Cargo Gear Certificate | 46 CFR 91.37-75 |

Notes:

Logs and Manuals:

| - Lifesaving equipment maintenance record - Periodic checks as required - Visual inspection of survival craft/rescue boat and launching appliances - Operation of lifeboat/rescue boat engines - Lifesaving appliances, including lifeboat equipment examined |  | 46 CFR 199.190(e) SOLAS 74/78 III/19 |
| --- | --- | --- |
| - Emergency training and drills - Onboard training in use of lifesaving equipment (all crew members) - SOLAS training manual - Logbook records - Fire and lifeboat drills - General alarm tested |  | SOLAS 74/78 III/18 46 CFR 199.180 SOLAS 74/78 III/18.5 SOLAS 74/78 III/25 |
| - Bridge log - Pre-arrival tests conducted - Casualties (navigation equipment and steering gear failures reported) - Steering gear drills - Emergency steering drills |  | SOLAS 74/78 V/19 STCW 95 I/14 33 CFR 164.25 33 CFR 164.53 46 CFR 97.35 |
| - Stability information | 46 CFR 78.17-20 46 CFR 97.15-5 SOLAS 74/78 II-1/22.1 |  |
| - Information available to master (as required) - Loading manual - Trim and stability book | 46 CFR 78.17-22 46 CFR 97.12-1 |  |

Notes:

Pollution Prevention Records:

| - Oil record book (Part 1) - Each operation signed by person-in-charge - Each complete page signed by master - Book maintained for 3 years | MARPOL Ax. I/20 33 CFR 151.25 |
| --- | --- |
| - Shipboard oil pollution emergency plan - Approved by Coast Guard / class society - Contact numbers correct - Immediate Actions List | MARPOL Ax. I/26.1 33 CFR 151.26 |
| - Vessel response plan (vessels carrying oil as secondary cargo) - Approved by Coast Guard - Annual review by owner / operator | 33 CFR 155.1030 33 CFR 155.1045 33 CFR 155.1065 33 CFR 155.1070 |
| - Oil transfer procedures - Posted - List of products carried by vessel - Description of transfer system including a line diagram of piping - Number of persons required on duty - Duties by title of each person - Means of communication - Procedures to top off tanks - Procedures to report oil discharges | 33 CFR 155.720 |
| - Non-tank Vessel Response Plan - Approved by Coast Guard | 33 CFR 155 |

Notes:

# Section 3:  Inspection Items

Navigation Equipment:

| - Navigation publications (as applicable) - Current and corrected charts - U.S. Coast Pilot - Great Lakes Pilot - Sailing directions - Coast Guard Light List - Notice to mariners - Tide tables - Tidal current tables - International Rules of the Road - Inland Rules of the Road | 33 CFR 164.33 SOLAS 74/78 V/20 |
| --- | --- |
| - Operationally test radar(s) and ARPA - 2 required if over 10,000 GT - Operate independently - ARPA acquires targets | 33 CFR 164.35 33 CFR 164.37 33 CFR 164.38 SOLAS 74/78 V/12 |
| - Compasses - Illuminated gyrocompass with repeater at stand - Illuminated magnetic compass - Current deviation table | 33 CFR 164.35 SOLAS 74/78 V/12 |
| - Test electronic depth sounding device and recorder - Accurate readout - Test all transducers - Continuous recorder (chart) | 33 CFR 164.35 SOLAS 74/78 V/12 |
| - Speed and distance indicator | 33 CFR 164.40 SOLAS 74/78 V/12 |

Notes:

| - Propulsion shaft tachometer | SOLAS 74/78 V/12 |  |
| --- | --- | --- |
| - Voyage Data Recorder (VDR) - Simplified (SVDR) if permitted |  | SOLAS 74/78 V/20 |
| - Long Range Identification & Tracking (LRIT) - Conformance Test Report - Automatic Identification System (AIS) |  | 33 CFR 169 SOLAS 74/78 V19-1 33 CFR 164.46 |
| - Radio equipment - Radios, RDF, Loran - Electronic position fixing device tested - GMDSS meets requirements for vessel operating area |  | 33 CFR 26.03 SOLAS 74/78 V/7 SOLAS 74/78 V/12 33 CFR 164.41 NVIC 9-93 |

Notes:

| - Internal communications and control system - EOT failure alarms - Telephones - Voice tubes - Emergency loudspeaker system - Public address system - Bell pulls - Pilothouse controls | 46 CFR 113.30 46 CFR 113.35 46 CFR 113.50 |
| --- | --- |
| - Navigation lights and signals - Control panels - Running lights - Anchor lights - Special day and night signals - Distress signals and stowage - Flag signals, international code - Whistle, light, bells, gongs - Day and night signal devices - Certificate of Alternate Compliance | 72 COLREGS 46 CFR 199.60 33 CFR 81.9 |
| - Steering gear - Main gear tested (all stations) - Auxiliary gear tested (all stations) - Instructions and markings - Rudder angle indicator - Illumination - Alarms - Block diagram | 33 CFR 164.35 MSM Ch. C4.F 46 CFR 113.43 |
| - Maneuvering facts sheet with warning statements | 33 CFR 164.35 |

Notes:

| - EPIRB (406 MHz) - Float-free arrangement - Battery date current - Hydrostatic release | SOLAS 74/78 IV/7.1.6 46 CFR 199.510 |
| --- | --- |
| - GMDSS lifeboat radios (VHF) - 3 if over 500 GT - Operable condition | SOLAS 74/78 III/6.2 |
| - 9 GHz radar transponder (SART) - Freight vessels > 300 GT and < 500 require 1 - Freight vessels > 500 GT and passenger vessels require 2 - Stowed so to be rapidly placed in survival craft or stowed in survival craft | SOLAS 74/78 III/6.2 NVIC 9-93 |
| - NAVTEX | SOLAS 74/78 IV/7.1.4 |

General Health and Safety:

| - Hospital and first aid equipment | 46 CFR 72.20-35 46 CFR 92.20-35 |
| --- | --- |
| - Operating room explosion-proof | 46 CFR 111.105-37 |
| - Emergency lighting | 46 CFR 112.43 |
| - Protection of spaces specially suited for vehicles - Gas detection systems - Electrical hazardous locations | 46 CFR 70.10-44 46 CFR 90.10-38 |
| - Crew and passenger accommodations - Size - Lighting and wiring - Heating - Ventilation - Sanitation - Screens - Insulation - Fire retardant | 46 CFR 72.20 46 CFR 72.25 46 CFR 92.20 46 CFR 92.15-15 |

Notes:

| - Galley - Equipment - Sanitation - Ventilation | MSM Ch. B1.N.8 46 CFR 72.05-50 |
| --- | --- |
| - Means of escape from accommodation, machinery, and other spaces - Two required (some exceptions) - Dead end corridors - Absence of locks | 46 CFR 72.10 46 CFR 92.10 SOLAS 74/78 II-2/45 |
| - Gas freeing for repairs - Current Gas Chemist Certificate for areas as required - Date - Chemist No. | MSM Ch. A5.H |

Structural Integrity:

| - Hull structure (list inaccessible compartments or areas) - Decks - Shell - Bulkheads - Tank tops - Strength members | 46 CFR 72.01 46 CFR 92.01 ICLL 66 Reg. 1 |
| --- | --- |
| - Hull openings and closures - Side ports - Air ports and dead covers - Refuse chutes - Hatch covers - Closing devices, gaskets - Light / water test | 46 CFR 42.15 MSM Ch. B1.E.5 ICLL 66 Regs. 12 - 23 |
| - Deck openings and closures - Closing devices - Gaskets - Light / water test | MSM Ch. B1.E.5 ICLL 66 Regs. 12 - 20 |

Notes:

| - Guards, ladders, rails, and gangways (including accommodation ladders or pilot ladders) | 46 CFR 72.40 46 CFR 92.25 ICLL 66 Reg. 25 |
| --- | --- |
| - Elevators and escalators | 46 CFR 72.05-20 46 CFR 111.91 |
| - Watertight doors in subdivision bulkhead tested by: - Local control by hand - Local control by power - Remote control by hand - Indicators - Remote control by power - Indicators | 46 CFR 170.270 |
| - Cargo gear examined (in absence of Cargo Gear Certificate) - Records - Safe Working Load markings | 46 CFR 91.37 |
| - Exercise valves and controls - Bilge valves - Overboard discharge valves - Equalizing valves - Emergency shutoff valves - Scupper valves - Remote control - Reach rods | 46 CFR 78.17-5 |
| - Bilge wells, cofferdams, and suctions | 46 CFR 56.50-50 |
| - Bulkhead penetrations | MSM Ch. B1.E.6 |
| - Piping protection - Removable guards (where required) - Cargo hold - Baggage spaces | MSM Vol. IV, Ch. 3.G.2.b |

Notes:

| - Hull marks - Name - Hailing port - Official number - Net tonnage | 46 CFR 67.120 |
| --- | --- |
| - Draft marks - Legible - Properly sized - Properly spaced | 46 CFR 78.50-10 46 CFR 97.40-10 |
| - Load line marks - Conform to certificate - Legible | 46 CFR 78.50-15 46 CFR 97.40-15 ICLL 66 Reg. 4-9 |

Integral Cargo Tanks:

| - Warning notices and signals posted | 46 CFR 70.05-30 46 CFR 90.05-35 |
| --- | --- |
| - Pumps - Pumps and controls - Relief valves - Piping-valves - Lighting and wiring - Cofferdams | 46 CFR 70.05-30 46 CFR 90.05-35 46 CFR 111.105 |
| - Cargo spaces - Trunks and hatches - Ullage openings - Liquid level gauges - Deck penetrations - Heating coils - Internal examination | 46 CFR 91.15-1 46 CFR 91.25-37 |

Notes:

| - Cargo piping - Expansion joints - Controls - Supports - Hoses-drip pans - Valves | 46 CFR 56.30 46 CFR 56.35 46 CFR 56.20 |
| --- | --- |
| - Cargo tank venting - Independent PV valves - Independent goosenecks - Flame screen - Closure device | 46 CFR 70.05-30 46 CFR 90.05-35 |
| - Explosion-proof fixtures | 46 CFR 111.105 |
| - Independent tanks, fixed, portable, or marine portable - External examination - Date of internal examination - Date of hydrostatic test - Metal information plate - Marking and labeling - Saddles; foundation and stowage - Piping and valves - Relief valves - Lifting fittings - Securing devices - Pump and controls - Cargo hose - Electrical grounding - Firefighting requirements - Authorized cargo | 46 CFR 98.30 |

Notes:

Ground Tackle:

| - Anchors - Tested - Windlass - Capstans - Automatic tensioning device | 46 CFR 77.07 46 CFR 96.07 |
| --- | --- |
| - Mooring, standing and running gear (other than gear covered by Cargo Gear Certificate) | 46 CFR 91.37-70 |

Lifesaving Equipment:
NOTE:  Exemptions and alternatives for vessels not subject to SOLAS can be found in 
46 CFR 199.600.

| - General alarms - Controls - Batteries and fuses - Tested - Markings - Bell locations audible | 46 CFR 113.25 SOLAS 74/78 III/6.4 |
| --- | --- |
| - Type of lifeboat - Davit launched - Free fall |  |
| - Lifeboats stripped, cleaned and inspected - Date of annual servicing | 46 CFR 199.190(f) |
| - Lifeboats and work boats - Hull and fittings - Tanks and fittings - Cradles - Gripes - Compressed air cylinders - Markings | 46 CFR 199.201 46 CFR 199.202 46 CFR 199.261 46 CFR 199.262 SOLAS 74/78 III/19.2 SOLAS 74/78 III/26 MSM Ch. C2.H.2 46 CFR 199.176 |

Notes:

| - Lifeboat equipment and stowage (use table below to determine required equipment) |  |  |  |  | 46 CFR 199.175 SOLAS 74/78 III/41 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ###### Item Number | Item | International Voyage |  | Short International Voyage |  |  |  |
|  |  |  | Lifeboat | Rescue Boat | Lifeboat | Rescue Boat |  |  |
|  | ###### 1 | Bailer1 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 2 | Bilge pump2 | 1 |  | 1 |  |  |  |
|  | ###### 3 | Boathook | 2 | 1 | 2 | 1 |  |  |
|  | ###### 4 | Bucket3 | 2 | 1 | 2 | 1 |  |  |
|  | ###### 5 | Can opener | 3 |  | 3 |  |  |  |
|  | ###### 6 | Compass | 1 | 1 | 1 | 1 |  |  |
|  | ###### 7 | Dipper | 1 |  | 1 |  |  |  |
|  | ###### 8 | Drinking cup | 1 |  | 1 |  |  |  |
|  | ###### 9 | Fire extinguisher | 1 | 1 | 1 | 1 |  |  |
|  | ###### 10 | First aid kit | 1 | 1 | 1 | 1 |  |  |
|  | ###### 11 | Fishing kit | 1 |  |  |  |  |  |
|  | ###### 12 | Flashlight | 1 | 1 | 1 | 1 |  |  |
|  | ###### 13 | Hatchet | 2 |  | 2 |  |  |  |
|  | ###### 14 | Heaving line | 2 | 2 | 2 | 2 |  |  |
|  | ###### 15 | Jackknife | 1 |  | 1 |  |  |  |
|  | ###### 16 | Knife1,4 |  | 1 |  | 1 |  |  |
|  | ###### 17 | Ladder | 1 | 1 | 1 | 1 |  |  |
|  | ###### 18 | Mirror, signalling | 1 |  | 1 |  |  |  |
|  | 19 | Oars, units 5,6 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 20 | Painter | 2 | 1 | 2 | 1 |  |  |
|  | ###### 21 | Provisions (units/persons) | 1 |  |  |  |  |  |
|  | ###### 22 | Pump7 |  | 1 |  | 1 |  |  |
|  | ###### 23 | Radar reflector | 1 | 1 | 1 | 1 |  |  |
|  | ###### 24 | Rainwater collection device | 1 |  | 1 |  |  |  |
|  | ###### 25 | Repair kit7 |  | 1 |  | 1 |  |  |
|  | ###### 26 | Sea anchor | 1 | 1 | 1 | 1 |  |  |
|  | ###### 27 | Searchlight | 1 | 1 | 1 | 1 |  |  |
|  | ###### 28 | Seasickness kit (units/person) | 1 |  | 1 |  |  |  |
|  | ###### Item Number | Item | International Voyage |  | Short International Voyage |  |  |  |
|  |  |  | Lifeboat | Rescue Boat | Lifeboat | Rescue Boat |  |  |
|  | ###### 29 | Signal, smoke | 2 |  | 2 |  |  |  |
|  | ###### 30 | Signal, hand flare | 6 |  | 6 |  |  |  |
|  | ###### 31 | Signal, parachute flare | 4 |  | 4 |  |  |  |
|  | ###### 32 | Skates and fenders8 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 33 | Sponge7 |  | 2 |  | 2 |  |  |
|  | ###### 34 | Survival instructions | 1 |  | 1 |  |  |  |
|  | ###### 35 | Table of lifesaving signals | 1 |  | 1 |  |  |  |
|  | ###### 36 | Thermal protective aids9 | 10% | 10% | 10% | 10% |  |  |
|  | ###### 37 | Tool kit | 1 |  | 1 |  |  |  |
|  | ###### 38 | Tow line10 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 39 | Water (liters/person) | 3 |  | 3 |  |  |  |
|  | ###### 40 | Whistle | 1 | 1 | 1 | 1 |  |  |
|  | Footnotes: - Each rigid liferaft equipped for 13 persons or more must carry two of these items. - Not required for boats of self-bailing design. - Not required for inflated or rigid-inflated rescue boats. - A hatchet counts towards this requirement in rigid rescue boats. - Oars are not required on a free-fall lifeboat; a unit of oars means the number of oars specified by the boat manufacturer. - Rescue boats may substitute buoyant paddles for oars, as specified by the manufacturer. - Not required for a rigid rescue boat. - Required if specified by the boat manufacturer. - Sufficient thermal protective aids are required for at least 10% of the persons the survival craft is equipped to carry, but not less than two. - Required only if the lifeboat is also the rescue boat. |  |  |  |  |  |  |  |

Notes:

| - Disengaging apparatus examined or tested and marked as required* - Universal joints - Safety latches - Hooks - Locking knuckles - Frame | 46 CFR 199.45(b) SOLAS 74/78 III/41 MSM Ch. C2.H.2.b |
| --- | --- |
| - Motor lifeboats* - Engine operating test - Cooling water pump - Ahead and astern test - Fuel tanks - Searchlight test (passenger vessels) - Annual fuel changed - Extinguishers serviced | 46 CFR 199.45(b) SOLAS 74/78 III/41 MSM Ch. C2.H.9 |
| - Hand-propelled lifeboats* - Flemming gear - Transmission oil | 46 CFR 199.45(b) |
| - Radio installation for lifeboats complies with FCC and/or international convention (where required)* - Portable - Fixed | 46 CFR 199.60 SOLAS 74/78 III/6 |
| - Lifeboat operational test* | 46 CFR 199.45(b) SOLAS 74/78 III/19 MSM Ch. C2.H.2.b |
| - Davits - Foundations - Moving parts - Fittings - Fairleads, cleats, or cruciform bits | 46 CFR 160.032 SOLAS 74/78 III/48 MSM Ch. C2.H.6 |
| - Falls (date last renewed or end-for-ended) | 46 CFR 199.190(j) SOLAS 74/78 III/48 MSM Ch. C2.H.8 |
| * Items tested in conjunction with Abandon Ship Drill. |  |

Notes:

| - Lifeboat winches - Brakes - Controls - Cranks - Covers - Limit switches and electrical controls | 46 CFR 199.199(i) SOLAS 74/78 III/19 MSM Ch. C2.H.6 |
| --- | --- |
| - Embarkation aids - Ladders - Access - Spans and lifelines - Illumination - Frapping and tricing lines | 46 CFR 199.190(f) SOLAS 74/78 III/11 |
| - Lifeboat weight test - Light load - Full load: date | 46 CFR 199.45(b) SOLAS 74/78 III/19 MSM Ch. B1.P |
| - Liferafts - Launching instructions posted - Equipment and stowage - Annual service dates - Hydro release service dates - Weak link - Float free - Illumination - Markings - Capacities - Launching devices tested | 46 CFR 199.190 SOLAS 74/78 III/19 SOLAS 74/78 III/26 MSM Ch. C2.H.3 46 CFR 199.190(g) 46 CFR 199.190(h) MSM Ch. C2.H.3 MSM Ch. C2.H.3 |

Notes:

| - Lifefloats and buoyant apparatus - Equipment - Stowage - Markings | 46 CFR 199.640 |
| --- | --- |
| - Line-throwing apparatus - Equipment - Required drills held - Magazine - Type | 46 CFR 199.170 SOLAS 74/78 III/17.49 |
| - Lifebuoys - Lights - Lines - Smoke signals - Stowage - Markings | 46 CFR 199.70(a) SOLAS 74/78 III/7 SOLAS 74/78 III/21 SOLAS 74/78 III/27 |
| - Lifejackets Adult Children - Retro-reflective tape - Lights - Whistles - Work vests - Stamped passed - Number of lifejackets rejected by inspector | 46 CFR 199.70(b) SOLAS 74/78 III/7.2.2 MSM Ch. C2.H.4 |
| - Lifejacket stowage - Required notices and markings - Stowage lockers - Wearing instructions - Location instructions (passenger vessels) | 46 CFR 199.70(b) SOLAS 74/78 III/7.2 46 CFR 199.80(c) 46 CFR 199.70(b)(2) 46 CFR 199.80(c) 46 CFR 199.217 |
| - Stateroom notices posted | 46 CFR 199.80(c) |
| - Immersion suits - Condition - Retro-reflective material | 46 CFR 199.273 SOLAS 74/78 III/21.4 SOLAS 74/78 III/27.3 SOLAS 74/78 III/30.2.7 MSM Ch. C2.H.5 |

Notes:

| - Placard of lifesaving signals | 46 CFR 97.43 SOLAS 74/78 V/16 |
| --- | --- |
| - Pilot ladder and hoists in good condition | SOLAS 74/78 V/17 |
| - Station bill posted | 46 CFR 199.80 SOLAS 74/78 III/8.3 |

Fire Protection Equipment:

| - Fire control plan - Permanently posted - Copy permanently stored in weathertight container outside deckhouse | 46 CFR 78.45-1 46 CFR 97.36-1 SOLAS 74/78 II-2/20 |
| --- | --- |
| - Patrol system - Stations properly numbered and sealed - Key clock charts | 46 CFR 78.30-10 46 CFR 78.30-15 46 CFR 95.05-1 |
| - Fire detection systems - Smoke/fire alarms - Remote pull stations - Smoke/flame/heat detectors and sensors | 46 CFR 76.05-1 46 CFR 95.05-1 SOLAS 74/78 II-2/13 SOLAS 74/78 II-2/11.8 SOLAS 74/78 II-2/53 NVIC 7-80 |
| - Portable extinguishers - Serviced annually - Bottles hydrostatically tested (every 5 years) - Markings (weight and hydrostatic test date) - Spare charges, spare extinguishers | 46 CFR 76.50 46 CFR 95.50 SOLAS 74/78 II-2/6 SOLAS 74/78 II-2/21 MSM Ch. C2.I.3 NVIC 7-70 NVIC 13-86 |
| - Semiportable extinguishers - Serviced annually - Bottles hydrostatically tested - Controls, instructions, markings - Hose and diffuser - Flexible loops tested or replaced (same as bottle) | 46 CFR 76.50 46 CFR 95.50 SOLAS 74/78 II-2/6 SOLAS 74/78 II-2/21 MSM Ch. C2.I.4 |

Notes:

| - Fixed fire extinguishing systems - Controls, instructions, marking - Alarms tested - Piping - Heads, distribution - Bottles weighed annually - Bottles hydrostatically tested (every 12 years) - Flexible loops tested or replaced (10% per year) - Foam tanks (refilled or tested) - Sea suction, strainers - Fixed system tested - Storage space / door - Sprinkler head tank |  |  |  |  | 46 CFR 71.25-20(a)(2) 46 CFR 71.25-20(a)(3) 46 CFR 76.15 46 CFR 76.17 46 CFR 76.23 46 CFR 76.25 46 CFR 91.25-20(a)(2) 46 CFR 91.25-20(a)(3) 46 CFR 95.15 46 CFR 95.17 SOLAS 74/78 II-2/21 MSM Ch. C2.I.9 NVIC 6-72, Change 1 NVIC 8-73 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type of system: (circle appropriate type) |  |  |  |  |  |  |
|  | Low Pressure CO2 | High Pressure CO2 | Halon | Foam |  | Sprinkler |  |
| - Fire main system - Pumps - Piping - Cut-off valves - Drains |  |  |  |  | 46 CFR 76.10 46 CFR 95.10 NVIC 6-72, Change 1 SOLAS 74/78 II-2/4 SOLAS 74/78 II-2/21 |  |  |
| - Fire stations - Hydrants (2 effective streams) - Nozzles and spanners - Fog applicators (<6 feet in length, in engineroom) - Hose - UL approved - Correct length - Markings - Equipment compatible |  |  |  |  | 46 CFR 76.10-10 46 CFR 95.10-10 NVIC 6-72, Change 1 SOLAS 74/78 II-2/4 SOLAS 74/78 II-2/21 |  |  |
| - Total length of all hose tested - Hydrostatically tested to at least 100 psi - Proper threads - Approved hose |  |  |  |  | 46 CFR 76.10-10 46 CFR 95.10-10 MSM Ch. C2.I.6 46 CFR 91.25-20(a)(4) 46 CFR 71.25-20(a)(4) |  |  |

Notes:

| - Passenger vessels: structural fire protection - Draft stops - Fire dampers - Bulkheads - Insulation - Ventilation - Penetrations | 46 CFR 72.05 SOLAS 74/78 II-2/23 |
| --- | --- |
| - Cargo and miscellaneous vessels: structural fire protection - Bulkheads - Insulation | 46 CFR 92.07 SOLAS 74/78 II-2/42 |
| - Fire doors and controls tested - Machinery space and stair towers - Not tied or blocked open - Installed closure devices working | 46 CFR 72.05-25 46 CFR 92.07-10 SOLAS 74/78 II-2/46 SOLAS 74/78 II-2/47 |
| - Remote controls to power ventilation marked and tested | 46 CFR 111.103-1 |
| - Closures for spaces protected by fixed smothering systems | 46 CFR 76.15-35 46 CFR 95.15-35 |
| - International shore connection | 46 CFR 76.10-10(c) 46 CFR 95.10-10(c) SOLAS 74/78 II-2/19 |

Pollution Prevention:

| - Pollution placard posted | 33 CFR 155.450 |
| --- | --- |
| - MARPOL V placard posted | 33 CFR 151.59 |
| - Person-in-charge designation | 33 CFR 155.700 |

Notes:

| - Cargo oil containment - Size - Drains - Scupper closures |  | 33 CFR 155.310 |
| --- | --- | --- |
| - Fuel oil containment - Portable - Fixed |  | 33 CFR 155.320 |
| - Prohibited oil spaces |  | 33 CFR 155.470 |
| - Emergency shutdown |  | 33 CFR 155.780 |
| - Deck lighting |  | 33 CFR 155.790 |
| - Oil transfer hose - Condition - Markings - Hose assembly requirements - Tests and inspections |  | 33 CFR 155.800 |
| - Fuel tank vents - Flame screens - Closures - Pollution containment | 33 CFR 155.320 |  |
| - Garbage - Shipboard garbage properly disposed | 33 CFR 151.63 MARPOL Ax. V/3 |  |

Notes:

MTSA/ISPS Compliance

Vessel Security Plans (VSP):

(If using ASP, skip to next section)

| - Compliance documentation - Approved Vessel Security Plan | 33 CFR 104.120 ISPS, Part A, 9.1 |  |
| --- | --- | --- |
| - Waiver - Approved by CG-543 | 33 CFR 104.130 |  |
| - Equivalents - Approved by CG-543 | 33 CFR 104.135 |  |
| - Maritime Security (MARSEC) directive - Proper safeguards - Incorporated into VSP | 33 CFR 104.145 |  |
| - Master - Aware of responsibility and authority with regards to MTSA | 33 CFR 104.205 |  |
| - Company Security Officer (CSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.210 ISPS, Part A, 11 |  |
| - Vessel Security Officer (VSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.215 ISPS, Part A, 12 |  |

Notes:

| Company or vessel personnel with security duties - Training / experience - Valid TWIC - See list of example questions. - Security Training for all other vessel personnel. - Vessel Record Keeping Requirements - Training - Drills and exercises - Breaches of security - Change in MARSEC levels - Maintenance, calibration, and testing of security equipment. - Security threats - Annual audit of the VSP - Declaration of Security (DoS) - Retained for Two years | 33 CFR 104.220 ISPS, Part A, 13.3 33 CFR 104.225 ISPS, Part A, 13.3 CFR 104.235 ISPS, Part A, 10 |  |  |
| --- | --- | --- | --- |
| - MARSEC level coordination and implementation - Proper MARSEC level - MARSEC level at least at current port level | 33 CFR 104.240 |  |  |  |
| - Communications - Vessel security personnel - Facility - National and local authorities - Demonstrate communications operations consistent with the VSP | 33 CFR 104.245 ISPS, Part A, 7.2.7 |  |  |  |
| - Declaration of Security (DoS) - Required for cruise ships or manned CDC bulk vessels and any vessel or facilities with which it interfaces. - Valid (for MARSEC level and effective time period) Must have last 10 or continuous DoS reviewed at interval consistent with MARSEC level. - Signed | 33 CFR 104.255 ISPS, Part A, 5.1 |  |  |  |

| - Security systems and equipment maintenance - Testing completed IAW manufacturer’s recommendations - Working properly, effectively functions IAW VSP. - Ship Security Alert System (SSAS) | 33 CFR 104.260 |  |
| --- | --- | --- |
| - Security measures for access control - Access points examined – signs posted in conspicuous locations. - Control areas for authorized dangerous substances / devices - Means of identifying unauthorized personnel - TWIC for unescorted access to secure areas | 33 CFR 104.265 ISPS, Part A, 7.2.2 |  |
| - Security measures for newly hired employees (Pending receipt of TWIC) - Access permitted for up to 30 days if: - Has applied for TWIC - Accompanied by TWIC holder in secure areas - Operator enters new hire personal info in HOMEPORT - Notified via HOMEPORT that new hire has passed initial name check. - Provision does not apply to CSO, VSO or individual hired to perform security duties - Security measures for restricted areas - Secure areas protected - Properly marked - Control measures adequate - Do not conflict with safety measures | 33CFR 104.267 33 CFR 104.270 ISPS, Part A, 7.2.4 |  |
| - Security measures for handling cargo - Identifying cargo tamper - Identifying approved cargo - Access point – inventory control - Checking cargo for dangerous substances | 33 CFR 104.275 ISPS, Part A, 7.2.6 |  |
| - Security measures for delivery of vessel stores and bunker - Security procedures followed - Standing agreements valid | 33 CFR 104.280 ISPS, Part A, 7.2.6 |  |
| - Security measures for monitoring - IAW VSP - Lighting - Test intrusion alarms - Emergency search procedures | 33 CFR 104.285 ISPS, Part A, 7.2.5 |  |
| - Security Incident Procedures - Witness during drill - Additional requirements for passenger vessels or ferries | 33 CFR 104.290 33 CFR 104.292 |  |
| - Additional requirements for cruise ships | 33 CFR 104.295 |  |
| - Additional requirements – vessels on international voyages - ISSC issued - CSR updated | 33 CFR 104.297 ISPS, Part A, 19.1 SOLAS XI-2 Regulation 5.4.1 |  |
| - Vessel Security Assessment Report - Reviewed and attached to VSP - Vessel Security Plan - Reviewed - Amendment and audit - CSO / VSO audit letter attached to VSP as required ISSC - Audits conducted as required (annually or after vessel modifications) | 33 CFR 104.305(d) 33 CFR 104.400 33 CFR 104.415 |  |
| - Ship Security Alert System (vessels subject to SOLAS only) - On the bridge and one other location - Designed to prevent inadvertent activation - Covert (unmarked, silent, and need to know) - Tested IAW VSP | ISPS Part A, 9.4.18 |  |

Notes:

# Alternative Security Program (ASP):

Vessels operating under the auspices of an approved ASP are required to address the relevant areas cited in 33 CFR parts 104. However, the ASP provision of the rule has provided a mechanism by which segments of the maritime industry, through application by the industry associations or other representative groups, are able to tailor their program to the unique functions inherent of their specific operations. The result is a set of relevant, performance-based security measures for the industry groups choosing to utilize an approved ASP. For this reason, the inspector of a vessel using an approved ASP may find that certain language or security measures contained in some parts of the rule will differ from the language or security measures listed in the ASP. Additionally, an industry or group may determine that a section of the regulations is not applicable to their operations. For example, a passenger vessel group may state in their ASP that they do not need to address 33 CFR 104.275 or 33 CFR 105.265, respectively – security measures for handling cargo – because they do not handle cargo of any type.

In those cases where both the vessels and the facilities serving those vessels are owned and/or operated by the same entity, an alternative plan may recognize that the same party is responsible for security in both areas and approve an approach that addresses vulnerabilities and mitigation strategies for the vessels and the facility under one ASP. Therefore, the inspector will not be using separate plans for the vessels and the facility to determine compliance and, likewise, will not see some citations addressed in the plan if they are redundant between 33 CFR 104 and 33 CFR 105.

| - Name of approved ASP - ________________________________ | 33 CFR 104.140 |  |
| --- | --- | --- |
| - Compliance Documentation - Copy of ASP aboard vessel - Vessel Specific Security Assessment (VSA) completed - Letter to MSC stating use of approved ASP and that it has been fully implemented | 33 CFR 104.120 |  |
| - Master - Aware of responsibility and authority with regards to MTSA | 33 CFR 104.205 IAW ASP |  |
| - Company Security Officer (CSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.210 IAW ASP |  |
| - Vessel Security Officer (VSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.215 IAW ASP |  |
| - Company or vessel personnel with security duties - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.220 IAW ASP |  |
| - Security Training for all other vessel personnel | 33CFR 104.225 IAW ASP |  |

Notes:

| - Vessel Record Keeping Requirements - Training - Drills and exercises - Breaches of security - Change in MARSEC levels - Maintenance, calibration, and testing of security equipment. - Security threats - Annual audit of the VSP - Declaration of Security (DoS) - Retained for Two years |  | 33 CFR 104.235 IAW ASP |  |
| --- | --- | --- | --- |
| - MARSEC level coordination and implementation - Proper MARSEC level - MARSEC level at least at current port level |  | 33 CFR 104.240 IAW ASP |  |
| - Communications - Vessel security personnel - Facility - National and local authorities - Demonstrate communications operations consistent with the ASP |  | 33 CFR 104.245 IAW ASP |  |
| - Declaration of Security (DoS) - Required for cruise ships or manned CDC bulk vessels and any vessel or facilities with which it interfaces. - Valid (for MARSEC level and effective time period) Must have last 10 or continuous DoS reviewed at interval consistent with MARSEC level. - Signed |  | 33 CFR 104.255 33 CFR 104 |  |

Notes:

| - Security systems and equipment maintenance - Testing completed IAW manufacturer’s recommendations - Working properly, effectively functions IAW VSP. - Ship Security Alert System (SSAS) |  | 33 CFR 104.260 IAW ASP |  |
| --- | --- | --- | --- |
| - Security measures for access control - Access points examined – signs posted in conspicuous locations. - Control areas for authorized dangerous substances / devices - Means of identifying unauthorized personnel - TWIC for unescorted access to secure areas |  | 33 CFR 104.265 IAW ASP |  |
| - Security measures for newly hired employees (Pending receipt of TWIC) - Access permitted for up to 30 days if: - Has applied for TWIC - Accompanied by TWIC holder in secure areas - Operator enters new hire personal info in HOMEPORT - Notified via HOMEPORT that new hire has passed initial name check. - Provision does not apply to CSO, VSO or individual hired to perform security duties - Security measures for restricted areas - Secure areas protected - Properly marked - Control measures adequate - Do not conflict with safety measures |  | 33 CFR 104.267 IAW ASP 33 CFR 104.270 IAW ASP |  |
| - Security measures for handling cargo - Identifying cargo tamper - Identifying approved cargo - Access point – inventory control - Checking cargo for dangerous substances |  | IAW ASP 33 CFR 104.275 |  |
| - Security measures for delivery of vessel stores and bunker - Security procedures followed - Standing agreements valid |  | 33 CFR 104.280 IAW ASP |  |
| - Security measures for monitoring - IAW VSP - Lighting - Test intrusion alarms - Emergency search procedures |  | 33 CFR 104.285 IAW ASP |  |
| - ASP Implemented in its entirety - Amendment and audit - Users of ASP’s are required under condition of ASP approval to conduct yearly audit and advise submitting organization if amendment is needed. - CSO / VSO audit letter attached to VSP as required ISSC - Audits conducted as required (annually or after vessel modifications) |  | 33 CFR 104.140 IAW ASP 33 CFR 104.415 IAW ASP |  |

Should an enforcement inspection reveal that an owner/operator has correctly implemented an approved ASP in its entirety but security vulnerabilities exist in the vessel operation, the COTP shall be advised.  Under 33 CFR 104.415 (a) (ii) for vessels or 33 CFR 105.415 (a) (ii), the Coast Guard can determine that an amendment is necessary and advise the organization that submitted the ASP for approval accordingly. Following such notification, it will be necessary for the original submitting organization to provide their proposed amendment to the Commandant (CG-543) for review and approval. If the submitting organization does not wish to amend the ASP, the vessel owner must submit a VSP for the vessel to the MSC.

Notes:

# Section 4:  Drills

- Fire Drill:

| Initial notifications | Familiarity with duties | Space isolation |
| --- | --- | --- |
| General alarms/signals | Familiarity with equipment | Smoke control |
| Crew response | Fire pumps started | Arrange care of passengers |
| Properly dressed/equipped | Two jets of water | Communications w/ bridge |
| Language understood by crew | Fire doors and dampers |  |

(SOLAS 74/78 III/18.3; MSM Vol. II/D5.C.7.i; NVIC 6-91)

| Location: | Time on Scene: |
| --- | --- |

Notes:

- Abandon Ship Drill:

| General alarms/signals | Familiarity with duties |  | Boat release |
| --- | --- | --- | --- |
| Muster lists | Provide equipment |  | Boat operation |
| Muster of crew/passengers | Familiarity with equipment |  | Egress procedures |
| Crew response | Lower lifeboat |  | Davit-launched liferaft drill |
| Language understood by crew | Brake operation |  | Communication w/ bridge |
| Lifejackets | Engine start |  | Lighting |
| (SOLAS 74/78 III/18.3; MSM Vol. II/C5.C.7.h) |  |  |  |
| Location: |  | Time to Water: |  |

Notes:

- Security Drill:

- Observe security drill exercising the activation of the provisions in the VSP or ASP related to a security threat, breach, security communications, change of security level, or other security related incident or action as describe in the VSP or ASP. - Drill selection and location shall be as directed by the Master and VSO. - Critique Drill with VSO/CSO

Location:

Notes:

Section 5:  Appendices

Recommended U.S. Vessel Deficiency Procedures:

| ##### Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Identify deficiency. |  |  |  |
| 2 | Inform vessel representative. |  |  |  |
| 3 | Record on the Deficiency Summary Worksheet (next page). |  |  |  |
| 4 | If deficiency is corrected prior to end of inspection, go to Step 7. |  |  |  |
| 5 | If deficiency is unable to be corrected prior to end of inspection, issue CG-835 in accordance with table below. |  |  |  |
|  |  | IF deficiency: | THEN issue CG-835: |  |
|  |  | Does NOT immediately impact crew/passenger safety, security, hull seaworthiness, or the environment, e.g., - Missing placards - Non-metallic expansion joints more than 10 years in service | That provides a specific time for correcting deficiency, e.g., - “X” number of days - At next drydock |  |
|  |  | Allows vessel operations to be MODIFIED to meet less stringent requirements, e.g., - Expired international certificates - Automation defect - Insufficient lifesaving equipment | That restricts operation of vessel to meet current vessel conditions, e.g., - Reduced route - Increased crew - Fewer passengers |  |
|  |  | DOES immediately impact crew/passenger safety, security, hull seaworthiness, or the environment, and cannot be modified to meet less stringent requirements, e.g., - Missing or defective firefighting equipment - Structural defect or damage - Substantially not in compliance with approved VSP or ASP. | That requires the deficiency to be corrected prior to operating vessel (“NO SAIL” item), e.g., - Prior to carrying passengers - Prior to carrying cargo |  |
| 6 | Enter CG-835 data in MISLE. |  |  |  |

Deficiency Summary Worksheet:

|  | Name of Vessel |  |  | VIN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ## Deficiency |  |  |  |  | ## Req’t. Issued / Date Completed |

Notes:

Notes:

Conversions:

| Distance and Energy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kilowatts (kW) |  |  |  |  |  |  |  | X |  |  |  | 1.341 |  |  |  |  |  | = |  |  |  | Horsepower (hp) |  |  |  |  |  |  |  |  |
| Feet (ft) |  |  |  |  |  |  |  | X |  |  |  | 3.281 |  |  |  |  |  | = |  |  |  | Meters (m) |  |  |  |  |  |  |  |  |
| Long Ton (LT) |  |  |  |  |  |  |  | X |  |  |  | .98421 |  |  |  |  |  | = |  |  |  | Metric Ton (t) |  |  |  |  |  |  |  |  |
| Liquid (NOTE: Values are approximate.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Liquid |  |  |  |  |  |  |  | bbl/LT |  |  |  |  |  | m3/t |  |  |  |  |  | ###### bbl/m3 |  |  |  |  |  |  | bbl/t |  |  |  |
| Freshwater |  |  |  |  |  |  |  | 6.40 |  |  |  |  |  | 1.00 |  |  |  |  |  | 6.29 |  |  |  |  |  |  | 6.29 |  |  |  |
| Saltwater |  |  |  |  |  |  |  | 6.24 |  |  |  |  |  | .975 |  |  |  |  |  | 6.13 |  |  |  |  |  |  | 5.98 |  |  |  |
| Heavy Oil |  |  |  |  |  |  |  | 6.77 |  |  |  |  |  | 1.06 |  |  |  |  |  | 6.66 |  |  |  |  |  |  | 7.06 |  |  |  |
| DFM |  |  |  |  |  |  |  | 6.60 |  |  |  |  |  | 1.19 |  |  |  |  |  | 7.48 |  |  |  |  |  |  | 8.91 |  |  |  |
| Lube Oil |  |  |  |  |  |  |  | 7.66 |  |  |  |  |  | 1.20 |  |  |  |  |  | 7.54 |  |  |  |  |  |  | 9.05 |  |  |  |
| Weight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Long Ton |  |  |  |  | = | 2240 lbs |  |  |  |  |  |  |  |  |  | 1 Metric Ton |  |  |  |  |  |  | = |  | 2204 lbs |  |  |  |  |  |
| 1 Short Ton |  |  |  |  | = | 2000 lbs |  |  |  |  |  |  |  |  |  | 1 Cubic Foot |  |  |  |  |  |  | = |  | 7.48 gal |  |  |  |  |  |
| 1 Barrel (oil) |  |  |  |  | = | 5.61 ft = 42 gal = 6.29 m3 |  |  |  |  |  |  |  |  |  | 1 psi |  |  |  |  |  |  | = |  | .06895 Bar = 2.3106 ft of water |  |  |  |  |  |
| Temperature: Fahrenheit = Celsius (F = 9/5 C + 32 and C = 5/9 (F – 32)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | = |  | -17.8 |  |  |  |  |  |  |  | 80 |  | = |  |  |  | 26.7 |  |  |  |  |  |  |  | 200 | = |  |  |  | 93.3 |
| 32 | = |  | 0 |  |  |  |  |  |  |  | 90 |  | = |  |  |  | 32.2 |  |  |  |  |  |  |  | 250 | = |  |  |  | 121.1 |
| 40 | = |  | 4.4 |  |  |  |  |  |  |  | 100 |  | = |  |  |  | 37.8 |  |  |  |  |  |  |  | 300 | = |  |  |  | 148.9 |
| 50 | = |  | 10.0 |  |  |  |  |  |  |  | 110 |  | = |  |  |  | 43.3 |  |  |  |  |  |  |  | 400 | = |  |  |  | 204.4 |
| 60 | = |  | 15.6 |  |  |  |  |  |  |  | 120 |  | = |  |  |  | 48.9 |  |  |  |  |  |  |  | 500 | = |  |  |  | 260 |
| 70 | = |  | 21.1 |  |  |  |  |  |  |  | 150 |  | = |  |  |  | 65.6 |  |  |  |  |  |  |  | 1000 | = |  |  |  | 537.8 |
| Pressure: Bars = Pounds per square inch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Bar |  | = |  | 14.5 psi |  |  |  |  |  | 5 Bars |  |  |  |  | = |  | 72.5 psi |  |  |  |  |  |  | 9 Bars |  |  |  | = | 130.5 psi |  |
| 2 bars |  | = |  | 29.0 psi |  |  |  |  |  | 6 Bars |  |  |  |  | = |  | 87.0 psi |  |  |  |  |  |  | 10 Bars |  |  |  | = | 145.0 psi |  |
| 3 Bars |  | = |  | 43.5 psi |  |  |  |  |  | 7 Bars |  |  |  |  | = |  | 101.5 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Bars |  | = |  | 58.0 psi |  |  |  |  |  | 8 Bars |  |  |  |  | = |  | 116.0 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |