United States Coast Guard

MACHINERY
INSPECTION BOOK

| ## Name of Vessel |  |  |
| --- | --- | --- |
| ## Official Number |  | ## Class |
| ## Date Completed |  |  |
| ## Location |  |  |
| Vessel Built in Compliance with SOLAS: 60 74 74/78 NA |  |  |
| ## Inspection Type |  |  |
| - Inspection for Certification (COI) - Annual - Periodic - Other __________ | - Reinspection |  |
|  | - First - Second Passenger vessels only - Third |  |
| ## Inspectors 1. 2. |  | 3. 4. |

CG-840 MI
Rev. July 2009

Total Time Spent Per Activity:

| Regular Personnel (Active Duty) |  |  |  |
| --- | --- | --- | --- |
| ACTIVITY TYPE | ACTIVITY | TRAINING | (PERS) MI |

| TOTAL ADMIN HOURS | TOTAL TRAVEL HOURS |
| --- | --- |

| Reserve Personnel |  |  |  |
| --- | --- | --- | --- |
| ACTIVITY TYPE | ACTIVITY | TRAINING | (PERS) MI |

| TOTAL ADMIN HOURS | TOTAL TRAVEL HOURS |
| --- | --- |

| ##### Auxiliary Resources |  |
| --- | --- |
| TOTAL BOAT HOURS | TOTAL AIRCRAFT HOURS |

Use of Machinery Inspection Book:

This inspection book is intended to be used as a job aid by Coast Guard marine inspectors during machinery inspections of U.S. flagged vessels.  The lists contained within this book are not intended to limit the inspection.  Each marine inspector should determine the depth of inspection necessary.  A checked box should be a running record of what has been inspected.  It does not imply that the entire system has been inspected or that all or any items are in full compliance.  This job aid does not constitute part of the official inspection record.

This document does not establish or change Federal laws or regulations.  References given are only general guides.  Refer to IMO publications, CFRs, NVICs, or any locally produced cite guides for specific regulatory references.  Not all items in this book are applicable to all vessels or types of propulsion systems.

NOTE:  Guidance on how to conduct machinery inspections of U.S. flagged vessels can be found in Marine Safety Manual (MSM) Volume II:  Inspection of Vessels for Certification.  All MSM cites listed in this book refer to MSM Volume II unless otherwise indicated.

| # Pre-inspection Items: | Post-inspection Items: |
| --- | --- |
| - Review MISLE records. | - Issue/endorse certificates as appropriate. |
| - Obtain copies of Certificates to be issued. | - Complete MISLE entries. - Activity Type - Team Members - Vessel Details - Fleet of Resp - Deficiencies - Certificates |
|  | - Initiate Notice of Violation (NOV) if necessary. |

Table of Contents:

Section 1:  Inspection Items

Boilers	1

Diesels	6

Pressure Vessels	7

Auxiliary Machinery	7

Electrical Systems	8

Firefighting Equipment	11

Watertight Integrity	12

Pollution Prevention	13

Marine Sanitation Devices	13

Miscellaneous	14

## Section 2:  Appendices

Recommended US Vessel Deficiency Procedures	15

Deficiency Summary Worksheet	16

Notes	18

Conversions	19

Section 1:  Inspection Items

Boilers:

| - Propulsion machinery - Safety devices - Foundations - Guards - Controls |  |  |  |  | 46 CFR 58.05 SOLAS 74/78 II-1/27 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Propulsion and auxiliary boilers - Shells or drums - Headers - Superheater - Blow off piping and valves - Tubes or flues - Furnaces - Soot blowers - Economizers - Combustion chambers - Refractory - Casing and insulation - Uptakes - Air preheaters - Forced draft blowers - Foundations - Gauges - Water level indicators |  |  |  |  | 46 CFR 52.01-2 46 CFR 52.01-35 SOLAS 74/78 II-1/32 MSM Vol II 46 CFR 52.15-5 46 CFR 56.50-40 46 CFR 52.01-110 |  |  |  |
| - Periodic test and inspection of boilers in accordance with 46 CFR Table 61.05-10 |  |  |  |  |  |  |  |  |
|  | Boiler ID Number | Date Hydrostatically Tested | Date Mountings Opened | Date Mountings Removed and Studs Examined |  | Fireside | Waterside | External |

Notes:

Testing of Boiler Safety Valve 
46 CFR 52.01-120

| Step | Action | # D/S | S/S | ###### S/P |
| --- | --- | --- | --- | --- |
| 1 | Determine MAWP of boiler. psi |  |  |  |
| 2 | Record pressure setting stamped on each valve. | psi | psi | psi |
| 3 | Observe opening and closing of valves and record lift and seating pressures of each valve. 3a. Lift pressure 3b. Seating pressure | psi psi | psi psi | psi psi |
|  | WARNING: NEVER allow test pressure to be greater than MAWP during test. If lift pressure is above MAWP, the valve must be adjusted or replaced before test continues. NOTE: Safety valves must be tested in highest-to-lowest pressure order; typically D/S-S/S-S/P. This avoids the risk of damaging a valve or changing its setting by placing a gag on it after it has been tested. |  |  |  |
| 4 | Ensure Step 3 pressures are within acceptable limits (± 5%) of stamped pressure. Use the following calculations. 4a. Step 2 (stamped pressure) x .05 4b. Step 2 (stamped pressure) – 4a (-5%) 4c. Step 2 (stamped pressure) + 4a (+5%) | psi psi psi | psi psi psi | psi psi psi |
|  | IMPORTANT: Step 3 (lift pressure) must be between pressures recorded in 4b and 4c. If NOT, safety valve lift pressure MUST be adjusted within specified limits. |  |  |  |
| 5 | Record superheater pressure drop value from boiler manual. |  | psi | psi |
| 6 | Ensure S/S and S/P lift pressures (from Step 3) are  pressures recorded in 6b. 6a. Step 5 (superheater pressure drop) + 5 psi 6b. Step 3a (D/S pressure) – 6a pressure |  | psi psi | psi psi |
|  | IMPORTANT: If Step 3a (S/S and S/P) is NOT  6b, S/S and S/P lift pressures MUST be adjusted. |  |  |  |
| 7 | Determine blowdown and ensure it is between 2% and 4% of lift pressure for each valve. Use the following calculations. 7a. 3a pressure – 3b pressure = blowdown 7b. 3a pressure x .02 (2%) 7c. 3a pressure x .04 (4%) | psi psi psi | psi psi psi | psi psi psi |
|  | IMPORTANT: If 7a (blowdown) is not between 7b and 7c, blowdown setting MUST be adjusted within specified limits. |  |  |  |
| 8 | After hand-relieving gear is reinstalled, observe each valve as it is hand-relieved from the fireroom or engineroom floor (46 CFR 52.01-120(d)(2)). |  |  |  |

D/S = Drum Safety Valve	S/S = Superheater Safety Valve	S/P = Superheater Pilot Valve

| - Safety valves - Relieving gear - Escape pipes - Drains |  |  |  | 46 CFR 52.01-120 MSM Vol. IV 46 CFR 56.50-25 |
| --- | --- | --- | --- | --- |
|  | Boiler | Date Set and Sealed | Pressure Setting |  |
| - Superheater safety valves |  |  |  | 46 CFR 52.01-120 |
|  | Boiler | Date Set and Sealed | Pressure Setting |  |
| - Automation - Reduced manning - Yes - No - Approved test procedure - Satisfactory test - Reviewed logs/records - Interviewed personnel |  |  |  | 46 CFR Part 62 SOLAS 74/78 II-1/46-54 MSM Vol. IV NVIC 1-69 NVIC 7-73 NVIC 6-84 46 CFR 62.50 46 CFR 62.30-10 |

Notes:

| - Fusible plugs - Examined - Renewed at this inspection | 46 CFR 52.01-50 46 CFR Table 61.05-10 MSM Vol. IV |
| --- | --- |
| - High pressure steam piping - Steam piping > 3 inches subject to boiler pressure hydrostatically tested (46 CFR 61.05-10) - Lagging or insulation - Hangers or supports | 46 CFR 52.01-105 46 CFR 56.50-15 SOLAS 74/78 II-1/33 |
| - Fuel systems - Service and transfer pumps - Remote shutoff valves - Remote cutouts - Reliefs and bypass valves - Strainers - Drip pans - Torch pots - Piping - Heaters | 46 CFR 56.50-65 |
| - Feedwater system (including condensate) - Pumps - Injectors - Valves and controls - Water heaters (including deaerator) - Water regulators - Water level indicators - Grease extractors - Piping - Gauges and thermometers - Air ejectors - Condensers | 46 CFR 52.01-115 46 CFR 56.50-35 46 CFR 56.50-45 46 CFR 56.50-30 |

Notes:

| - Automatic auxiliary boilers - Controls and safety devices - Fuel systems - Alarms - Inspections/test |  |  | 46 CFR 63.15-1 46 CFR 63.20 46 CFR 63.15-3 46 CFR 63.15-7 46 CFR 63.15-9 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Boiler repairs in accordance with 46 CFR Part 59 |  |  |  |  |  |  |
| - Low pressure heating boilers - Safety or relief valves - Gauges - Thermometers - Automatic controls - Bottom blow off - Water level indicator - Connections - Refractory |  |  | 46 CFR 53.01 46 CFR 53.05 46 CFR 53.12 |  |  |  |
| - Periodic test and inspection of low pressure heating boilers in accordance with 46 CFR Table 61.05-10 |  |  |  |  |  |  |
|  | ###### Boiler Number | ###### Date Hydrostatically Tested |  | Fireside | Waterside | External |

Notes:

Diesels:

| - Propulsion machinery - Safety devices - Foundations - Guards - Controls | 46 CFR 58.05 SOLAS 74/78 II-1/27 |
| --- | --- |
| - Main propulsion diesels - Fuel lines - Air starting lines - Exhaust system - Manifold - Exhaust pipe - Protective devices - Lube oil system - Coolers - Standby L/O pump - Engine protection - Remote shutdowns - Overspeed protection - Low lube oil - High temperature - Crank case - Explosion covers | 46 CFR 58.05 46 CFR 58.10 SOLAS 74/78 II-1/27 |
| - Gas Turbine Installations - Design, construction, and materials - Exhaust system - Cooling and ventilation - Automatic shutdowns - Fuel systems - Fire extinguishing systems | 46 CFR 58.10-15 |
| - Automation - Reduced manning - Yes - No - Approved test procedure - Satisfactory test - Reviewed logs/records - Interviewed personnel - Verify programmable systems/devices | 46 CFR Part 62 SOLAS 74/78 II-1/46-54 MSM Vol. IV NVIC 1-69 NVIC 6-84 46 CFR 62.50 46 CFR 62.30-10 46 CFR 62.25-25 |

Pressure Vessels:

| - Pressure vessels hydrostatically tested or internally examined |  |  |  | 46 CFR 61.10 MSM Vol. II MSM Vol. IV |  |
| --- | --- | --- | --- | --- | --- |
|  | ###### Service | MAWP | Date Tested or Examined Internally |  | Relief Valve Tested |
| - Relief valves springs set within range |  |  |  | 46 CFR 54.15-10(g) |  |

Auxiliary Machinery:

| - Bilge and ballast systems - Pumps - Eductors - Emergency bilge pump - Manifold, valves, and piping - Remote controls (hydraulic, pneumatic, manual, electric) - Strainers - Sounding and vent piping - Markings and indicators | 46 CFR 56.50-50 46 CFR 56.50-55 46 CFR 56.50-57 |
| --- | --- |
| - Compressed air system - Compressor - Controls and gauges - Relief valves | 46 CFR 58.30 |

Notes:

| - Lubrication systems - Pumps - Heat exchangers - Valves and controls - Piping - Gauges, thermometers, and alarms - Tanks, vents, and strainers | 46 CFR 56.50-80 |
| --- | --- |
| - Refrigeration and air conditioning systems - Compressors - Valves and controls - Spare refrigerant stowage - Gas mask (ammonia) with spare charges - Ventilation - Alarms | 46 CFR 58.20 |
| - Evaporators - Pumps - Valves and controls | 46 CFR 54.01-10 |
| - Freshwater systems (potable and domestic) - Pumps - Valves and controls - Sump tanks - Tank pressure - Air cushion supply line |  |
| - Steering gear systems tested - Motors and pumps - Telemotor or other control - Indicators and alarms - Instructions and markings - Final emergency power source | 46 CFR 58.25 46 CFR 61.20 46 CFR 58.25-70 46 CFR 58.25-25 33 CFR 164.34 46 CFR 58.25-65 |

Electrical Systems:

NOTE:  Guidance for inspecting electrical systems is detailed in NVIC 2-89.

| - Ship’s service generators - Protective guards - Reverse power relay - Overspeed trip (> 110% < 115%) - Low oil pressure alarm / shutdown | 46 CFR 110.10 46 CFR 111.12 SOLAS 74/78 II-1/41 MSM Vol. II MSM Vol. IV 46 CFR 111.12-1 |
| --- | --- |

Notes:

| - Switchboards (including emergency) - Automatic bus transfer - Ground detectors - Personnel safeguards (guards, rails, mats, etc.) - Drip shields - Nameplates - Warning notices posted - Fuse/circuit breaker ratings | 46 CFR 111.30 MSM Vol. II MSM Vol. II |
| --- | --- |
| - Panel boards - Overcurrent devices - Circuit directory - Locking device | 46 CFR 111.40 46 CFR 111.40-11 |
| - Motor controllers - Drip shields - Disconnect switch - Wiring diagram posted - Remote shutdowns tested | 46 CFR 111.70 MSM Vol. II |
| - Ventilation systems - Remote shutdown tested - Cargo fans - Machinery space fans - Accommodation fans | 46 CFR 111.103 SOLAS 74/78 II-1/48 |
| - Ship’s service lighting systems - Panelboards - Circuit directory - Fuses - Circuit breakers - Berth lights - Globes and guards - Explosion-proof or watertight (where required) | 46 CFR 111.75 46 CFR 111.40 |

Notes:

| - Emergency generator tested - Starting system - Fuel system - Overspeed trip (> 110% < 115%) - Low oil pressure alarm / shutdown - High jacket water temperature alarm - Fixed firefighting system shutdown | 46 CFR 112.25 46 CFR 112.50 SOLAS 74/78 II-1/42-44 |
| --- | --- |
| - Emergency batteries tested - Protection - Charger - Ventilation | 46 CFR 112.55 |
| - Adequate emergency power and lighting | 46 CFR 112.43 MSM Vol. II |
| - Internal communications and control system - General alarms - Engine order telegraph - Failure alarms - Telephones - Voice tubes - Public address system - Pilothouse controls - Fire detection and alarm systems - Steering gear alarm and indicator | 46 CFR 113.25 46 CFR 113.35 46 CFR 113.30 46 CFR 113.50 46 CFR 113.10 46 CFR 113.43 |
| - Lifeboat electrical installation - Winches and controls tested - Master switch opened - Limit switches opened - Emergency disconnect switch opened | 46 CFR 111.95 MSM Vol. II |
| - Emergency Loads - Temporary loads - Final loads | 46 CFR 112.15 SOLAS II-1 |

Notes:

| - General electrical installation - Jury rigs - Connection boxes - Dead-end cables - Splices - Grounding - Personnel safeguards (guards, rails, etc.) - Hazardous locations - Portable electrical equipment | 46 CFR 111.01-1 SOLAS 74/78 II-1/40 46 CFR 111.60 46 CFR 111.60-17 46 CFR 111.60-19 46 CFR 111.05 46 CFR 111.30-11 46 CFR 111.105 MSM Vol. IV MSM Vol II |
| --- | --- |

Firefighting Equipment:

| - Portable extinguishers (machinery spaces) - Required number, type, and class - Annually serviced - Bottles hydrostatically tested (every 5 years) - Markings (weight and hydrostatic test date) - Spare charges, spare extinguishers | 46 CFR 34.50 46 CFR 76.50 46 CFR 95.50 SOLAS 74/78 II-2/6 SOLAS 74/78 II-2/21 MSM Vol. II NVIC 7-70 NVIC 13-86 |
| --- | --- |
| - Semiportable extinguishers (machinery spaces) - Required number, type, and class - Annually serviced - Bottles hydrostatically tested (every 12 years) - Controls, instructions, markings - Hose and diffuser - Flexible loops tested or replaced (same as bottle) | 46 CFR 34.50 46 CFR 76.50 46 CFR 95.50 SOLAS 74/78 II-2/6 SOLAS 74/78 II-2/21 MSM Vol. II |
| - Sprinkler system tested - Type - Pumps - Manifold - Controls - System diagram posted | 46 CFR 34.30 46 CFR 76.25 46 CFR 95.30 MSM Vol II NFPA 13-1996 |

Notes:

| - Fixed fire extinguishing system (machinery spaces) (System servicing is recorded in Hull Inspection 840 Book.) - Piping/flexible loops - Heads - Alarms - Markings | 46 CFR 34.15 46 CFR 34.17 46 CFR 76.15 46 CFR 76.17 46 CFR 95.05-10 SOLAS 74/78 II-2/11 |
| --- | --- |
| - Fire main systems and stations (machinery spaces) - Required number and type, proper threads - Nozzles (combination, etc.) - Applicators - Spanners - Markings | 46 CFR 34.10-10 46 CFR 76.10-10 46 CFR 95.10-10 |
| - Pumps tested - Controls and gauges - Relief valves - Markings | 46 CFR 34.10-5 46 CFR 76.10-5 46 CFR 95.10-5 SOLAS 74/78 II-2/4 |
| - Paint locker | 46 CFR 34.05-5 46 CFR Table 76.05-1(a) 46 CFR 95.05-10(c) SOLAS 74/78 II-2/18.7 |

Watertight Integrity:

| - Watertight integrity of machinery spaces - Watertight doors - Alarms - Controls - Bulkheads (penetrations) - Markings | 46 CFR 170.270 MSM Vol. II ASTM F-1197 |
| --- | --- |

Notes:

Pollution Prevention:

NOTE:  Guidance for inspecting pollution prevention items is detailed in MSM Volume II, Chapter 31.

| - Oil record book maintained and submitted | 33 CFR 151.25 MARPOL Ax. I/20 |
| --- | --- |
| - Oily water separating equipment - Approved equipment - Operationally tested - Alarms - Shutdowns | 33 CFR 155.380 MARPOL Ax. I/6 MSM Vol. IV MSM Vol. II |
| - Ballast discharge - Piping system - Outlet - Stop valve - Acceptable processing equipment | 33 CFR 155.330 33 CFR 155.350 33 CFR 155.360 33 CFR 155.370 MSM Vol. II |
| - Pollution placard posted | 33 CFR 155.450 MSM Vol. II |
| - Oily waste retention - Bilge - Tank - Emission Controls - NOx Requirements - EPA engine emission stds for vsls on intl' voyages; - EIAPP Cert. issued by the EPA for vsls on intl' voyages - IAPP Cert. - Fuel and SOx Requirements - Incinerator - Ozone Depleting Substance | MSM Vol. II MARPOL VI CG-543 Policy Ltr 09-01 40 CFR 94 or 1042 46 CFR 63.25-9 |

Marine Sanitation Devices:

NOTE:  Guidance for inspecting marine sanitation devices is detailed in MSM Volume II, Chapter 18.K.

| - Marine sanitation device - Type I - Type II - Type III | 33 CFR 159.55 MSM Vol. II |
| --- | --- |
| - Certified for inspected vessels | MSM Vol. II |
| - Capacity satisfactory | MSM Vol. II |

| - Installation - Operation - Ventilation - Wiring and piping - Maintenance - Placard posted - Safety - Accessibility to parts requiring routine servicing - Manufacturer’s instructions available | 33 CFR 159.57 MSM Vol. IV 33 CFR 159.59 |
| --- | --- |

# Miscellaneous:

| - Liquefied petroleum gases for cooking and heating - Approved type - Cylinder - Test dates - Stowage - Safety relief device - Regulators - Piping and fittings - Location | 46 CFR 61.15-10 |
| --- | --- |
| - Tank tops, bilges, cofferdams, and bilge wells |  |
| - Sea suctions and overboard discharges | MSM Vol. II |
| - Nonmetallic expansion joints - External exam - 10-year service replacement | 46 CFR 61.15-12 MSM Vol. II |
| - Means of escape - Accessibility - Absence of locks - ISM - Machinery space maintenance - Non-conformity record keeping - Documents and reports | 46 CFR 32.01-1 46 CFR 72.10-5 46 CFR 92.10-5 33 CFR Part 96 |

Notes:

# Section 2:  Appendices

Recommended US Vessel Deficiency Procedures:

| ##### Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Identify deficiency. |  |  |  |
| 2 | Inform vessel representative. |  |  |  |
| 3 | Record on the Deficiency Summary Worksheet (next page). |  |  |  |
| 4 | If deficiency is corrected prior to end of inspection, go to Step 7. |  |  |  |
| 5 | If deficiency is unable to be corrected prior to end of inspection, issue CG-835 in accordance with table below. |  |  |  |
|  |  | IF deficiency: | THEN issue CG-835: |  |
|  |  | Does NOT immediately impact crew/passenger safety, hull seaworthiness, or the environment, e.g., - Missing placards | That provides a specific time for correcting deficiency, e.g., - "X" number of days |  |
|  |  | Allows vessel operations to be MODIFIED to meet less stringent requirements, e.g., - Automation defect | That restricts operation of vessel to meet current vessel conditions, e.g., - Increased crew |  |
|  |  | DOES immediately impact crew/passenger safety, hull seaworthiness, or the environment, and cannot be modified to meet less stringent requirements, e.g., - Missing or defective firefighting equipment | That requires the deficiency to be corrected prior to operating vessel ("NO SAIL" item), e.g., - Prior to carrying passengers - Prior to carrying cargo |  |
| 6 | Enter CG-835 data in MISLE. |  |  |  |
| 7 | Initiate Report of Violation (ROV) if necessary. |  |  |  |

Deficiency Summary Worksheet:

|  | Name of Vessel |  |  | VIN |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ## Deficiency |  |  |  |  | ## Req’t. Issued/ Date Completed |  |

| ## Deficiency |  |  | ## Req’t. Issued/ Date Completed |
| --- | --- | --- | --- |

Notes:

Notes:

Conversions:

| Distance and Energy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kilowatts (kW) |  |  |  |  |  |  |  | X |  |  |  | 1.341 |  |  |  |  |  | = |  |  |  | Horsepower (hp) |  |  |  |  |  |  |  |  |
| Feet (ft) |  |  |  |  |  |  |  | X |  |  |  | 3.281 |  |  |  |  |  | = |  |  |  | Meters (m) |  |  |  |  |  |  |  |  |
| Long Ton (LT) |  |  |  |  |  |  |  | X |  |  |  | .98421 |  |  |  |  |  | = |  |  |  | Metric Ton (t) |  |  |  |  |  |  |  |  |
| Liquid (NOTE: Values are approximate.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Liquid |  |  |  |  |  |  |  | bbl/LT |  |  |  |  |  | m3/t |  |  |  |  |  | ###### bbl/m3 |  |  |  |  |  |  | bbl/t |  |  |  |
| Freshwater |  |  |  |  |  |  |  | 6.40 |  |  |  |  |  | 1.00 |  |  |  |  |  | 6.29 |  |  |  |  |  |  | 6.29 |  |  |  |
| Saltwater |  |  |  |  |  |  |  | 6.24 |  |  |  |  |  | .975 |  |  |  |  |  | 6.13 |  |  |  |  |  |  | 5.98 |  |  |  |
| Heavy Oil |  |  |  |  |  |  |  | 6.77 |  |  |  |  |  | 1.06 |  |  |  |  |  | 6.66 |  |  |  |  |  |  | 7.06 |  |  |  |
| DFM |  |  |  |  |  |  |  | 6.60 |  |  |  |  |  | 1.19 |  |  |  |  |  | 7.48 |  |  |  |  |  |  | 8.91 |  |  |  |
| Lube Oil |  |  |  |  |  |  |  | 7.66 |  |  |  |  |  | 1.20 |  |  |  |  |  | 7.54 |  |  |  |  |  |  | 9.05 |  |  |  |
| Weight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Long Ton |  |  |  |  | = | 2240 lbs |  |  |  |  |  |  |  |  |  | 1 Metric Ton |  |  |  |  |  |  | = |  | 2204 lbs |  |  |  |  |  |
| 1 Short Ton |  |  |  |  | = | 2000 lbs |  |  |  |  |  |  |  |  |  | 1 Cubic Foot |  |  |  |  |  |  | = |  | 7.48 gal |  |  |  |  |  |
| 1 Barrel (oil) |  |  |  |  | = | 5.61 ft = 42 gal = 6.29 m3 |  |  |  |  |  |  |  |  |  | 1 psi |  |  |  |  |  |  | = |  | .06895 Bar = 2.3106 ft of water |  |  |  |  |  |
| Temperature: Fahrenheit = Celsius (F = 9/5 C + 32 and C = 5/9 (F – 32)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | = |  | -17.8 |  |  |  |  |  |  |  | 80 |  | = |  |  |  | 26.7 |  |  |  |  |  |  |  | 200 | = |  |  |  | 93.3 |
| 32 | = |  | 0 |  |  |  |  |  |  |  | 90 |  | = |  |  |  | 32.2 |  |  |  |  |  |  |  | 250 | = |  |  |  | 121.1 |
| 40 | = |  | 4.4 |  |  |  |  |  |  |  | 100 |  | = |  |  |  | 37.8 |  |  |  |  |  |  |  | 300 | = |  |  |  | 148.9 |
| 50 | = |  | 10.0 |  |  |  |  |  |  |  | 110 |  | = |  |  |  | 43.3 |  |  |  |  |  |  |  | 400 | = |  |  |  | 204.4 |
| 60 | = |  | 15.6 |  |  |  |  |  |  |  | 120 |  | = |  |  |  | 48.9 |  |  |  |  |  |  |  | 500 | = |  |  |  | 260 |
| 70 | = |  | 21.1 |  |  |  |  |  |  |  | 150 |  | = |  |  |  | 65.6 |  |  |  |  |  |  |  | 1000 | = |  |  |  | 537.8 |
| Pressure: Bars = Pounds per square inch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Bar |  | = |  | 14.5 psi |  |  |  |  |  | 5 Bars |  |  |  |  | = |  | 72.5 psi |  |  |  |  |  |  | 9 Bars |  |  |  | = | 130.5 psi |  |
| 2 bars |  | = |  | 29.0 psi |  |  |  |  |  | 6 Bars |  |  |  |  | = |  | 87.0 psi |  |  |  |  |  |  | 10 Bars |  |  |  | = | 145.0 psi |  |
| 3 Bars |  | = |  | 43.5 psi |  |  |  |  |  | 7 Bars |  |  |  |  | = |  | 101.5 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Bars |  | = |  | 58.0 psi |  |  |  |  |  | 8 Bars |  |  |  |  | = |  | 116.0 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |