United States Coast Guard

TANKSHIP
INSPECTION BOOK

| ## Name of Vessel |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## Official Number |  |  | ## Class |  |
| ## Date Completed |  |  | ## Location |  |
| Vessel Built in Compliance with SOLAS: 60 74 74/78 N/A |  |  |  |  |
| ## Route |  |  |  |  |
| - Oceans | - Limited Coastwise |  |  | - Lakes/Bays/Sounds |
| - Coastwise | - Great Lakes |  |  | - Rivers |
| ## Inspection Type |  |  |  |  |
| - Inspection for Certification (COI) |  |  |  | - Annual Inspection - Periodic Inspection |
| ## Inspectors 1. 2. |  | 3. 4. |  |  |

CG-840 TS
Rev. Jan 2010

Total Time Spent Per Activity:

| Regular Personnel (Active Duty) |  |  |  |
| --- | --- | --- | --- |
| ACTIVITY TYPE | ACTIVITY | TRAINING | (PERS) MI |

| TOTAL ADMIN HOURS | TOTAL TRAVEL HOURS |
| --- | --- |

| Reserve Personnel |  |  |  |
| --- | --- | --- | --- |
| ACTIVITY TYPE | ACTIVITY | TRAINING | (PERS) MI |

| TOTAL ADMIN HOURS | TOTAL TRAVEL HOURS |
| --- | --- |

| ##### Auxiliary Resources |  |
| --- | --- |
| TOTAL BOAT HOURS | TOTAL AIRCRAFT HOURS |

Use of Tankship Inspection Book:

This inspection book is intended to be used as a job aid by Coast Guard marine inspectors during inspections of U.S. flagged tankships.  The lists contained within this book are not intended to limit the inspection.  Each marine inspector should determine the depth of inspection necessary.  A checked box should be a running record of what has been inspected.  It does not imply that the entire system has been inspected or that all or any items are in full compliance.  This job aid does not constitute part of the official inspection record.

This document does not establish or change Federal laws or regulations.  References given are only general guides.  Refer to IMO publications, CFRs, NVICs or any locally produced cite guides for specific regulatory references.  Not all items in this book are applicable to all vessels.

NOTE:  Guidance on how to conduct inspections of U.S. flagged tankships can be found in the Marine Safety Manual (MSM) Volume II, Chapter B1:  Inspection of Vessels for Certification.  All MSM cites listed in this book refer to MSM Volume II unless otherwise indicated.

Guide to Examinations:

- All vessels

- Vessels carrying dangerous cargoes in bulk

| # Pre-inspection Items: | Post-inspection Items: |
| --- | --- |
| - Review MISLE records. - COI - Vessel Critical Profile | - Issue letters/certificates to vessel. |
| - Obtain copies of forms to be issued. | - Complete MISLE Activity |
|  | - Initiate Report of Violation (ROV) if necessary. |

Table of Contents:

## Section 1:  Administrative Items

IMO Applicability Dates	1

Involved Parties & General Information	2

Vessel Information	3

Vessel Description	3

## Section 2:  Certificates and Documents

Certificates	8

Logs and Manuals	9

Pollution Prevention Records	10

Chemical Cargo Records	11

## Section 3:  Inspection Items for All Vessels

Navigation Equipment	12

General Health and Safety	15

Structural Integrity	17

Ground Tackle	19

Cargo Operations	19

Inert Gas Systems	22

Vapor Control Systems	22

Lifesaving Equipment	23

Fire Protection	29

Pollution Prevention	32

Marine Sanitation Devices	34

MTSA/ISPS Compliance	35

## Section 4:  Additional Inspection Items: Vessels Carrying Dangerous Chemicals in Bulk

Emergency Equipment	45

Cargo Operations	46

Cargo Tanks	46

Cargo Piping	47

Quick-closing Valves	48

Venting	48

Gauging	49

Pump rooms	50

Electrical Systems	50

Gas Detection	51

## Section 5:  Drills

Fire Drill	52

Abandon Ship Drill	53

Security Drill	54

## Section 6:  Appendices

Vessel Layout 	55

Cargoes Requiring a Response Plan	57

Prohibited Chemical Cargoes	58

Recommended US Vessel Deficiency Procedures	59

Deficiency Summary Worksheet	60

Conversions	61

# Section 1:  Administrative Items

IMO Applicability Dates:

| Reference | # Date |
| --- | --- |
| SOLA8 1960 | 26 MAY 65 |
| SOLAS 1974 | 25 MAY 80 |
| 1978 Protocol to SOLAS 1974 | 01 MAY 81 |
| 1981 Amendments (II-1 & II-2) | 01 SEP 84 |
| 1983 Amendments (III) | 01 JUL 86 |
| Various additional amendments to SOLAS |  |
| MARPOL 73/78 Annex I MARPOL 73/78 Annex II MARPOL 73/78 Annex III MARPOL 73/78 Annex V MARPOL 73/78 Annex VI | 02 OCT 83 06 APR 87 01 JUL 92 31 DEC 88 19 MAY 05 |
| IBC Code BCH Code | After 01 JUL 86 Prior to 01 JUL 86 |
| IGC Code | After 01 JUL 86 |
| IGC Code (for existing vessels) | Prior to 01 JUL 86 |
| COLREGS 1972 | 15 JUL 77 |
| Various additional amendments to COLREGS |  |
| Load Line 1966 | 21 JUL 68 |
| STCW 1978 | 28 APR 84 |
| 1991 Amendments 1994 Amendments 1995 Amendments | 01 DEC 92 01 JAN 96 01 FEB 97 |

Involved Parties & General Information:

Vessel’s Representatives

Phone Numbers

Owner—Listed on DOC or COFR

- No Change

Operator

- No Change

Vessel Information:

| Classification Society |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ISM Issuer: Same as above? |  |  |  |  |  |  |  |
| - Yes |  | - No If not the same, which Recognized Organization? |  |  |  |  |  |
| NOTE: The period of validity for ISM documents should correspond to the following list. If they do NOT, ISM documents should be further investigated. |  |  |  |  |  |  |  |
| - 5 years = Full term (SMS and DOC) - 6 months = Interim (SMC) |  |  |  | - 12 months = Interim (DOC) - 5 months = Short term (SMC) |  |  |  |
| Gross Tons |  |  |  |  |  | - No Change |  |
| Net Tons |  |  |  |  |  | - No Change |  |
| Built Date (use delivery date) |  |  |  |  |  | - No Change |  |
| Overall Length (in feet) |  |  |  |  |  | - No Change |  |
| Does vessel meet double-hull requirements? |  |  |  |  |  |  |  |
| - Yes | - No If not, vessel must meet requirements by (date) in accordance with 33 CFR Part 157 Appendix G. |  |  |  |  |  |  |
| Last Three Cargoes |  |  |  |  |  |  |  |
| Is pumproom gas-free? |  |  | - Yes |  | - No |  | - N/A |

Vessel Description:

| - Crude Carrier | - Combination |
| --- | --- |
| - Product Carrier | - Oil / Bulk / Ore |
| - Chemical Carrier | - Other |

# Section 2:  Certificates and Documents

| Endorse Date |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Exp. Date |  |  |  |  |  |  |
| Issue Date |  |  |  |  |  |  |
| Port Issued |  |  |  |  |  |  |
| ID # |  |  |  |  |  |  |
| Issuing Agency | USCG |  | # USCG |  | # USCG |  |
| Name of Certificate | Certificate of Documentation - No Change | Classification Document - No Change | Certificate of Financial Responsibility (COFR) - No Change | Safety Construction (SLC) - No Change | Safety Equipment (SLE) - No Change | Safety Radio (SLT) - No Change |

| Endorse Date |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Exp. Date |  |  |  |  |  |  |
| Issue Date |  |  |  |  |  |  |
| Port Issued |  |  |  |  |  |  |
| ID # |  |  |  |  |  |  |
| Issuing Agency |  |  | USCG |  |  |  |
| Name of Certificate | International Load Line - No Change | International Oil Pollution Prevention w/Form B (IOPP) - No Change | Certificate of Fitness (COF) - No Change | International Tonnage (ITC) - No Change | Safety Management (SMC) - No Change | Document of Compliance (DOC) - No Change |

| Exp. Date |  |  |  |
| --- | --- | --- | --- |
| Issue Date |  |  |  |
| Port Issued |  |  |  |
| ID # |  |  |  |
| Issuing Agency |  |  |  |
| Name of Certificate | International Ship Security Certificate (ISSC) - No Change | International Air Pollution Prevention Certificate (IAPP) - No Change | Engine International Air Pollution Prevention Certificate (EIAPP) - No Change |

Continuous Synopsis Record:

(SOLAS Vessels only)

Review Record and Enter Most Current Data

| Flag State: |  |
| --- | --- |
| Date Registered: |  |
| Ship ID #: |  |
| Ship Name: |  |
| Port of Registry: |  |
| Registered Owners: |  |
| Company-as defined in SOLAS Chapter IX: |  |

Certificates:

| - COI posted - All pages visible | 46 CFR 31.05-5 46 CFR 153.901 |  |
| --- | --- | --- |
| - ISM Code - Safety Management System - Shipboard Operations Plan | SOLAS 74/78 IX/3 |  |
| - Stability letter posted | 46 CFR 35.08-1 |  |
| - Waste management plan (oceangoing vessels  40 feet) | 33 CFR 151.57 |  |
| - Annual drug and alcohol program audit - Transportation Worker ID Credential (TWIC) - All MMC holders - All non-MMC holders with security duties or unrestricted access to restricted areas. | 46 CFR Part 16 46 CFR 10.113 & 12.01-11 |  |
| - Officers’ licenses current | 46 CFR 35.05-1 |  |
| - GMDSS endorsement - FCC Station License - Safety Radio Certificate | 47 CFR Part 80 |  |
| - GMDSS maintenance personnel - 1st Class Radiotelegraph Operator’s Certificate - 2nd Class Radiotelegraph Operator’s Certificate - 3rd Class Radiotelegraph Operator’s Certificate | 47 CFR 80.1074 |  |
| - Required international safety convention certificates posted and valid | 46 CFR 31.40-35 |  |
| - Liferaft servicing certificates - Annual service | 46 CFR 160.151-57(p) SOLAS 74/78 III/19.8 |  |
| - Cargo Gear Certificate | 46 CFR 31.37-75 |  |

Notes:

Logs and Manuals:

| - Lifesaving equipment maintenance record - Periodic checks as required - Visual inspection of survival craft / rescue boat and launching appliances - Operation of lifeboat / rescue boat engines - Lifesaving appliances, including lifeboat equipment examined | 46 CFR 199.190(e) SOLAS 74/78 III/19 |
| --- | --- |
| - Emergency training and drills - Onboard training in use of lifesaving equipment (all crew members) - SOLAS training manual - Logbook records - Fire and lifeboat drills - General alarm tested | SOLAS 74/78 III/18 46 CFR 199.180 SOLAS 74/78 III/18.5 SOLAS 74/78 III/25 |
| - Bridge log - Pre-arrival tests conducted - Casualties (navigation equipment and steering gear failures reported) - Steering gear drills - Emergency steering drills | SOLAS 74/78 V/19 STCW 95 I/14 33 CFR 164.25 33 CFR 164.53 46 CFR 35.07 |
| - Information available to master (as required) - Loading manual - Trim and stability book | 46 CFR 42.15-1 46 CFR 31.10-32 46 CFR 45.105(a) |
| - Cargo and ballast system instruction manual | 33 CFR 157.23 46 CFR 153.806 |

Notes:

Pollution Prevention Records:

| - Oil record book - Each operation signed by person-in-charge - Each complete page signed by master - Book maintained for 3 years | MARPOL Ax. I/20 33 CFR 151.25 |
| --- | --- |
| -  Review the Shipboard Oil Pollution Emergency Plan (SOPEP) for the following: - Approval from flag state or classification society - Written in English and working language of crew - Procedures for reporting oil pollution incidents - List of authorities or persons to be contacted in the event of an oil pollution incident - Action to be taken immediately by persons on board to reduce or control discharge of oil following an incident - Procedures and POC on the ship for coordinating shipboard action with national and local authorities in combating pollution | MARPOL 73/78 Annex I/26.1 33 CFR 151.26 |

| - Review Tank Vessel Response Plan (TVRP) for the following: - Verify that USCG TVRP Approval Letter is valid and not expired - Verify that your COTP Zone is an approved zone listed on letter - Verify that QI is identified in Plan (Optional – Recommended, not required – call QI and verify contact information in plan is accurate) - Verify if OSRO resources are adequate for vessel and they are listed for COTP Zone (Optional – Recommended, not required – call QI and ask for copy of OSRO contract for your COTP Zone) - Check plan for any recent updates/ significant changes made since date on Approval Letter. | 33 CFR 155.1030 33 CFR 155.1035 33 CFR 155.1065 33 CFR 155.1070 |
| --- | --- |
| - Verify vessel has access to shore-based computerized damage stability programs | 33 CFR 155.240 |
| - Oil transfer procedures - Posted - List of products carried by vessel - Description of transfer system including a line diagram of piping - Number of persons required on duty - Duties by title of each person - Means of communication - Procedures to top off tanks - Procedures to report oil discharges | 33 CFR 155.720 MARPOL 73/78 I/20 |

Chemical Cargo Records:

| - Approved Procedures & Arrangement Manual | 46 CFR 153.490(a) 46 CFR 153.490(b) |
| --- | --- |
| - Cargo record book | 46 CFR 153.490 |
| - Cargo information cards | 46 CFR 153.907 |
| - Cargo location plan - Cargo compatibility | 46 CFR 153.907 46 CFR Part 150 |
| - Cargo piping plan | 46 CFR 153.910 |
| - Shipping document | 46 CFR 153.907 |
| - Waiver letters carried | 46 CFR 153.10 |
| - Certificate of inhibition or stabilization - Date added to cargo - Length of time effective - Temperature limitations | 46 CFR 153.912 |
| - Current copy of 46 CFR Parts 35, 150, and 153 aboard | 46 CFR 153.905 |

Notes:

# Section 3:  Inspection Items for All Vessels

Navigation Equipment:

| - Navigation publications (as applicable) - Current and corrected charts - U.S. Coast Pilot - Great Lakes Pilot - Sailing directions - Coast Guard Light List - Notice to mariners - Tide tables - Tidal current tables - International Rules of the Road - Inland Rules of the Road | 46 CFR 35.20-1 33 CFR 164.33 SOLAS 74/78 V/20 |
| --- | --- |
| - Operationally test radar(s) and ARPA - 2 required if over 10,000 GT - Operate independently - ARPA acquires targets | 46 CFR 32.15-30 33 CFR 164.35 33 CFR 164.37 33 CFR 164.38 SOLAS 74/78 V/12 |
| - Compasses - Illuminated gyrocompass with repeater at stand - Illuminated magnetic compass - Current deviation table | 46 CFR 32.15-35 33 CFR 164.35 SOLAS 74/78 V/12 |
| - Test electronic depth sounding device and recorder - Accurate readout - Test all transducers - Continuous recorder (chart) | 46 CFR 32.15-10 33 CFR 164.35 SOLAS 74/78 V/12 |
| - Speed and distance indicator | 33 CFR 164.40 SOLAS 74/78 V/12 |
| - Propulsion shaft tachometer | SOLAS 74/78 V/12 |

Notes:

| - Radio equipment - Radios, RDF, Loran - Electronic position fixing device tested - GMDSS meets requirements for vessel operating area | 47 CFR Part 80 33 CFR 26.03 SOLAS 74/78 V/7 SOLAS 74/78 V/12 33 CFR 164.41 NVIC 9-93 |
| --- | --- |
| - Internal communications and control system - EOT failure alarms - Telephones - Voice tubes - Emergency loudspeaker system - Public address system - Bell pulls - Pilothouse controls | 46 CFR 113.30 46 CFR 113.35 MSM Ch. B1.J.4 46 CFR 32.30-1 46 CFR 113.50 |
| - Navigation lights and signals - Control panels - Running lights - Anchor lights - Special day and night signals - Distress signals and stowage - Flag signals, international code - Whistle, light, bells, gongs - Day and night signal devices - Certificate of Alternate Compliance | 72 COLREGS 46 CFR 111.75-17 MSM Ch. B1.J.5.c MSM Ch. C2.J 46 CFR 32.15-5 33 CFR 81.9 |
| - Steering gear - Main gear tested (all stations) - Auxiliary gear tested (all stations) - Instructions and markings - Rudder angle indicator - Illumination - Alarms - Block diagram | 46 CFR 31.10-15 46 CFR 61.20-1 46 CFR 58.25 33 CFR 164.35 33 CFR 164.39 46 CFR 113.43 SOLAS 74/78 II-1/29 SOLAS 74/78 II-1/30 SOLAS 74/78 V/19-1 SOLAS 74/78 V/19-2 MSM Ch. C4.F NVIC 1-69 NVIC 1-81 |

Notes:

| - Maneuvering facts sheet with warning statements | 33 CFR 164.35 |
| --- | --- |
| - EPIRB (406 MHz) - Float-free arrangement - Battery date current - Hydrostatic release | SOLAS 74/78 IV/7.1.6 46 CFR 199.510 NVIC 2-93 |
| - GMDSS lifeboat radios (VHF) - 3 if over 500 GT - Operable condition | SOLAS 74/78 III/6.2 NVIC 9-93 |
| - 9 GHz radar transponder (SART) - Vessels > 300 GT and < 500 require 1 - Vessels > 500 GT require 2 - Stowed so to be rapidly placed in survival craft, or stowed in survival craft | SOLAS 74/78 III/6.2 NVIC 9-93 |
| - NAVTEX | SOLAS 74/78 IV/7.1.4 |
| - Voyage Data Recorder (VDR) - Simplified (SVDR) if permitted | SOLAS 74/78 V/20 |
| - Long Range Identification & Tracking (LRIT) - Conformance Test Report - Automatic Identification System (AIS) | 33 CFR 169 SOLAS 74/78 V19-1 33 CFR 164.46 |

Notes:

General Health and Safety:

| - Hospital and first aid equipment | 46 CFR 32.40-35 |
| --- | --- |
| - Operating room explosion-proof | 46 CFR 111.105-37 |
| - Emergency lighting | 46 CFR 112.43 46 CFR 35.10-15 SOLAS 74/78 II-2/43.2 |
| - Crew accommodations - Size - Lighting and wiring - Heating - Ventilation - Sanitation - Screens - Insulation - Fire retardant | 46 CFR 32.40 46 CFR 31.10-45 |
| - Galley - Equipment - Sanitation - Ventilation | MSM Ch. B1.N.8 |
| - Means of escape from accommodation, machinery, and other spaces - Two required (some exceptions) - Dead end corridors - Absence of locks | 46 CFR 32.02-1 SOLAS 74/78 II-2/45 |
| - Notices and markings where required - Conspicuous - Legible - Proper size | 46 CFR 35.40 |
| - Gas freeing for repairs - Current Gas Chemist Certificate for areas as required - Date - Chemist No. | MSM Ch. A5.H |
| - Paint stowage - Closures - Fireproof / metal lined - Lighting / electrical - Fire protection - Markings | 46 CFR 32.85 SOLAS 74/78 II-2/18.7 |
| - Storerooms - Stowage - Fire hazards - Lighting and wiring |  |
| - Oxygen sensor (oxygen analysis equip.) | SOLAS 74/78 VI/3.1 |
| - Combustible gas indicator | 46 CFR 35.30-15 NVIC 12-86 |
| - LPG systems, cylinders, tests | 46 CFR 61.15-10 |

| - Pilot ladder and hoists in good condition - Illumination - Spreaders | 46 CFR 32.90-1 SOLAS 74/78 V/17 |
| --- | --- |
| - Intrinsically safe portable radios | 46 CFR 35.30-30 |
| - Safe access to tanker bows (vessels built prior to 1 JUL 98 not required to comply until 1 JUL 2001) | SOLAS 74/78 II-1/3-3 |

Notes:

Structural Integrity:

| - Hull structure (list inaccessible compartments or areas) - Decks - Shell - Bulkheads - Tank tops - Strength members - Approved plans on board | 46 CFR 31.10-15 ICLL 66 Reg. 1 46 CFR 31.10-22 |
| --- | --- |
| - Hull openings and closures - Side ports - Air ports and dead covers - Refuse chutes - Cargo tank hatches - Butterworth plates - Closing devices, gaskets - Light/water test | 46 CFR 42.13-20 46 CFR 42.15 MSM Ch. B1.E.5 ICLL 66 Regs. 12 - 23 NVIC 5-82 NVIC 10-82, Change 1 |
| - Deck openings and closures - Closing devices - Gaskets - Light/water test | MSM Ch. B1.E.5 ICLL 66 Regs. 12 - 20 |
| - Guards, ladders, rails, and gangways (including accommodation ladders or pilot ladders) | 46 CFR 32.02-10 46 CFR 32.02-15 ICLL 66 Reg. 25 |
| - Catwalks, lifelines at hazardous places, cable traveler | 46 CFR 32.02-10 |
| - Elevators | 46 CFR 111.91 |
| - Watertight doors in subdivision bulkhead tested by: - Local and remote control - Alarms - Markings | 46 CFR 111.97 46 CFR 170.248 46 CFR 170.270 |
| - Cargo gear examined (in absence of Cargo Gear Certificate) - Records - Safe Working Load markings | 46 CFR 31.10-16 46 CFR 31.37 |
| - Exercise valves and controls - Bilge valves - Overboard discharge valves - Equalizing valves - Emergency shutoff valves - Scupper valves - Remote control - Reach rods | 46 CFR 38.10 |
| - Bilge wells, cofferdams, and suctions | 46 CFR 56.50-50 |
| - Bulkhead penetrations | MSM Ch. B1.E.6 |
| - Piping protection - Removable guards (where required) - Baggage spaces | MSM Vol. IV Ch. 3.G.2.b |
| - Hull marks - Name - Hailing port - Official number - Net tonnage | 46 CFR 67.120 46 CFR 32.05-10 |

| - Draft marks - Legible - Properly sized - Properly spaced | 46 CFR 32.05-1 |
| --- | --- |
| - Load line marks - Conform to certificate - Legible | 46 CFR 31.25-1 46 CFR 42.07-5 46 CFR 42.13-20 ICLL 66 Regs. 4 - 9 |

Notes:

Ground Tackle:

| - Anchors - Tested - Windlass - Capstans - Automatic tensioning device | 46 CFR 32.15-15 ABS Rules |
| --- | --- |
| - Mooring, standing and running gear (other than gear covered by Cargo Gear Certificate) | 46 CFR 32.15-15 ABS Rules |
| - Emergency towing arrangements (vessels  20,000 DWT only) - Approved by Administration | SOLAS 74/78 II-2/3-4 |

Cargo Operations:

| - Warning notices and signals posted | 46 CFR 35.30-1 |
| --- | --- |
| - Pump rooms/pumps - Pumps and controls - Relief valves - Cofferdams - Ventilation - Bulkhead penetrations - Bilges clean - Free of excessive vapors - Closures - Remote shutdown - Electrical controls outside compartment | 46 CFR 32.60-20 46 CFR 32.50-1 46 CFR 35.35-70 46 CFR 32.55-45 46 CFR 32.55-1 46 CFR 32.50-1 46 CFR 32.52-5 46 CFR 34.15-35 46 CFR 32.50-35 46 CFR 111.105 |

Notes:

| - Cargo tanks - Trunks and hatches - Ullage openings - Liquid level gauges - Open - Restricted - Closed - Liquid level devices - Deck penetrations - Heating coils - Explosion-proof electrical fittings - Overfill device |  | 46 CFR 32.20-20 46 CFR 39.20-3 46 CFR 32.22T 46 CFR 32.50-15 46 CFR 111.105 33 CFR 155.840 |
| --- | --- | --- |
| - Cargo piping - Expansion joints - Controls - Supports - Hoses - Valves |  | 46 CFR 32.50-15 33 CFR 156.170 46 CFR 56.30 46 CFR 56.35 46 CFR 56.20 |
| - Hoses - External examination - Hydrostatic test - Markings | 46 CFR 32.50-30 33 CFR 155.800 33 CFR 156.170 33 CFR 154.500 |  |
| - Cargo oil containment - Size - Drains - Scupper closures |  | 33 CFR 155.310 SOLAS 74/78 II-2/59 |

Notes:

| - Cargo tank venting - Common header system - P/V valves - Flame arrestors - Flush and drain connections - Inert gas controls - Piping - Independent PV valves - Flame screen - Independent goosenecks - Flame screen - Closure device | 46 CFR 32.55-20 46 CFR 32.20-5 46 CFR 32.20-10 46 CFR 151.15-5 46 CFR 32.55-20 46 CFR 32.20-10 46 CFR 32.55-25 46 CFR 32.20-10 |
| --- | --- |
| - Explosion-proof fixtures | 46 CFR 111.105 |
| - Independent tanks - External examination - Date of internal examination - Date of hydrostatic test - Saddles; foundation and stowage - Piping and valves - Relief valves - Securing devices - Cargo hose - Electrical grounding - Authorized cargo | 46 CFR 32.60-30 46 CFR 153.251 |
| - Weather decks - Sources of vapor ignition - Doors, ports, scuttles,, gaskets, and closures satisfactory - Portable window air conditioners and fans - Ventilation systems | 46 CFR 111.105 46 CFR 32.56-21 |
| - Air compressor intakes - Prohibited locations | 46 CFR 32.35-15 |

Notes:

Inert Gas Systems:

| - Purity of nitrogen | 46 CFR 32.53 SOLAS 74/78 II-2/62 MSM Ch. C5 |
| --- | --- |
| - % concentration of nitrogen in vapor space |  |
| - % oxygen vapor |  |
| - Provisions for hold space and tank pad |  |
| - Sampling/testing of gas pad |  |
| - Gas generator or spare gas on board | 46 CFR 32.53-20 |
| - General - Supply capacity adequate - Positive pressure - Independent blowers - Oxygen detector and recorder - Pressure indicator and recorder - Portable detecting instruments - Alarms and controls - High oxygen alarm (> 8%) - Low pressure alarm (< 100mm) - Loss of water supply to deck water seal alarm and shutdown - High inert gas temperature (> 150F) alarm and shutdown - Loss of water supply to scrubber alarm and shutdown - Automatic shutdown valve - Instruction manual | 46 CFR 32.50-30 46 CFR 32.53-45 46 CFR 32.53-60 46 CFR 32.53-70 46 CFR 32.53-75 46 CFR 32.53-85 |

Vapor Control Systems:

| - Piping - Drain lines - Electrically bonded to hull - Flange stud - Vapor connection painted red / yellow / red and labeled vapor in 2-inch black letters | 46 CFR 39.20-1 |
| --- | --- |

Notes:

| - Closed gauging arrangement | 46 CFR 39.20-3 |
| --- | --- |
| - Liquid overfill protection - High-level and tank overfill alarms - Alarm with automatic shutdown system - Spill valve - Rupture disk - Intrinsically safe - Audible and visual alarms - Operational test | 46 CFR 39.20-9 |

Lifesaving Equipment:
NOTE:  Exemptions and alternatives for vessels not subject to SOLAS can be found in 
46 CFR 199.600.

| - General alarms - Controls - Batteries and fuses - Tested - Markings - Bell locations audible | 46 CFR 32.25 46 CFR 113.25 SOLAS 74/78 III/6.4 46 CFR 35.40 |
| --- | --- |
| - Type of lifeboat - Davit launched - Free fall |  |
| - Lifeboats stripped, cleaned and inspected - Date of annual servicing | 46 CFR 199.190(f) |
| - Lifeboats and work boats - Hull and fittings - Tanks and fittings - Cradles - Gripes - Compressed air cylinders - Markings | 46 CFR 199.201 46 CFR 199.202 46 CFR 199.261 46 CFR 199.262 SOLAS 74/78 III/19.2 SOLAS 74/78 III/26 MSM Ch. C2.H.2 46 CFR 199.176 |

Notes:

| - Lifeboat equipment and stowage (use table below to determine required equipment) |  |  |  |  | 46 CFR 199.175 SOLAS 74/78 III/41 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ###### Item Number | Item | International Voyage |  | Short International Voyage |  |  |  |
|  |  |  | Lifeboat | Rescue Boat | Lifeboat | Rescue Boat |  |  |
|  | ###### 1 | Bailer1 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 2 | Bilge pump2 | 1 |  | 1 |  |  |  |
|  | ###### 3 | Boathook | 2 | 1 | 2 | 1 |  |  |
|  | ###### 4 | Bucket3 | 2 | 1 | 2 | 1 |  |  |
|  | ###### 5 | Can opener | 3 |  | 3 |  |  |  |
|  | ###### 6 | Compass | 1 | 1 | 1 | 1 |  |  |
|  | ###### 7 | Dipper | 1 |  | 1 |  |  |  |
|  | ###### 8 | Drinking cup | 1 |  | 1 |  |  |  |
|  | ###### 9 | Fire extinguisher | 1 | 1 | 1 | 1 |  |  |
|  | ###### 10 | First aid kit | 1 | 1 | 1 | 1 |  |  |
|  | ###### 11 | Fishing kit | 1 |  |  |  |  |  |
|  | ###### 12 | Flashlight | 1 | 1 | 1 | 1 |  |  |
|  | ###### 13 | Hatchet | 2 |  | 2 |  |  |  |
|  | ###### 14 | Heaving line | 2 | 2 | 2 | 2 |  |  |
|  | ###### 15 | Jackknife | 1 |  | 1 |  |  |  |
|  | ###### 16 | Knife1,4 |  | 1 |  | 1 |  |  |
|  | ###### 17 | Ladder | 1 | 1 | 1 | 1 |  |  |
|  | ###### 18 | Mirror, signalling | 1 |  | 1 |  |  |  |
|  | 19 | Oars, units 5,6 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 20 | Painter | 2 | 1 | 2 | 1 |  |  |
|  | ###### 21 | Provisions (units/persons) | 1 |  |  |  |  |  |
|  | ###### 22 | Pump7 |  | 1 |  | 1 |  |  |
|  | ###### 23 | Radar reflector | 1 | 1 | 1 | 1 |  |  |
|  | ###### 24 | Rainwater collection device | 1 |  | 1 |  |  |  |
|  | ###### 25 | Repair kit7 |  | 1 |  | 1 |  |  |
|  | ###### 26 | Sea anchor | 1 | 1 | 1 | 1 |  |  |
|  | ###### 27 | Searchlight | 1 | 1 | 1 | 1 |  |  |
|  | ###### 28 | Seasickness kit (units/person) | 1 |  | 1 |  |  |  |
|  | ###### Item Number | Item | International Voyage |  | Short International Voyage |  |  |  |
|  |  |  | Lifeboat | Rescue Boat | Lifeboat | Rescue Boat |  |  |
|  | ###### 29 | Signal, smoke | 2 |  | 2 |  |  |  |
|  | ###### 30 | Signal, hand flare | 6 |  | 6 |  |  |  |
|  | ###### 31 | Signal, parachute flare | 4 |  | 4 |  |  |  |
|  | ###### 32 | Skates and fenders8 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 33 | Sponge7 |  | 2 |  | 2 |  |  |
|  | ###### 34 | Survival instructions | 1 |  | 1 |  |  |  |
|  | ###### 35 | Table of lifesaving signals | 1 |  | 1 |  |  |  |
|  | ###### 36 | Thermal protective aids9 | 10% | 10% | 10% | 10% |  |  |
|  | ###### 37 | Tool kit | 1 |  | 1 |  |  |  |
|  | ###### 38 | Tow line10 | 1 | 1 | 1 | 1 |  |  |
|  | ###### 39 | Water (liters/person) | 3 |  | 3 |  |  |  |
|  | ###### 40 | Whistle | 1 | 1 | 1 | 1 |  |  |
|  | Footnotes: - Each rigid liferaft equipped for 13 persons or more must carry two of these items. - Not required for boats of self-bailing design. - Not required for inflated or rigid-inflated rescue boats. - A hatchet counts towards this requirement in rigid rescue boats. - Oars are not required on a free-fall lifeboat; a unit of oars means the number of oars specified by the boat manufacturer. - Rescue boats may substitute buoyant paddles for oars, as specified by the manufacturer. - Not required for a rigid rescue boat. - Required if specified by the boat manufacturer. - Sufficient thermal protective aids are required for at least 10% of the persons the survival craft is equipped to carry, but not less than two. - Required only if the lifeboat is also the rescue boat. |  |  |  |  |  |  |  |

Notes:

| - Disengaging apparatus examined or tested and marked as required* - Universal joints - Safety latches - Hooks - Locking knuckles - Frame | 46 CFR 199.45(b) SOLAS 74/78 III/41 MSM Ch. C2.H.b |
| --- | --- |
| - Motor lifeboats* - Engine operating test - Cooling water pump - Ahead and astern test - Fuel tanks - Searchlight test (passenger vessels) - Annual fuel changed - Extinguishers serviced | 46 CFR 199.45(b) SOLAS 74/78 III/41 MSM Ch. C2.H.2 |
| - Hand-propelled lifeboats* - Flemming gear - Transmission oil | 46 CFR 199.45(b) |
| - Radio installation for lifeboats complies with FCC and/or international convention (where required)* - Portable - Fixed | 46 CFR 199.60 SOLAS 74/78 III/6 |
| - Lifeboat operational test* | 46 CFR 199.45(b) SOLAS 74/78 III/19 MSM Ch. C2.H.2.b |
| - Davits - Foundations - Moving parts - Fittings - Fairleads, cleats, or cruciform bits | 46 CFR 160.032 SOLAS 74/78 III/48 MSM Ch. C2.H.2.b |
| - Falls (date last renewed or end-for-ended) | 46 CFR 199.190(j) SOLAS 74/78 III/48 MSM Ch. C2.H.8 |
| * Items tested in conjunction with Abandon Ship Drill. |  |

Notes:

| - Lifeboat winches - Brakes - Controls - Cranks - Covers - Limit switches and electrical controls | 46 CFR 199.199(i) SOLAS 74/78 III/19 MSM Ch. C2.H.2.b |
| --- | --- |
| - Embarkation aids - Ladders - Access - Spans and lifelines - Illumination - Frapping and tricing lines | 46 CFR 199.190(f) SOLAS 74/78 III/11 |
| - Lifeboat weight test - Light load - Full load: date | 46 CFR 199.45(b) SOLAS 74/78 III/19 MSM Ch. C2.H.6 |
| - Liferafts - Launching instructions posted - Equipment and stowage - Annual service dates - Hydro release service dates - Weak link - Float free - Illumination - Markings - Capacities - Launching devices tested | 46 CFR 199.190 SOLAS 74/78 III/19 SOLAS 74/78 III/26 MSM Ch. C2.H.3 46 CFR 199.190(g) 46 CFR 199.190(h) MSM Ch. 6.R.3.d |

Notes:

| - Lifefloats and buoyant apparatus - Equipment - Stowage - Markings | 46 CFR 199.640 |
| --- | --- |
| - Line-throwing apparatus - Equipment - Required drills held - Magazine - Type | 46 CFR 199.170 SOLAS 74/78 III/17 NVIC 8-69 |
| - Lifebuoys - Lights - Lines - Smoke signals - Stowage - Markings | 46 CFR 199.70(a) SOLAS 74/78 III/7 SOLAS 74/78 III/27 |
| - Lifejackets Adult Children - Retro-reflective tape - Lights - Whistles - Work vests - Stamped passed - Number of lifejackets rejected by inspector | 46 CFR 199.70(b) SOLAS 74/78 III/7.2.2 MSM Ch. 6.R.3.m NVIC 2-63 MSM Ch. 6.R.3.q |
| - Lifejacket stowage - Required notices and markings - Stowage lockers - Wearing instructions - Location instructions (passenger vessels) | 46 CFR 199.70(b) SOLAS 74/78 III/7.2 46 CFR 199.80(c) 46 CFR 199.70(b)(2) 46 CFR 199.80(c) 46 CFR 199.217 |
| - Work vests - Approved type - Stowage | 46 CFR 35.03 46 CFR 35.03-5 46 CFR 35.03-15 |
| - Stateroom notices posted | 46 CFR 199.80(c) |

Notes:

| - Immersion suits - Condition - Retro-reflective material | 46 CFR 199.273 SOLAS 74/78 III/27.3 SOLAS 74/78 III/30.2.7 MSM Ch. C2.H.5 |
| --- | --- |
| - Placard of lifesaving signals | 46 CFR 35.12 SOLAS 74/78 V/16 |
| - Pilot ladder and hoists in good condition | 46 CFR 32.90-1 SOLAS 74/78 V/17 |
| - Station bill posted | 46 CFR 35.10-1 46 CFR 199.80 SOLAS 74/78 III/8.3 |
| - Distress signals - 12 rocket parachute flares | 46 CFR 199.60 SOLAS 74/78 III/6.3 |

Fire Protection Equipment:

| - Fire control plan - Permanently posted - Copy permanently stored in weather-tight container outside deckhouse | 46 CFR 35.10-3 SOLAS 74/78 II-2/20 |
| --- | --- |
| - Portable extinguishers - Annually serviced - Bottles hydrostatically tested (every 5 years) - Markings (weight and hydrostatic test date) - Spare charges, spare extinguishers | 46 CFR 34.05-10 46 CFR 34.50 46 CFR 31.10-18(a) SOLAS 74/78 II-2/6 SOLAS 74/78 II-2/21 MSM Ch. C2.I.3 NVIC 7-70 NVIC 13-86 |
| - Semi-portable extinguishers - Annually serviced - Bottles hydrostatically tested - Controls, instructions, markings - Hose and diffuser - Flexible loops tested or replaced (same as bottle) | 46 CFR 34.05-10 46 CFR 34.50 46 CFR 31.10-18(a) SOLAS 74/78 II-2/6 SOLAS 74/78 II-2/21 MSM Ch. C2.I.4 NVIC 13-86 |

Notes:

| - Fixed fire extinguishing systems - Controls, instructions, marking - Alarms tested - Piping - Heads, distribution - Bottles weighed annually - Bottles hydrostatically tested (every 12 years) - Flexible loops tested or replaced (10% per year) - Deck foam system - Sea suction, strainers - Fixed system tested - Storage space/door |  |  |  |  | 46 CFR 34.05-5 46 CFR 34.15 46 CFR 34.17 SOLAS 74/78 II-2/21 MSM Ch. C2.I NVIC 6-70 NVIC 6-72, Change 1 NVIC 8-73 46 CFR 31.10-18(a) 46 CFR 34.20 SOLAS 74/78 II-2/61 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Type of system: (circle appropriate type) |  |  |  |  |  |
|  | Low Pressure CO2 | High Pressure CO2 | Halon | Foam |  |  |
| - Foam tanks - Markings - Test - Analysis - Refilled - Polar/non-polar foam - Cargo compatible |  |  |  |  | 46 CFR 34.17 46 CFR 35.40-10 SOLAS 74/78 II-2/61 NVIC 6-72 NVIC 11-82 |  |
| - Fire main system - Pumps - Piping - Cut-off valves - Drains |  |  |  |  | 46 CFR 34.05 46 CFR 31.10-18(g) 46 CFR 34.10-5 46 CFR 31.10-18(f) 46 CFR 34.10-15(c) NVIC 6-72, Change 1 SOLAS 74/78 II-2/4 SOLAS 74/78 II-2/21 MSM Ch. C2.I.9 MSM Ch. C2.I.8 |  |

Notes:

| - Fire stations - Hydrants (2 effective streams) - Nozzles and spanners - Fog applicators (<6 feet in length, in engineroom) - Hose - UL approved - Correct length - Markings - Equipment compatible | 46 CFR 34.10-10 NVIC 6-72, Change 1 SOLAS 74/78 II-2/4 SOLAS 74/78 II-2/21 MSM Ch. 18.I.9 MSM Ch. 19.I.7 46 CFR 34.25-20 46 CFR 32.05-5 |
| --- | --- |
| - Total length of all hose tested - Hydrostatically tested to at least 100 psi - Proper threads - Approved hose | 46 CFR 34.10-10 MSM Ch. 18.I.6 46 CFR 31.10-18(g) |
| - Structural fire protection - Bulkheads - Insulation | 46 CFR 32.56 46 CFR 32.57 46 CFR 34.05 46 CFR 34.10 46 CFR 34.15 46 CFR 35.10-3 SOLAS 74/78 II-2/42 |
| - Fire doors and controls tested - Machinery space and stair towers - Not tied or blocked open - Installed closure devices working | 46 CFR 32.56-35 46 CFR 111.103-1 SOLAS 74/78 II-2/46 SOLAS 74/78 II-2/47 |
| - Remote controls to power ventilation marked and tested | 46 CFR 111.103-1 |
| - Closures for spaces protected by fixed smothering systems |  |
| - International shore connection | 46 CFR 34.10-15(d) SOLAS 74/78 II-2/19 |
| - Fire axe - Markings | 46 CFR 34.60 46 CFR 32.05-5 |

Notes:

| - Firemen’s outfits - Two lockers - Four outfits - Protective clothing - Helmet, boots, and gloves - Lamp - Axe - Self-contained breathing apparatus and lifeline - Spare charges | 46 CFR 35.30-20 SOLAS 74/78 II-2/17.3 NVIC 2-63 NVIC 4-68 |
| --- | --- |
| - Water spray system - Coverage (all tank domes, cargo manifolds, deck tanks) - Operation - Local/remote control - Manual or automatic - Tested - Can operate simultaneously with fire main systems - Controls marked - Material condition | 46 CFR 34.25 |

Pollution Prevention:

NOTE:  Guidance for inspecting pollution prevention items is detailed in MSM Volume II, Chapter 31.

| - Pollution placard posted |  | 33 CFR 155.450 |
| --- | --- | --- |
| - MARPOL V placard posted |  | 33 CFR 151.59 |
| - Person-in-charge designation |  | 33 CFR 155.700 33 CFR 155.820 |
| - Fuel oil containment - Portable - Fixed |  | 33 CFR 155.320 |
| - Fuel tank vents - Flame screens - Closures | 46 CFR 56.50-85 |  |

Notes:

| - Oily waste retention - Bilge - Tank |  | 33 CFR 157.17 MSM Ch. B6.D.7 |
| --- | --- | --- |
| - Ballast discharge - Acceptable processing equipment |  | 33 CFR 155.330 33 CFR 155.350 33 CFR 155.360 33 CFR 155.370 MSM Ch. B6.D.10 |
| - Oily bilge discharge - Piping system - Stop valve - Standard discharge connection - Pump stop |  | 33 CFR 155.380 33 CFR 155.430 |
| - Prohibited oil spaces |  | 33 CFR 155.470 |
| - Emergency shutdown |  | 33 CFR 155.780 |
| - Discharge removal equipment - Sorbents - Non-sparking tools - Containers - Emulsifiers - Protective clothing - Scupper plugs - Non-sparking portable pump |  | 33 CFR 155.205 33 CFR 155.210 |
| - Deck lighting |  | 33 CFR 155.790 |
| - Garbage - Shipboard garbage properly disposed | 33 CFR 151.63 MARPOL Ax. V/3 |  |
| - Discharge from machinery space bilges - Approved monitoring and control system - Approved oil water separating equipment | 33 CFR 151.10 |  |

Notes:

| - MARPOL Annex I survey - Discharge of cargo residue - Approved monitoring and control system | 33 CFR 151.09 |
| --- | --- |
| - MARPOL Annex II survey - Discharge of cargo residue - Approved monitoring and control system | 33 CFR 151.30 |

Marine Sanitation Devices:

NOTE:  Guidance for inspecting marine sanitation devices is detailed in MSM Volume II, Chapter C2.K.

| - Marine sanitation device - Type I - Type II - Type III | 33 CFR 159.55 MSM Ch. B6.F |
| --- | --- |
| - Certified for inspected vessels | MSM Ch. B6.F.4 |
| - Capacity satisfactory | MSM Ch. C2.K.7.d |
| - Installation - Operation - Ventilation - Wiring and piping - Maintenance - Placard posted - Safety - Accessibility to parts requiring routine servicing - Manufacturer’s instructions available | 33 CFR 159.57 MSM Vol. IV Ch. 3.K.1 33 CFR 159.59 |

Notes:

MTSA/ISPS Compliance

Vessel Security Plans (VSP):

(If using ASP, skip to next section)

| - Compliance documentation - Approved Vessel Security Plan | 33 CFR 1043120 ISPS, Part A, 9.1 |  |
| --- | --- | --- |
| - Waiver - Approved by CG-543 | 33 CFR 104.130 |  |
| - Equivalents - Approved by CG-543 | 33 CFR 104.135 |  |
| - Maritime Security (MARSEC) directive - Proper safeguards - Incorporated into VSP | 33 CFR 104.145 |  |
| - Master - Aware of responsibility and authority with regards to MTSA | 33 CFR 104.205 |  |
| - Company Security Officer (CSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.210 ISPS, Part A, 11.1 |  |
| - Vessel Security Officer (VSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.215 ISPS, Part A, 12.1 |  |

Notes:

| Company or vessel personnel with security duties - Training / experience - Valid TWIC - See list of example questions. - Security Training for all other vessel personnel. - Vessel Record Keeping Requirements - Training - Drills and exercises - Breaches of security - Change in MARSEC levels - Maintenance, calibration, and testing of security equipment. - Security threats - Annual audit of the VSP - Declaration of Security (DoS) - Retained for Two years | 33 CFR 104.220 ISPS, Part A, 13.1 33 CFR 104.225 ISPS, Part A, 13.5 CFR 104.235 ISPS, Part A, 10.1 |  |  |
| --- | --- | --- | --- |
| - MARSEC level coordination and implementation - Proper MARSEC level - MARSEC level at least at current port level | 33 CFR 104.240 |  |  |  |
| - Communications - Vessel security personnel - Facility - National and local authorities - Demonstrate communications operations consistent with the VSP | 33 CFR 104.245 ISPS, Part A, 7.2.7 |  |  |  |
| - Declaration of Security (DoS) - Required for cruise ships or manned CDC bulk vessels and any vessel or facilities with which it interfaces. - Valid (for MARSEC level and effective time period) Must have last 10 or continuous DoS reviewed at interval consistent with MARSEC level. - Signed | 33 CFR 104.255 ISPS, Part A, 5.1 |  |  |  |

| - Security systems and equipment maintenance - Testing completed IAW manufacturer’s recommendations - Working properly, effectively functions IAW VSP. - Ship Security Alert System (SSAS) | 33 CFR 104.260 |  |
| --- | --- | --- |
| - Security measures for access control - Access points examined – signs posted in conspicuous locations. - Control areas for authorized dangerous substances / devices - Means of identifying unauthorized personnel - TWIC for unescorted access to secure areas | 33 CFR 104.265 ISPS, Part A, 7.2.2 |  |
| - Security measures for newly hired employees (Pending receipt of TWIC) - Access permitted for up to 30 days if: - Has applied for TWIC - Accompanied by TWIC holder in secure areas - Operator enters new hire personal info in HOMEPORT - Notified via HOMEPORT that new hire has passed initial name check. - Provision does not apply to CSO, VSO or individual hired to perform security duties - Security measures for restricted areas - Secure areas protected - Properly marked - Control measures adequate - Do not conflict with safety measures | 33CFR 104.267 33 CFR 104.270 ISPS, Part A, 7.2.4 |  |
| - Security measures for handling cargo - Identifying cargo tamper - Identifying approved cargo - Access point – inventory control - Checking cargo for dangerous substances | 33 CFR 104.275 ISPS, Part A, 7.2.6 |  |
| - Security measures for delivery of vessel stores and bunker - Security procedures followed - Standing agreements valid | 33 CFR 104.280 ISPS, Part A, 7.2.6 |  |
| - Security measures for monitoring - IAW VSP - Lighting - Test intrusion alarms - Emergency search procedures | 33 CFR 104.285 ISPS, Part A, 7.2.5 |  |
| - Security Incident Procedures - Witness during drill - Additional requirements for passenger vessels or ferries | 33 CFR 104.290 33 CFR 104.292 |  |
| - Additional requirements for cruise ships | 33 CFR 104.295 |  |
| - Additional requirements – vessels on international voyages - ISSC issued - CSR updated | 33 CFR 104.297 ISPS, Part A, 19.1 SOLAS XI-2 Regulation 5.4.1 |  |
| - Vessel Security Assessment Report - Reviewed and attached to VSP - Vessel Security Plan - Reviewed - Amendment and audit - CSO / VSO audit letter attached to VSP as required ISSC - Audits conducted as required (annually or after vessel modifications) | 33 CFR 104.300 33 CFR 104.400 33 CFR 104.415 |  |
| - Ship Security Alert System (vessels subject to SOLAS only) - On the bridge and one other location - Designed to prevent inadvertent activation - Covert (unmarked, silent, and need to know) - Tested IAW VSP | ISPS Part A, 9.4.18 |  |

Notes:

# Alternative Security Program (ASP):

Vessels operating under the auspices of an approved ASP are required to address the relevant areas cited in 33 CFR parts 104. However, the ASP provision of the rule has provided a mechanism by which segments of the maritime industry, through application by the industry associations or other representative groups, are able to tailor their program to the unique functions inherent of their specific operations. The result is a set of relevant, performance-based security measures for the industry groups choosing to utilize an approved ASP. For this reason, the inspector of a vessel using an approved ASP may find that certain language or security measures contained in some parts of the rule will differ from the language or security measures listed in the ASP. An example would be the requirement in 33 CFR 104.265 (e) (3) that vessels check the identification of any person seeking to board the vessel at MARSEC Level 1. In an ASP, the approval authority may take into account the availability of video monitoring capable of facial feature recognition and recording and approve this as satisfying the intent of the requirement for individual identification. Additionally, an industry or group may determine that a section of the regulations is not applicable to their operations. For example, a passenger vessel group may state in their ASP that they do not need to address 33 CFR 104.275 or 33 CFR 105.265, respectively – security measures for handling cargo – because they do not handle cargo of any type.

In those cases where both the vessels and the facilities serving those vessels are owned and/or operated by the same entity, an alternative plan may recognize that the same party is responsible for security in both areas and approve an approach that addresses vulnerabilities and mitigation strategies for the vessels and the facility under one ASP. Therefore, the inspector will not be using separate plans for the vessels and the facility to determine compliance and, likewise, will not see some citations addressed in the plan if they are redundant between 33 CFR 104 and 33 CFR 105.

| - Name of approved ASP - ________________________________ | 33 CFR 104.140 |  |
| --- | --- | --- |
| - Compliance Documentation - Copy of ASP aboard vessel - Vessel Specific Security Assessment (VSA) completed - Letter to MSC stating use of approved ASP and that it has been fully implemented | 33 CFR 104.120 |  |
| - Master - Aware of responsibility and authority with regards to MTSA | 33 CFR 104.205 IAW ASP |  |
| - Company Security Officer (CSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.210 IAW ASP |  |
| - Vessel Security Officer (VSO) - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.215 IAW ASP |  |
| - Company or vessel personnel with security duties - Training / experience - Valid TWIC - See list of example questions | 33 CFR 104.220 IAW ASP |  |
| - Security Training for all other vessel personnel | 33CFR 104.225 IAW ASP |  |

Notes:

| - Vessel Record Keeping Requirements - Training - Drills and exercises - Breaches of security - Change in MARSEC levels - Maintenance, calibration, and testing of security equipment. - Security threats - Annual audit of the VSP - Declaration of Security (DoS) - Retained for Two years |  | 33 CFR 104.235 IAW ASP |  |
| --- | --- | --- | --- |
| - MARSEC level coordination and implementation - Proper MARSEC level - MARSEC level at least at current port level |  | 33 CFR 104.240 IAW ASP |  |
| - Communications - Vessel security personnel - Facility - National and local authorities - Demonstrate communications operations consistent with the ASP |  | 33 CFR 104.245 IAW ASP |  |
| - Declaration of Security (DoS) - Required for cruise ships or manned CDC bulk vessels and any vessel or facilities with which it interfaces. - Valid (for MARSEC level and effective time period) Must have last 10 or continuous DoS reviewed at interval consistent with MARSEC level. - Signed |  | 33 CFR 104.255 33 CFR 104 |  |

Notes:

| - Security systems and equipment maintenance - Testing completed IAW manufacturer’s recommendations - Working properly, effectively functions IAW VSP. - Ship Security Alert System (SSAS) |  | 33 CFR 104.260 IAW ASP |  |
| --- | --- | --- | --- |
| - Security measures for access control - Access points examined – signs posted in conspicuous locations. - Control areas for authorized dangerous substances / devices - Means of identifying unauthorized personnel - TWIC for unescorted access to secure areas |  | 33 CFR 104.265 IAW ASP |  |
| - Security measures for newly hired employees (Pending receipt of TWIC) - Access permitted for up to 30 days if: - Has applied for TWIC - Accompanied by TWIC holder in secure areas - Operator enters new hire personal info in HOMEPORT - Notified via HOMEPORT that new hire has passed initial name check. - Provision does not apply to CSO, VSO or individual hired to perform security duties - Security measures for restricted areas - Secure areas protected - Properly marked - Control measures adequate - Do not conflict with safety measures |  | 33 CFR 104.267 IAW ASP 33 CFR 104.270 IAW ASP |  |
| - Security measures for handling cargo - Identifying cargo tamper - Identifying approved cargo - Access point – inventory control - Checking cargo for dangerous substances |  | IAW ASP 33 CFR 104.275 |  |
| - Security measures for delivery of vessel stores and bunker - Security procedures followed - Standing agreements valid |  | 33 CFR 104.280 IAW ASP |  |
| - Security measures for monitoring - IAW VSP - Lighting - Test intrusion alarms - Emergency search procedures |  | 33 CFR 104.285 IAW ASP |  |
| - ASP Implemented in its entirety - Amendment and audit - Users of ASP’s are required under condition of ASP approval to conduct yearly audit and advise submitting organization if amendment is needed. - CSO / VSO audit letter attached to VSP as required ISSC - Audits conducted as required (annually or after vessel modifications) |  | 33 CFR 104.140 IAW ASP 33 CFR 104.415 IAW ASP |  |

Should an enforcement inspection reveal that an owner/operator has correctly implemented an approved ASP in its entirety but security vulnerabilities exist in the vessel operation, the COTP shall be advised.  Under 33 CFR 104.415 (a) (ii) for vessels or 33 CFR 105.415 (a) (ii) (f), the Coast Guard can determine that an amendment is necessary and advise the organization that submitted the ASP for approval accordingly. Following such notification, it will be necessary for the original submitting organization to provide their proposed amendment to the Commandant (CG-543) for review and approval. If the submitting organization does not wish to amend the ASP, the vessel owner must submit a VSP for the vessel to the MSC.

Notes:

Section 4:  Additional Inspection Items for Vessels Carrying Dangerous Chemicals in Bulk

Emergency Equipment:

| - Safety gear lockers - Proper locations (minimum of 2) - Labeled “Safety Equipment” - Proper type and amount of safety equipment |  |  |  | 46 CFR 153.214 |  |
| --- | --- | --- | --- | --- | --- |
|  | Equipment | Amount Required per Safety Gear Locker Location |  |  |  |
|  |  | Primary Shutdown Station | Secondary Shutdown Station |  |  |
|  | 2 stretchers with lifting equipment | 1 | 1 |  |  |
|  | 30-minute SCBA | 3 | 2 |  |  |
|  | SCBA refill tanks | 11 | 6 |  |  |
|  | First aid equipment | 2 | 1 |  |  |
|  | Overalls or long apron | 2 | 1 |  |  |
|  | Boots | 2 | 1 |  |  |
|  | Long-sleeved gloves | 2 | 1 |  |  |
|  | Goggles | 2 | 1 |  |  |
|  | Steel-cored lifeline with harness | 2 | 1 |  |  |
|  | Explosion-proof flashlight | 2 | 1 |  |  |
| - Self-contained breathing apparatus - Material condition - Operation |  |  |  |  |  |
| - Decontamination showers and eye wash on deck - Operation - Suitably marked |  |  |  | 46 CFR 153.216 |  |

Notes:

| - Crew respiratory/eye protection - EEBD > 15-minute duration | 46 CFR 153.527 |
| --- | --- |

Cargo Operations:

| - Warning signs and signals - Red signal visible, light/flag - Warning signs at gangway - “Warning” - “Dangerous Cargo” - “No Smoking” - “No Open Lights” | 46 CFR 153.953 46 CFR 153.955 |
| --- | --- |
| - Portable cargo hose - Marked or stenciled - Date - Tested - Test pressure - MAWP - Service temperature - Maximum - Minimum - Test pressure | 46 CFR 153.940 |
| - Cargo discharge methods - Pumproom - Deepwell pump - Submerged pump - Gas pressurization - Liquid displacement | 46 CFR 153.964 46 CFR 153.966 |

Cargo Tanks:

| - Trunks and hatches, ullage openings - Condition - Gaskets - Closure - Butterworth openings - Closed - Fitted with flame screens | 46 CFR 153.256 |
| --- | --- |

Notes:

| - Special tank linings | 46 CFR 153.266 |
| --- | --- |
| - Heating system - If toxic cargoes only, system must be separate from vessel’s heating system and contamination detection must be available | 46 CFR 153.430 |
| - Cooling system | 46 CFR 153.432 |
| - Cargo high pressure and temperature alarms - Audible - Visual - Location | 46 CFR 153.438 |

Cargo Piping:

| - Material condition - Annual hydrostatic test - Properly marked | 33 CFR 156.170 46 CFR 15.294 |
| --- | --- |
| - Valves - Tank valves - Hose connection points | 46 CFR 153.283 |
| - Agreement with plans - New piping - No unauthorized modification | 46 CFR 153.910 |
| - Prohibited materials - Aluminum or aluminum alloys - Copper or copper alloys - Zinc, galvanized steel, or alloys having > 10% zinc - Magnesium - Lead - Silver or silver alloys - Mercury | 46 CFR 153.236 |

Notes:

Quick-closing Valves:

NOTE:  Requirements for quick-closing valves are detailed in 46 CFR 153.284.

| - Operation - Tested from at least 2 remote locations - Closure time < 30 seconds - All valves fully closed - Pumps automatically shut off |  |
| --- | --- |
| - Capable of local manual operation - Reasonably short time - Under emergency conditions |  |
| - Fusible elements - Located at tank domes and loading manifold - No paint on face of plug |  |

Venting:

| - Type of vent system - Open, gooseneck - P/V - High velocity | 46 CFR 153.355 46 CFR 153.353 |
| --- | --- |
| - Vent piping - No stop valves allowed - Bypass capability - Material condition - Segregation - Independent - Common - Portable - Agreement with plans - Suitable connections for flushing and draining - Coated or lined same as tank - Prohibited materials | 46 CFR 153.361 46 CFR 153.362 46 CFR 153.236 |

Notes:

| - Vent outlets - Height above highest cargo area working level - Directed vertically upward - Flame arrestor (where required) - Weather hood - Flame screens - Located away from air intakes and openings to accommodation spaces | 46 CFR 153.350 46 CFR 153.351 46 CFR 153.352 |
| --- | --- |
| - P/V valves - Material condition - Operation - Flame screens | 46 CFR 153.368 46 CFR 153.463 |

Gauging:

| - Type of gauging system - Open - Restricted - Closed | 46 CFR 153.400 |
| --- | --- |
| - High-level alarms - Operation - Audible and visual signals - Last tested and inspected - Markings | 46 CFR 153.409 |
| - Overfill controls - Set points | 46 CFR 153.408 |
| - Cargo samples - Designated stowage locations - Ventilation | 46 CFR 153.935(a) |

Notes:

Pump rooms:

| - Ventilation WARNING: Pump room must be adequately ventilated prior to personnel entry. - Forced exhaust - Termination of external openings - Intake below and above floor plates - Operable from outside pumproom - Adequate volume - Material condition of ducting | 46 CFR 153.310 46 CFR 153.312 |
| --- | --- |
| - Pumps - Pressure gauges outside pump room and operable - Type of drive - Material condition | 46 CFR 153.333 |
| - Shaft seals (where motor rooms are installed) - Leakage - Vapor-tight - Adequate supply of oil (where required) |  |
| - General - Hoisting arrangement - Bilge pumping system - Operational - Remote controls - High-level alarm | 46 CFR 153.332 46 CFR 153.334 |

Electrical Systems:

NOTE:  Requirements for inspecting the following electrical items are detailed in Subchapter J as per 46 CFR 153.466.

| - Explosion-proof lighting fixtures - Tight globe - Heavy construction - Explosion-proof seals around cables | 46 CFR 111.105 |
| --- | --- |

Notes:

| - Cable - Condition |  |
| --- | --- |
| - Electrical fixtures on deck - Explosion-proof or intrinsically safe - No frayed or exposed wires - Material condition | NVIC 2-89 |

Gas Detection:

| - Portable gas detectors - Number - Type - Flammable gas detectors - Calibrated - Tested - Toxic gas detectors - Capable of measuring all toxic cargoes authorized to be carried - Calibrated | 46 CFR 153.465 46 CFR 153.526 |
| --- | --- |
| - Fixed gas detectors - Calibration - Crew knowledgeable in operation - Span gas available - Zeroing - Calibration curves available for all cargoes carried - Tested | 46 CFR 153.526 |

Notes:

# Section 5:  Drills

- Fire Drill:

| Initial notifications | Familiarity with duties | Space isolation |
| --- | --- | --- |
| General alarms / signals | Familiarity with equipment | Smoke control |
| Crew response | Fire pumps started | Communications w/bridge |
| Properly dressed / equipped | Two jets of water |  |
| Language understood by crew | Fire doors and dampers |  |

(SOLAS 74/78 III/18.3; MSM Vol. II/D5.C.7.i; NVIC 6-91)

| Location: | Time on Scene: |
| --- | --- |

Notes:

- Abandon Ship Drill:

| General alarms / signals | Familiarity with duties |  | Boat release |
| --- | --- | --- | --- |
| Muster lists | Provide equipment |  | Boat operation |
| Muster of crew | Familiarity with equipment |  | Egress procedures |
| Crew response | Lower lifeboat |  | Davit-launched liferaft drill |
| Language understood by crew | Brake operation |  | Communication w/ bridge |
| Lifejackets | Engine start |  | Lighting |
| (SOLAS 74/78 III/18.3; MSM Vol. II/D5.C.7.h) |  |  |  |
| Location: |  | Time to Water: |  |

Notes:

- Security Drill:

- Observe security drill exercising the activation of the provisions in the VSP or ASP related to a security threat, breach, security communications, change of security level, or other security related incident or action as describe in the VSP or ASP. - Drill selection and location shall be as directed by the Master and VSO. - Critique Drill with VSO/CSO

Location:

Notes:

Section 6:  Appendices

Vessel Layout:

Double hull/bottom/sides

Ballast tanks (SBT/CBT)

Chemical tank type:  I   II   III

Tank arrangement

Deckhouse location

External/internal framing

Layout of pumps – type

Bow

Stern

Stern

Bow

Cargoes Requiring a Response Plan:

| ##### Type of Cargo | Name of Cargo |  |  |
| --- | --- | --- | --- |
| Asphalt Solution | - Blending stocks | - Roofers stock | - Straight run residue |
| Animal Oils | - Tallow - Lard - Stearic acid | - Olive acid - Sperm oil | - Fish oil - Fish liver |
| Distillates | - Flashed feed stocks | - Straight run |  |
| Easenal Oils | - Pinene | - Turpentine | - Dipentine |
| Edible Oils | - Corn - Coconut | - Soybean - Olive | - Cotton seed |
| Gasolines | - Automotive - Aviation - Casinghead | - Polymer - Straight run - Gas, oil cracked | - Akylates - Reformates |
| Naptha | - Aromatic - Cracking fraction - Heavy | - Paraffinic - Petroleum - Solvent | - Stoddard solvent - Varnish makers |
| Oils | - Clarified oil - Crude oil - Fuel oils [# 1 (Kerosene), # 2, # 2D, # 4, # 5, # 6] - Residual fuel oil - Transformer oil - Lube oil and blending stock | - Aromatic oil (excluding vegetable oil) - Mineral oil - Motor oil - Penetrating oil - Spindle oil - Turbine oil - Octene | - Olefin - Animal - Range - Residual - Resin - Road - White (mineral) |

Prohibited Chemical Cargoes:

The following cargoes have been determined to be too hazardous to be carried in U.S. waters:

- Acrolein

- Chlorine (on self-propelled vessels)

- Ethylenimine

- Hydrofluoric Acid

- Hydrogen

- Hydrogen Chloride

- Hydrogen Fluoride

- Methylcyclopentadienyl Manganese Tricarbonyl

- Nitric Acid (in concentrations > 70%)

- Nitrogen Tetroxide

- Oxygen

- Phosphorus Trichloride

- (Beta) Propiolactone

Recommended US Vessel Deficiency Procedures:

| ##### Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Identify deficiency. |  |  |  |
| 2 | Inform vessel representative. |  |  |  |
| 3 | Record on the Deficiency Summary Worksheet (next page). |  |  |  |
| 4 | If deficiency is corrected prior to end of inspection, go to Step 7. |  |  |  |
| 5 | If deficiency is unable to be corrected prior to end of inspection, issue CG-835 in accordance with table below. |  |  |  |
|  |  | IF deficiency: | THEN issue CG-835: |  |
|  |  | Does NOT immediately impact crew/passenger safety, hull seaworthiness, or the environment, e.g., - Missing placards - Non-metallic expansion joints more than 10 years in service | That provides a specific time for correcting deficiency, e.g., - “X” number of days - At next drydock |  |
|  |  | Allows vessel operations to be MODIFIED to meet less stringent requirements, e.g., - Expired international certificates - Automation defect | That restricts operation of vessel to meet current vessel conditions, e.g., - Reduced route - Increased crew |  |
|  |  | DOES immediately impact crew/passenger safety, hull seaworthiness, or the environment, and cannot be modified to meet less stringent requirements, e.g., - Missing or defective firefighting equipment - Structural defect or damage | That requires the deficiency to be corrected prior to operating vessel (“NO SAIL” item), e.g., - Prior to carrying cargo |  |
| 6 | Enter CG-835 data in MISLE |  |  |  |

Deficiency Summary Worksheet:

|  | Name of Vessel |  |  | VIN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ## Deficiency |  |  |  |  | ## Req’t. Issued / Date Completed |

Conversions:

| Distance and Energy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kilowatts (kW) |  |  |  |  |  |  |  | X |  |  |  | 1.341 |  |  |  |  |  | = |  |  |  | Horsepower (hp) |  |  |  |  |  |  |  |  |
| Feet (ft) |  |  |  |  |  |  |  | X |  |  |  | 3.281 |  |  |  |  |  | = |  |  |  | Meters (m) |  |  |  |  |  |  |  |  |
| Long Ton (LT) |  |  |  |  |  |  |  | X |  |  |  | .98421 |  |  |  |  |  | = |  |  |  | Metric Ton (t) |  |  |  |  |  |  |  |  |
| Liquid (NOTE: Values are approximate.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Liquid |  |  |  |  |  |  |  | bbl/LT |  |  |  |  |  | m3/t |  |  |  |  |  | ###### bbl/m3 |  |  |  |  |  |  | bbl/t |  |  |  |
| Freshwater |  |  |  |  |  |  |  | 6.40 |  |  |  |  |  | 1.00 |  |  |  |  |  | 6.29 |  |  |  |  |  |  | 6.29 |  |  |  |
| Saltwater |  |  |  |  |  |  |  | 6.24 |  |  |  |  |  | .975 |  |  |  |  |  | 6.13 |  |  |  |  |  |  | 5.98 |  |  |  |
| Heavy Oil |  |  |  |  |  |  |  | 6.77 |  |  |  |  |  | 1.06 |  |  |  |  |  | 6.66 |  |  |  |  |  |  | 7.06 |  |  |  |
| DFM |  |  |  |  |  |  |  | 6.60 |  |  |  |  |  | 1.19 |  |  |  |  |  | 7.48 |  |  |  |  |  |  | 8.91 |  |  |  |
| Lube Oil |  |  |  |  |  |  |  | 7.66 |  |  |  |  |  | 1.20 |  |  |  |  |  | 7.54 |  |  |  |  |  |  | 9.05 |  |  |  |
| Weight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Long Ton |  |  |  |  | = | 2240 lbs |  |  |  |  |  |  |  |  |  | 1 Metric Ton |  |  |  |  |  |  | = |  | 2204 lbs |  |  |  |  |  |
| 1 Short Ton |  |  |  |  | = | 2000 lbs |  |  |  |  |  |  |  |  |  | 1 Cubic Foot |  |  |  |  |  |  | = |  | 7.48 gal |  |  |  |  |  |
| 1 Barrel (oil) |  |  |  |  | = | 5.61 ft = 42 gal = 6.29 m3 |  |  |  |  |  |  |  |  |  | 1 psi |  |  |  |  |  |  | = |  | .06895 Bar = 2.3106 ft of water |  |  |  |  |  |
| Temperature: Fahrenheit = Celsius (F = 9/5 C + 32 and C = 5/9 (F – 32)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | = |  | -17.8 |  |  |  |  |  |  |  | 80 |  | = |  |  |  | 26.7 |  |  |  |  |  |  |  | 200 | = |  |  |  | 93.3 |
| 32 | = |  | 0 |  |  |  |  |  |  |  | 90 |  | = |  |  |  | 32.2 |  |  |  |  |  |  |  | 250 | = |  |  |  | 121.1 |
| 40 | = |  | 4.4 |  |  |  |  |  |  |  | 100 |  | = |  |  |  | 37.8 |  |  |  |  |  |  |  | 300 | = |  |  |  | 148.9 |
| 50 | = |  | 10.0 |  |  |  |  |  |  |  | 110 |  | = |  |  |  | 43.3 |  |  |  |  |  |  |  | 400 | = |  |  |  | 204.4 |
| 60 | = |  | 15.6 |  |  |  |  |  |  |  | 120 |  | = |  |  |  | 48.9 |  |  |  |  |  |  |  | 500 | = |  |  |  | 260 |
| 70 | = |  | 21.1 |  |  |  |  |  |  |  | 150 |  | = |  |  |  | 65.6 |  |  |  |  |  |  |  | 1000 | = |  |  |  | 537.8 |
| Pressure: Bars = Pounds per square inch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Bar |  | = |  | 14.5 psi |  |  |  |  |  | 5 Bars |  |  |  |  | = |  | 72.5 psi |  |  |  |  |  |  | 9 Bars |  |  |  | = | 130.5 psi |  |
| 2 bars |  | = |  | 29.0 psi |  |  |  |  |  | 6 Bars |  |  |  |  | = |  | 87.0 psi |  |  |  |  |  |  | 10 Bars |  |  |  | = | 145.0 psi |  |
| 3 Bars |  | = |  | 43.5 psi |  |  |  |  |  | 7 Bars |  |  |  |  | = |  | 101.5 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Bars |  | = |  | 58.0 psi |  |  |  |  |  | 8 Bars |  |  |  |  | = |  | 116.0 psi |  |  |  |  |  |  |  |  |  |  |  |  |  |